INTERNATIONAL ELECTROTECHNICAL COMMISSION SYSTEM FOR CERTIFICATION TO STANDARDS RELATING TO EQUIPMENT FOR USE IN EXPLOSIVE ATMOSPHERES (IECEx SYSTEM)

TITLE:	Draft ExTAG Decision Sheet – Software dependent semiconductor components

Circulated to: ExTAG – IECEx Testing and Assessment Group

INTRODUCTION

This document ExTAG/370 /CD Draft ExTAG Decision Sheet – Software dependent semiconductor components has been prepared by UL LLC with involvement from the following, IEC/SC 31G MT 60079-11

Please submit comments using the attached comments table by -

2015 06 29 to

Christine Kane

In the absence of comments the document will proceed for publication.

On behalf of Mr. Gauthier

Julien Gauthier

Julien Gauthier

ExTAG Secretary

| Address: Level 33, Australia Square 264 George Street Sydney NSW 2000 Australia | Contact Details: Tel: +61 2 46 28 4690 Fax: +61 2 46 27 5285 e-mail: chris.agius@iecex.com http://www.iecex.com |
| --- | --- |

| Member Body | Clause/ Sub-clause | Paragraph Figure/ Table | Type of comment General/ technical/ editorial | COMMENTS | Proposed change | Observation (to be completed by the originator) |
| --- | --- | --- | --- | --- | --- | --- |

COLLECTION OF IECEx / ExTAG DECISION

| Standard: IEC 60079-11:2011 (Edition 6.0) IEC 60079-11:2006 (Edition 5.0) | Clause: 7.5 7.5 |  |
| --- | --- | --- |
| Subject: Software dependent semiconductor components Status of document: Draft | Key words: - Semiconductor - Microcontroller - Protective component - Active voltage limiting circuit - Active current limiting circuit - Software | - Date: 2015 05 26 Originator of proposal: UL LLC TC/SC involved: IEC/SC 31G MT 60079-11 |
| Question Can software dependent components (for example microcontrollers), operating independently and meeting the requisite requirements for redundancy, be used as protective components in either active voltage or active current limiting circuits? Answer No. While reference may be made to “controllable semiconductor components”, this does not mean software controllable. The use of software dependent components would necessitate establishing an appropriate SIL for the software and conducting an evaluation of the software even though the microcontrollers are acting independently. As an appropriate SIL has not been defined in 60079-0 or 60079-11, this would be outside the scope of the standards and therefore cannot be used as protective components. |  |  |