### INTERNATIONAL ELECTROTECHNICAL COMMISSION (IEC) SYSTEM

FOR CERTIFICATION TO STANDARDS RELATING TO EQUIPMENT FOR

USE IN EXPLOSIVE ATMOSPHERES (IECEx SYSTEM)

TITLE: Draft ExTAG Decision Sheet – Revision of ExTAG DS 2006/003 Galvanically separating components

- Circulated to: ExTAG – IECEx Testing and Assessment Group

INTRODUCTION

This Draft Revision of Decision Sheet 2006/003 was prepared as a result of discussions held during the 2011 ExTAG Split meeting and is now issued for a 2 week comment period.

If no comment is received the document will then proceed for publication.

Comment to forwarded by 2011 10 10 to

Christine Kane

On behalf of Mr. Brenon

## Michel Brenon

ExTAG Secretary

| Address: IECEx Secretariat 286 Sussex Street Sydney NSW 2000 Australia | Contact Details: Tel: +61 2 8206 6940 Fax: +61 2 8206 6272 E-mail: chris.agius@iecex.com http://www.iecex.com | ExTAG Secretary Mr Michel Brenon/LCIE Tel: +33 1 40 95 5519 Fax: +33 1 40 95 5520 e-mail michel.brenon@lcie.fr |
| --- | --- | --- |

| Member Body/ Country | Clause/ Sub-clause | Paragraph Figure/ Table | Type of comment General/ technical/ editorial | COMMENTS | Proposed change | Observation |
| --- | --- | --- | --- | --- | --- | --- |

Draft revision ExTAG 2006/003

March 2006

IECEx / ExTAG DECISION SHEET

Title: ExTAG Decision Sheet – Galvanically separating components

| Standard: IEC 60079-11:1999 #### IEC 61241-11:2005 | Clause: 8.8 8 | Sheet No. |
| --- | --- | --- |
| Subject: Galvanically separating components Status of document: | Key words: - Isolating component - Opto-coupler | # Date: 2011-09-06 Originator of proposal: SC31G Secretary TC/SC involved: SC31G / MT60079-11 |
| Related documents: | ExTAG/70/CD ExTAG/73/CC ExTAG(Buxton/Secretariat)06 ExTAG 2006/003 |  |

Note: this ExTAG DS replaces ExTAG 2006/003

Question:  Under what conditions can opto-couplers be used to provide infallible separation between intrinsically safe circuits?

The conditions for use of isolating components to provide infallible separation of intrinsically safe circuits shall be acceptable provide they meet the requirements of;

- IEC 60079-11 Ed 5.0 Clause 8.8 or

- IEC 60079-11 Ed 6.0 Clause 8.9.