- INTERNATIONAL ELECTROTECHNICAL COMMISSION (IEC) SYSTEM FOR CERTIFICATION TO STANDARDS RELATING TO EQUIPMENT FOR USE IN EXPLOSIVE ATMOSPHERES (IECEx SYSTEM)

- Circulated to: ExTAG – IECEx Testing and Assessment Group

TITLE: Draft ExTAG Decision Sheet – Clarification on the use of IEC 60079-14 within IECEx Certificates of Conformity

INTRODUCTION

Document ExTAG/336/CD, prepared by the IECEx Secretariat as a Draft ExTAG Decision Sheet to assist in applying a consistent approach to inclusion of the relevant requirements of IEC 60079-14 when certifying an assembly of Ex equipment by ExCBs, was discussed during the ExTAG meeting in the Hague, August 2014 where it was agreed that this be re-issued to ExTAG for a further review following some changes to clarify requirements arising from the discussions

Revised version, ExTAG/336A/CD was prepared taking into account comments from the 2014 ExTAG Hague meeting and submitted for consideration by ExTAG.  Comments received were collated in document ExTAG/351/CC.

This further revised Draft ExTAG/336B/CD has now been prepared to take account of the comments received and is now issued for a final review.  It is therefore planned to forward this draft for publication unless ExTAG Members find some major issue that may prevent this moving forward.  Of course any proposed changes to add or improve the document could be considered at the next ExTAG Meeting.

Therefore please advise the IECEx Secretariat via Ms Christine Kane (email: christine.kane@iecex.com) by no later than 20 December 2014 if you have any major objection to this draft proceeding to publication.

To assist, the major changes to the previous version are shown in red text, eg addition of Requirements 7 + 8.

Julien Gauthier - ExTAG Secretary

| Address: IECEx Secretariat Level 33 Australia Square 264 George Street Sydney NSW 2000 Australia Web: www.iecex.com E-mail: info@iecex.com | ExTAG Secretary Mr Julien Gauthier LCIE S.A. 33 Avenue du General Leclerc 92260 Fontenay-aux-Roses - FRANCE Tel: +33 1 40 95 55 26 Fax: +33 1 40 95 89 37 Email : julien.gauthier@fr.bureauveritas.com |
| --- | --- |

| Member Body | Clause/ Sub-clause | Paragraph Figure/ Table | Type of comment General/ technical/ editorial | COMMENTS | Proposed change | Observation |
| --- | --- | --- | --- | --- | --- | --- |

COLLECTION OF IECEx / ExTAG DECISIONS

| Standard: IEC 60079-14:2013 | Clauses: All as relevant |  |
| --- | --- | --- |
| Subject: Reference to IEC 60079-14 within an IECEx Certificate of Conformity or Unit Verification Certificate Status of document: 2nd Revised Draft | Key words: Unit Verification installation assembly interconnection | # Date: # Revised Version Sept 2014 Originator of proposal: IECEx Secretariat + ExTAG Chairman |
| Background: With the increasing need of industry to seek “proof of compliance” with safety standards being driven by a combination of regulatory requirements and vendor requirements imposed through supplier contracts, the IECEx Certified Equipment Scheme is being seen as an effective compliance tool in this regard. The need for a supplier of Ex equipment to provide proof of compliance includes at the Ex Component Level, finished Ex product and Assemblies of Ex products that come together in the form of functioning equipment. This Decision Sheet is intended to apply to an assembly of items that come together in the form of functioning equipment. It is not intended for complex assemblies that are regarded as an installation in their own right nor for assemblies that contain their own source of explosion hazard for which additional considerations according to IEC 60079-10 series may be required. IECEx provides the IECEx 03 Certified Service Facilities Scheme (covering activities related to the installation, inspection, maintenance, repair and overhaul of Ex equipment) and the IECEx 05 Certification of Personnel Competence Scheme (CoPC) for such applications. For main stream Ex Components and finished Ex equipment, usually mass produced, the IECEx 02 Certified Equipment Scheme provides for an IECEx Certificate of Conformity that requires: - Ex Test Report (ExTR) - Quality Assessment Report (QAR) - On-going Surveillance To deal with special items produced as “one-off” items or in limited quantities, IECEx provides for the issuing of an IECEx Unit Verification Certificate, according to IECEx Operational Document OD 033. The IECEx Unit Verification Certificate is also seen as a solution for providing evidence that certain key requirements of the Installation Standard IEC 60079-14 have been met when dealing with the installation of Ex equipment in assemblies. Note that “skid mounted” parts of plant, e.g. blending stations in tank farms, are a part of the plant and cannot be seen as equipment and therefore it is up to the end user to design the plant in accordance with the legal requirements (local installation requirements at the place of installation must be known and must be considered). This ExTAG Decision Sheet provides guidance on the application of key aspects of IEC 60079-14 when being included in the overall assessment of assemblies of Ex equipment, such as fork-lift trucks, vacuum cleaners, heating systems etc. by defining a number of fundamentals that specify: - the minimum items that must be addressed during evaluation / assessment of Ex assemblies - the information that must be included in both ExTRs and IECEx Certificates of Conformity (including Unit Verification) and how this must be presented NOTE: Where reference is made to IECEx CoC throughout this DS, this shall include - IECEx Certificate of Conformity - IECEx Component Certificate - IECEx Unit verification Certificate QUESTION: What are the requirements that must be fulfilled both by an ExCB and the Certification process, when incorporating relevant requirements of IEC 60079-14 into the IECEx Certificate of Conformity process? ANSWER: Fundamental Requirements to be satisfied when incorporating relevant requirements of IEC 60079-14 into the IECEx Certificate of Conformity process: Requirement No. 1: An IECEx CoC is associated with equipment according to IEC 60079-0. If a product is out of the scope of the IEC 60079 series for electrical equipment or the ISO 80079 series for mechanical equipment an IECEx CoC cannot be issued. An IECEx CoC cannot be issued to show compliance with IEC 60079-14 alone. As a minimum, an IECEx CoC covering Ex Assemblies must show compliance with the General Requirements Standard, IEC 60079-0. Requirement No. 2: The “General Product Information” field of the ExTR supporting the IECEx CoC must clearly explain and detail the rationale of applying the various Clauses of IEC 60079-14 and clearly state which Clauses of IEC 60079-14 have been covered and the results of the relevant evaluations/tests. It is not possible to issue an ExTR for the installation of equipment in the end-users site. The latter may use IECEx Certified Service providers covered for this activity. Requirement No. 3: An IECEx CoC that includes reference to IEC 60079-14 shall state either within the IECEx CoC Equipment Description field, or as an attachment to the IECEx CoC the specific Clauses of IEC 60079-14 that have been satisfied. Requirement No. 4: Only ExCBs that have all of the following Ex protection techniques within their IECEx scope of acceptance are eligible to include references IEC 60079-14 when issuing IECEx CoCs: - Flameproof Ex ‘d’ - Increased safety Ex ‘e’ - Intrinsic safety Ex ‘i’ - Pressurized enclosures Ex ‘p’ - Dust ignition protection by enclosures Ex ‘t’ (required when dealing with assemblies designed for combustible dust areas) Requirement No. 5: Where installation requirements covered by Standards and Specifications other than IEC 60079-14 are applied, these maybe referenced in an Informative Attachment to the IECEx CoC but NOT within the body of the IECEx CoC. Requirement No. 6: Items to be considered when evaluating Ex assemblies for compliance by an ExCB with the relevant requirements of IEC 60079-14 shall include, as a minimum: - That the applicant for IECEx CoC has clearly identified the EPL or intended zone location for the Ex equipment, along with the overall T rating (including information on multiple EPLs or zones, if relevant), refer to IEC 60079-14 for relationship between zones and EPLs; - A Bill of Materials, assembly drawings or similar methods of defining the items of equipment and materials comprising the final Ex equipment assembly. - Items of Ex equipment that comprise the Ex assembly are to be covered by an IECEx CoC or shall be individually assessed by the ExCB and its associated ExTL for compliance with the relevant IEC standard(s) for that Ex protection; - The Ex protection of each item of Ex equipment comprising the Ex assembly shall be suitable for the declared EPL in item a) above; - The T rating of each item of Ex equipment comprising the Ex assembly shall be suitable for the overall declared T rating in item a) above. - Any completed cabling between individual equipment in the assembly shall be appropriate and in accordance with IEC 60079-14 and that it shall be provided with appropriate (and correctly assembled) cable entry devices. Note that cabling external to the assembly is not included. - Other requirements from IEC 60079-14 that are relevant to the particular equipment, for example that the mounting of flameproof enclosures takes account of the minimum obstruction distances for flange gaps in accordance with IEC 60079-14 - The final assembly shall be provided with the required documents such as the instructions for use according to Clause 30 of IEC 60079-0. Details of the evaluation of the above items a) to h) shall be recorded in the ExTR as referenced within the IECEx CoC. Requirement No.7: Items of Ex equipment that comprise the Ex assembly (except accessories such as cable entries, plugs or drainers) are to be listed within the IECEx CoC or as an attachment to the IECEx CoC with reference to the associated CoC number along with the relevant marking and identification of the applied standards, including the standard edition. Requirement No. 8: The assembly shall be marked with, and the IECEx CoC shall mention, a global marking including the symbol “Ex” and any important information regarding the safe use of the assembly, in addition to other mandatory requirements of the standard IEC 60079-0. For example : Ex IIC T4 Gb Tamb : -20°C to +85°C along with a statement that the individual protection types are given in the marking and CoCs for the incorporated equipment. |  |  |