INTERNATIONAL ELECTROTECHNICAL COMMISSION SYSTEM FOR CERTIFICATION TO STANDARDS RELATING TO EQUIPMENT FOR USE IN EXPLOSIVE ATMOSPHERES (IECEx SYSTEM)

- Circulated to: ExTAG – IECEx Testing and Assessment Group

TITLE:  Compilation of comments and observations on ExTAG/460/CD Draft ExTAG Decision Sheet – The tensile force of cable pull test according to IEC 60079-18

INTRODUCTION

This document contains the compilation of comments received on ExTAG/460/CD Draft ExTAG Decision Sheet – The tensile force of cable pull test according to IEC 60079-18, prepared by NEPSI, CN.

This document is issued for discussion during the 2017 ExTAG Washington meeting.

- Please inform the Secretariat immediately of any omissions or errors at-

- Christine Kane

On behalf of Mr. Julien Gauthier

- Julien Gauthier

ExTAG Secretary

| Address: IECEx Secretariat Level 33 Australia Square 264 George Street Sydney NSW 2000 Australia Web: www.iecex.com | ExTAG Secretary Mr Julien Gauthier LCIE S.A. 33 Avenue du General Leclerc 92260 Fontenay-aux-Roses FRANCE Tel: +33 1 40 95 55 26 Fax: +33 1 40 95 89 37 Email : julien.gauthier@fr.bureauveritas.com |
| --- | --- |

| ExCB/ ExTL | Clause/ Sub-clause | Paragraph Figure/ Table | Type of comment General/ technical/ editorial | COMMENTS | Proposed change | Observation (to be completed by the originator) |
| --- | --- | --- | --- | --- | --- | --- |
| IEC TC 31 Chairman |  |  |  | The intent of the clause in IEC 60079-18 edition 2 and 3 was to apply a test force (in newtons) of 20 times the cable diameter (in mm) or 5 times the force (in newtons) that will be applied as its weight. In edition 3, the MT tried to clean things up by putting the second condition in terms of newtons = mass x acceleration due to gravity (g) - rounding g to 10m/s2. If we ignore the rounding error in g, there was no change in the test condition from edition 2 to 3. The test force was the same value. Weight is already a force and takes into account the influence of g. Mass must be multiplied by g in order to get the force. In edition 4, responding to a comment that suggested that the requirement changed from edition 2 to edition 3 due to the number going from 5 to 50, the number was changed back to 5, but the text following the number was still in terms of mass, so the force (Newtons) now appears to have been reduced by 10, but only if you miss the fact that the force needs to be in Newtons, which is specified at the beginning of the clause. You will get the Newtons by multiplying by g. In all three editions of the standard, the test force is the same. |  |  |
| FME GB |  |  | te | The draft DS proposes a change to the technical requirements IEC 60079-18 Ed3 which is not permitted by the rules of ExTAG. However this draft DS does highlight that there is something wrong with the standard and we believe that this is with the 4th edition of IEC 60079-18 not the 3rd edition. The text of the current edition of the standard requires a pull test which has a safety factor of less than unity. | Send this to IEC TC31 MT60079-18 for an amendment to be created to reintroduce the text of edition 3 for this test. |  |
| FMG US |  |  | ge | We believe the issue is with Ed 4, not Ed 3. The MT was addressing a comment on the mis-match of units, but likely introduced an error with their change. When comparing mass and force, the acceleration of gravity (~10 m/sec2) has to be taken into account. Ed 4 specifies that the tensile force (in N) shall be 5 times the mass (in kg) of the equipment. Therefore, for a piece of “m” equipment with a 1 kg mass, the tensile force required would be 5 N which is roughly 0.5 kgf. A safety factor of 0.5 seems a bit low…. Had the standard specified that the required tensile force can be applied using a test mass of 5 times the mass of the “m” equipment, it would have resulted in the same test as Ed 3. Ed 3 specifies that the tensile force (in N) shall be 50 times the mass (in kg) of the equipment. Therefore, for a piece of “m” equipment with a 1 kg mass, the tensile force required would be 50 N which is roughly 5 kgf. The resulting safety factor of 5 seems to be about right. | A DS cannot be used to change the requirements of the standard. This requires a corrigendum, an amendment, or a new edition. Propose an amendment to Ed 4 to correct what appears to be an error. |  |
| NANIO CCVE (ExCB and ExTL |  |  | General | Despite the fact that in our opinion the requirement in accordance with 8.2.5.1 IEC 60079-18:2009 (“50 times the mass (in kilograms) of the “m” equipment”) is excessive, it is considered that the decision on approval of this DS shall be postponed until it will be requested about the clarification to MT 60079-18 whether this change was a misprint or correction. In case if it is a correction, the corrigendum to the IEC 60079-18 Ed. 4.0 is required to prepare with regard to identify this modification in the table of significant changes between IEC 60079-18 Edition 4.0 and IEC 60079-18 Edition 3.0. | To postpone the decision on the approval of this DS ExTAG until it will be requested about the clarification to MT 60079-18 To request to MT 60079-18 for the clarification of this issue |  |
| NEPSI CN |  |  | G | We support the draft decision ExTAG/460/CD. |  |  |
| QPS CA |  |  | General | This decision sheet seeks to modify a test requirement from a published standard. Decision Sheets cannot modify a requirement in a standard – OD35 | Reject DS |  |
| QPS CA |  |  | General | It is unclear what the purpose of this Decision sheet is. There is always a technical difference between one edition of the standard to the next, whether major or minor. We cannot arbitrarily decide to enforce a specific new test requirement on an older standard as this would seem to endorse the technical equivalency of the two editions without incorporating all of the other changes in the new version. | Reject DS Or Re propose that the 3rd edition of IEC 60079-18 cannot be used for the purposes of IECEx certification |  |
| TIIS JP |  |  |  | It is not clearly written in the Answer why the requirement (5 times) of Ed.4 shall be applied to the test with the standard Ed. 3.0 as well. With the clear and acceptable reason, TIIS would support the draft DS. |  |  |