INTERNATIONAL ELECTROTECHNICAL COMMISSION SYSTEM FOR CERTIFICATION TO STANDARDS RELATING TO EQUIPMENT FOR USE IN EXPLOSIVE ATMOSPHERES (IECEx SYSTEM)

- Circulated to: ExTAG – IECEx Testing and Assessment Group

TITLE:  Compilation of comments and observations on ExTAG/456/CD Draft ExTAG Decision Sheet – Temperature rise of the conductor bar for terminals in Level of Protection “eb” according to IEC 60079-7

INTRODUCTION

Draft Decision Sheet, ExTAG/456/CD Draft ExTAG Decision Sheet – Temperature rise of the conductor bar for terminals in Level of Protection “eb” according to IEC 60079-7 was prepared by LCIE.

This document contains the compilation of comments received on ExTAG/456/CD with observations from the originator.

As a result of comments received and considered, a revised Draft Decision Sheet ExTAG/456A/CD has now been published for consideration during the ExTAG Washington, USA, Meeting.

- Please inform the Secretariat immediately of any omissions or errors at

- Christine Kane

On behalf of Mr. Julien Gauthier

- Julien Gauthier

ExTAG Secretary

| Address: IECEx Secretariat Level 33 Australia Square 264 George Street Sydney NSW 2000 Australia Web: www.iecex.com | ExTAG Secretary Mr Julien Gauthier LCIE S.A. 33 Avenue du General Leclerc 92260 Fontenay-aux-Roses FRANCE Tel: +33 1 40 95 55 26 Fax: +33 1 40 95 89 37 Email : julien.gauthier@fr.bureauveritas.com |
| --- | --- |

| ExCB/ ExTL | Clause/ Sub-clause | Paragraph Figure/ Table | Type of comment General/ technical/ editorial | COMMENTS | Proposed change | Observation (to be completed by the originator) |
| --- | --- | --- | --- | --- | --- | --- |
| CNEX-Global B.V. NL | - | - | G | We support this DS. The 45K should not have been introduced in the IEC 60079-7 in the first place. | None | Noted. |
| FMG US |  |  | ge | According to MT60079-7 information, the change from 45K to 40K was done to better align with the cable limits and a T6 limitation. As the Note states, it is really the final application of the terminal in equipment that will establish the current rating / temperature class in accordance with 5.8, 6.8, and Annex E. It was not considered a major change. It should be noted that Ex Component Terminals with a current rating determined in accordance with 4.2.2.2 are tested in free air without an enclosure. They cannot be applied in this configuration and will require re-testing to determine a current rating / temperature class. | Do not issue this DS. | Not accepted. DS is limited to Ex components assessment. Requirement applies to terminals when certified as Ex component without consideration of later incorporation. |
| Kiwa NL | - | - | G | Kiwa does not support this DS. Changing the 45 K to 40 K is not a major change. This value is informative and can be used for an update of terminal assessments, not for complete equipment containing terminal components. | Withdraw this sheet. | Not accepted. The value is not given in a note and shall be considered as a requirement (not informative). DS is limited to Ex components assessment. Requirement applies to terminals when certified as Ex component without consideration of later incorporation. |
| NANIO CCVE (ExCB and ExTL |  |  | General | This decision sheet is not in line with clause 8.1.1 of IECEx 02, according to which IECEx certificate for series production can be issued either according to the current edition of the IEC standard or one previous edition. There was no information from MT 60079-7 that the previous version of the standard could not be used due to a change in this value and that this equipment could be dangerous. Due to the fact that in the table of changes in the standard this item is not specified and in IEC 60947-1: 2009 the value 45K for contact terminals is indicated, we suggest to apply to MT 60079-7 for an explanation of where the value of 40 K came from and whether it is a misprint. |  | Accepted in part. The intent of the DS was not to change the IECEx rules. Clarification of the DS to indicate that it applies only when an update of Ex component certificate to new standard is sought by the manufacturer or when the component certified to older edition is incorporated into an equipment assessed according to IEC 60079-7:2015. MT for IEC 60079-7 to decide about any further action with the standard. |
| NEPSI CN |  |  | G | We support to approve the draft decision sheet ExTAG/456/CD. |  | Noted. |
| QPS CA | - | Answer | general | Agree | No proposed changes | Noted. |
| QPS CA | - | Question | Editorial | The questions seems to ask if all terminals certified to the 4th edition be updated to the 5th edition. I believe that specific question should be answered only by the ExCB it is not the intention of the DS procedure. However a more general question should be asked to clarify the applicability of the DS. | Replace question with “Shall terminals, certified as components according to IEC 60079-7 Edition 4 be assessed according the new requirements of IEC 60079-7:2015 (Ed. 5.0) when included in an application or construction requiring compliance to IEC 60079-7:2015 (Ed. 5.0)” | Accepted in principle. DS reworded. |
| TIIS JP |  |  |  | TIIS supports the draft DS without comments. |  | Noted. |
| TestSafe AU | 4.2.2.2 Of IEC 60079-7 |  |  | TestSafe support the proposed decision sheet. |  | Noted. |
| UL- USA | Question |  | Editorial | The words “complementary assessed” are not clear. We believe the intent of the DS is to clarify that this requirement should have been identified as a technical change, and as such, requires evaluation to the requirements in the new edition. | Revise question to “additionally assessed” instead of “complementary assessed.” We also suggest that the MT for 60079-7 issue an errata or other notice, indicating this is a major technical change to the standard. | Accepted in part. “additionally assessed” modified with “complementary assessed.” MT for IEC 60079-7 to decide about any further action with the standard. |
| UL/DEMKO DK |  |  |  | Please consider UL LLC’s comments to ExTAG/456/CD also as UL Demko’s. |  | Noted. |