- INTERNATIONAL ELECTROTECHNICAL COMMISSION (IEC) SYSTEM FOR CERTIFICATION TO STANDARDS RELATING TO EQUIPMENT FOR USE IN EXPLOSIVE ATMOSPHERES (IECEx SYSTEM)

TITLE: Draft ExTAG Decision Sheet – Marking of combined Electrical/Non Electrical Equipment

- Circulated to: ExTAG – IECEx Testing and Assessment Group

INTRODUCTION

This document, ExTAG/447./CD Draft ExTAG Decision Sheet – Marking of combined electrical / non electrical equipment, has been prepared by INERIS in order to provide guidance to all ExCBs when assessing the Markings for combined electrical/non electrical equipment.

It is noted that IEC TC 31 is considering the marking requirements for combined electrical/non electrical equipment.  This Draft Decision Sheet is intended to provide consistency for such equipment covered by the IECEx Scheme and intended to be withdrawn once this issue is clarified by TC 31.

This draft is issued for a six week comment period for consideration by ExTAG Members.

Please submit comments using the comments table, a separate document by -

2017 05 05 to    christine.kane@iecex.com

If there are no comments received the document will be forwarded for publication.

Julien Gauthier

Julien Gauthier

ExTAG Secretary

| Address: IECEx Secretariat Level 33 Australia Square 264 George Street Sydney NSW 2000 Australia Web: www.iecex.com | ExTAG Secretary Mr Julien Gauthier LCIE S.A. 33 Avenue du General Leclerc 92260 Fontenay-aux-Roses FRANCE Tel: +33 1 40 95 55 26 Fax: +33 1 40 95 89 37 Email : julien.gauthier@fr.bureauveritas.com |
| --- | --- |

COLLECTION OF IECEx / ExTAG DECISIONS

| Standard: N/A | Clauses: N/A |  |
| --- | --- | --- |
| Subject: Guidance on marking requirements for combined electrical/non electrical equipment Status of document: Draft | Key words: - Marking - Labels | # Date: March 2017 Originator of proposal: INERIS |

Background:

Today the marking of electrical equipment is defined in IEC 60079-0 and the marking of non-electrical equipment is defined in IEC 80079-36.

Due to the fact, the combined marking is not defined in 60079-0 neither in

ISO 80079-36, the electrical part of the equipment has to be in accordance with IEC 60079-0 and the non-electrical part has to be in accordance with ISO 80079-36.

Therefore, the marking of electrical part and the non-electrical part has to be in two different lines:

- Ex db IIA T4 Gb

- Ex h IIA T4 Gb

QUESTION: How shall marking be shown for equipment covered by both the electrical and non electrical Standards (IEC 60079 and ISO 80079 series)

ANSWER:

Therefore, the marking of electrical part and the non-electrical part has to be in two different lines, for example:

- Ex db IIA T4 Gb

- Ex h IIA T4 Gb