- INTERNATIONAL ELECTROTECHNICAL COMMISSION (IEC) SYSTEM FOR CERTIFICATION TO STANDARDS RELATING TO EQUIPMENT FOR USE IN EXPLOSIVE ATMOSPHERES (IECEx SYSTEM)

TITLE:  Draft ExTAG Decision Sheet – Application of requirement for material remaining under a blind hole in flameproof enclosures

- Circulated to: ExTAG – IECEx Testing and Assessment Group

INTRODUCTION

This document, Draft ExTAG Decision Sheet   ExTAG/487A/CD Application of requirement for material remaining under a blind hole in flameproof enclosures

has been prepared by MASC, ZA.

As a result of comment received, a revised document ExTAG/487A/CD, was prepared by MASC, ZA.   A Compilation of Comments along with originator observations are contained in

ExTAG/521/CC Compilation of comments and observations on ExTAG/487/CD - Draft ExTAG Decision Sheet – Application of requirement for material remaining under a blind hole in flameproof enclosures

This updated version, ExTAG/487A/CD takes into account the comments received on ExTAG/487/CD and is issued for final discussion during the ExTAG 2018 Cannes meeting.

Changes made are shown in red text.

Julien Gauthier

Julien Gauthier

ExTAG Secretary

| Address: IECEx Secretariat Level 33 Australia Square 264 George Street Sydney NSW 2000 Australia Web: www.iecex.com | ExTAG Secretary Mr Julien Gauthier LCIE S.A. 33 Avenue du General Leclerc 92260 Fontenay-aux-Roses FRANCE Tel: +33 1 40 95 55 26 Fax: +33 1 40 95 89 37 Email : julien.gauthier@fr.bureauveritas.com |
| --- | --- |

COLLECTION OF IECEx / ExTAG DECISION

| Standard: IEC 60079-1:2014 (Ed 7.0) IEC 60079-1:2007 (Edition 6.0) IEC 60079-1:2003 (Edition 5.0) | Clause: 11.6 11.6 11.6 | Draft Decision Sheet: |
| --- | --- | --- |
| Subject: Application of requirement for material remaining under a blind hole in flameproof enclosures. Status of document: Draft | Key words: - Material remaining - Blind hole - 3mm - 1/3 diameter | - Date: 2018 08 10 Originator of original proposals: MASC TC/SC involved: TC31/MT 60079-1 |
| Background Various interpretations are being made by IECEx ExCB and ExTL staff regarding the application of the remaining material under a blind hole in a flameproof enclosure. Clause 11.6 (IEC 60079-1) uses the term “remaining thickness”, which is typically interpreted either as: - The thickness of the material around and under the hole. This interpretation is supported by the approach that no specific testing is applicable to test remaining thickness of material around a fastener, including cracks when fasteners are tightened into threaded blind holes with significant reduced material around the hole. The strength of the construction may be jeopardised by multiple internal explosions. The 3mm (or 1/3rd diameter, whichever is larger) requirement for material remaining is deemed a mechanical safeguard to ensure constructional integrity for the integrity of a hole, which is typically under stress by means of a threaded fastener inside the hole. In addition, fasteners in blind holes are not always applied at all points when the flameproof enclosure is tested (E.g. overpressure testing) and therefore the tests as described for flameproof cannot fully cover the strength requirements for the blind holes. - Only the thickness under the hole has a thickness requirement (3mm or 1/3rd diameter, whichever is larger) when holes are typically perpendicular into the enclosure. This approach is based on that there are no specific material thickness requirements for flameproof enclosures and therefore the material around a hole is not limited. Flameproof testing should consider most blind holes, e.g. impact and overpressure testing. Question: Clause 11.6 (IEC 60079-1) indicates “remaining thickness”, does “remaining thickness” mean thickness under the hole or thickness around and under the hole? Answer: The “remaining thickness” is the material under the hole and not around the hole. |  |  |