INTERNATIONAL ELECTROTECHNICAL COMMISSION (IEC) SYSTEM FOR CERTIFICATION TO STANDARDS RELATING TO EQUIPMENT FOR USE IN EXPLOSIVE ATMOSPHERES (IECEx SYSTEM)

- Title: Compilation of Comments on ExTAG/566/CD - Draft ExTAG Decision Sheet – External connections

Circulated to: ExTAG – IECEx Testing and Assessment Group

INTRODUCTION

This document contains the Compilation of Comments and Observations from the Originator Eurofins CML, GB, on ExTAG/566/CD Draft ExTAG Decision Sheet – External connections.

- Please inform the Secretariat immediately of any omissions or errors at

- Christine Kane

On behalf of Mr. Julien Gauthier

Julien Gauthier

ExTAG Secretary

| Address: IECEx Secretariat Level 33 Australia Square 264 George Street Sydney NSW 2000 Australia Web: www.iecex.com | ExTAG Secretary Mr Julien Gauthier LCIE S.A. 33 Avenue du General Leclerc 92260 Fontenay-aux-Roses FRANCE Tel: +33 1 40 95 55 26 Fax: +33 1 40 95 89 37 Email : julien.gauthier@fr.bureauveritas.com |
| --- | --- |

| ExCB/ ExTL | Clause/ Sub-clause | Paragraph Figure/ Table | Type of comment General/ technical/ editorial | COMMENTS | Proposed change | Observation (to be completed by the originator) |
| --- | --- | --- | --- | --- | --- | --- |
| CNEX-Global BV NL | - | - | t | The previous edition of the -18 standard (Ed.3) started in cl. 7.6.1 that the applied cables must ensure that no explosive atmosphere can enter the Ex m equipment. This was a clear and complete, generally stated requirement. 3rd Ed. text: However, this text was omitted in the removed in the 4th Ed. Note: it is not enough to state that ‘All cable terminations within the equipment must be fully encapsulated to prevent the ingress of the explosive atmosphere into free space within the equipment.’, because otherwise the ‘loose sheathed multi-core or co-axial cables’ would be acceptable is fully encapsulated while passing through the compound. | Re-instate the original first sentence of cl. 7.6.1 of 3rd Ed. in the 4th Edition cl. 7.6.1 |  |
| DEK NL |  |  |  | We think clause 7.6.1 and the note are clear and acceptable. We do not agree with the limitation as given in the answer of this sheet. Note that ExTAG decision sheets are not allowed to give limitations on requirements of the standard. We accept Ex m equipment having electronics in it with its own enclosure and its own fixed cable terminated inside this electronics enclosure. The inside of the electronics enclosure is seen as free volume, At least not filled with the same encapsulant as surrounding the electronics enclosure and providing Ex m. | Withdraw this sheet | Not accepted. The current standard text does not include the paragraph beginning ‘The entry of all electric conductors…’ and there is no note, this paragraph and note was in the 3rd edition but was removed in the 4th edition, hence the reason why this DS was prepared. However, noting the misreading of the DS, the introduction has been redrafted. |
| Kiwa NL |  |  | G | No comments |  | Noted |
| LOM ES |  |  | Technical | We agree with the proposal. But it could be extended a bit by saying that the cables cannot terminate in free space neither to pass through free spaces | change the question to include that wires cannot pass through free spaces either | Not accepted While it is an agreeable comment, this goes beyond the original intention of the DS. Suggest to pass this comment on to IEC TC31 MT60079-18 for consideration in the next edition. |
| NANIO CCVE (ExCB and ExTL) RU |  |  | General | We support DS ExTAG/566/CD without comments. |  | Noted |
| NCC BR | 7.3 7.6.1 |  |  | We agree. |  | Noted |
| NEPSI CN |  |  | G | We support the draft decision sheet ExTAG/566/CD. |  | Noted |
| PTB DE |  |  | ge | In favor with DS | - | Noted |
| QPS CA |  |  | ge | Agree with the proposed draft decision sheet. No comments |  | Noted |
| TC 31 | Answer |  | Te | The correct answer depends on the design of the “m” equipment, the level of protection and the volume of the free space. If the design is such that the pressure test according to 8.2.6 has to be performed and passes, there will no ingress of the explosive atmosphere and free space terminations are acceptable. If the design is such that the pressure test according to 8.2.6 is not required due to allowances given in Table 2 and 3 of the standard, ingress of the explosive atmosphere may be possible and terminations in the free space are not acceptable. | Change the answer from “No” to “If the design is such that the pressure test according to 8.2.6 has to be performed and passes, terminations in a free space are acceptable. If the test is not required or it is performed and fails, terminations in a free space are NOT acceptable. | Accepted in principle The original answer in the DS has been reworded based on the TC31 recommendation. However, considering that the pressure test of 8.2.6 does not require the equipment under test not to leak, further clarification has been added to the DS answer to use 8.2.6 as an acceptance method if no leakage through the cable is observed. |
| TIIS JP |  |  | ge | TIIS supports the draft DS without comments. |  | Noted |
| TÜV NORD CERT DE | IEC 60079-18:2009 Ed. 3 Clause 7.6.1 | - | Proposed answer | All cable terminations within the equipment shall be fully encapsulated to prevent the ingress of the explosive atmosphere into free space within the equipment. | - | Accepted “must” replaced by “shall”. |
| TUV SUD PS DE | 7.3, 7.6.1 |  | General | We concur with the answer. |  | Noted |
| UL USA |  |  |  | UL-USA agrees with 566/CD. |  | Noted |
| ULBR BR |  |  | General | ULBR supports this DS without comments. |  | Noted |
| ULD DK |  |  | General | UL Demko supports this DS without comments. |  | Noted |