| Reference No: |  |  |  |  |  |  |  |  |  |  | #### IEC 60079-1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Field of application This form is intended to be used when assessing and testing electrical apparatus for explosive atmospheres according to the standard specified in the adjoining column. The test results are valid for the tested items only. Reproduction This report must not be reproduced other than in its entirety except with the prior written approval of the issuing body | Standard: # IEC 60079-1 : 1990 Electrical apparatus for explosive gas atmospheres – Construction and verification test of flameproof enclosures of electrical apparatus |  |  |  |  | ## Instructions for use The comment columns can be used to e.g. specify test values, applied alternative requirements/tests or to refer to enclosed additional pages with comments. Abbreviations: N/A = Not applicable (for the assessment/test of the equipment and its design documentation) The applicant is the body/person who has requested the assessment/test and to which the results, as specified in this report, are provided. The short descriptions in the column for "Clause" do not replace the complete text in the standard. |  |  |  |  |  |
|  | Amendments: 1 : 1993-08 2 : 1998-05 |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | # This test report may only be used for the # IECEx Scheme and must be accompanied by an EXTR cover page issued by an ExCB. |  |  |  |  |  |
| ## Product name |  |  |  |  | # Model/Type designation |  |  |  |  |  |  |
| Serial No | Code (e.g. Ex II T ) |  |  |  |  |  | Ambient temperature [min-max] |  |  |  |  |
| Applicant [name and postal address] |  |  |  |  | Manufactured by [name and postal address] |  |  |  |  |  |  |
| Test sample received [date] |  |  |  |  |  |  |  |  |  |  |  |
| ## Product description |  |  |  |  |  |  |  |  |  |  |  |
| The equipment complies with the standard [yes/no] | ## Prepared by |  |  |  |  |  |  |  |  | Date [year-month-day] |  |
|  | [signature] |  |  |  |  |  |  |  |  |  |  |

Sheet 1 of 52

| Reference No: |  |  |  |  |  |  | IEC 60079-1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Clause |  | Results |  |  |  |  |  |  |
|  |  | Pass | Fail | N/A | Comments |  |  | ExCB Review |
| 1 | Scope |  |  |  |  |  |  |  |
| 2 See also DS 2010/006A | Definitions |  |  |  |  |  |  |  |
| 3 | Grouping and temperature classification |  |  |  |  |  |  |  |
| 4.1 | Flameproof joints – General requirements |  |  |  |  |  |  |  |
| 4.2.1 | Flameproof joints – Width of non-threaded joints |  |  |  |  |  |  |  |
| 4.2.2 | Flameproof joints – Surface roughness |  |  |  |  |  |  |  |
| 4.2.3 | Flameproof joints – Gap |  |  |  |  |  |  |  |
| 4.2.4 | Flameproof joints – Spigot joints |  |  |  |  |  |  |  |
| 4.2.5 | Flameproof joints – Flanged joints for Group IIC |  |  |  |  |  |  |  |
| 4.2.6 | Flameproof joints – Holes in joints |  |  |  |  |  |  |  |
| 4.2.6.1 | Flameproof joints – Holes in joints/Flanged joints |  |  |  |  |  |  |  |
| 4.2.6.2 | Flameproof joints – Holes in joints/Spigot joints |  |  |  |  |  |  |  |
| 4.3 | Flameproof joints – Threaded joints |  |  |  |  |  |  |  |
| 4.4 | Flameproof joints – Gaskets and O-rings |  |  |  |  |  |  |  |
| 4.5 | Flameproof joints – Cemented joints |  |  |  |  |  |  |  |
| 5 | Operating rods (spindles) |  |  |  |  |  |  |  |
| 6 | Shafts and bearings |  |  |  |  |  |  |  |
| 7 | Light-transmitting parts |  |  |  |  |  |  |  |
| 8 | Breathing and draining devices |  |  |  |  |  |  |  |
| 9 | Fasteners |  |  |  |  |  |  |  |
| 10 See also DS 2012/004 | Mechanical strength of the enclosure |  |  |  |  |  |  |  |
| 11 | Connection of conductors and cables |  |  |  |  |  |  |  |
| 11.1 | Connection of conductors and cables |  |  |  |  |  |  |  |
| 11.2 | Connection of conductors and cables; conduit connection |  |  |  |  |  |  |  |

Sheet 2 of 52

| Reference No: |  |  |  |  |  |  | IEC 60079-1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Clause |  | Results |  |  |  |  |  |  |
|  |  | Pass | Fail | N/A | Comments |  |  | ExCB Review |
| 11.3 11.3.1 11.3.2 | Connection of conductors and cables; indirect entry |  |  |  |  |  |  |  |
| 11.3.3 11.3.3.1 11.3.3.2 11.3.3.3 | Connection of conductors and cable; indirect entry (plug and socket connection) |  |  |  |  |  |  |  |
| 11.4 | Connection of conductors and cable; direct entry |  |  |  |  |  |  |  |
| 12 | Marking |  |  |  |  |  |  |  |
| 13 | General (Verifications and tests) |  |  |  |  |  |  |  |
| 14 | Type tests |  |  |  |  |  |  |  |
| 14.1 | Tests of ability of the enclosure to withstand pressure |  |  |  |  |  |  |  |
| 14.1.1 | Determination of explosion pressure |  |  |  |  |  |  |  |
| 14.1.2 | Overpressure test |  |  |  |  |  |  |  |
| 14.2 | Tests for non-transmission of an internal ignition |  |  |  |  |  |  |  |
| 15 | Routine tests |  |  |  |  |  |  |  |
| A.1 | Scope (Non-metallic parts of flameproof enclosures) |  |  |  |  |  |  |  |
| A.2 | Special requirements for construction |  |  |  |  |  |  |  |
| A.3.1.1 | Ambient temperature during tests |  |  |  |  |  |  |  |
| A.3.1.2 | Sequence of tests |  |  |  |  |  |  |  |
| A.3.1.3 | Thermal endurance to heat (Group I; 1ST test sequence) |  |  |  |  |  |  |  |
| A.3.1.4 | Thermal endurance to cold (Group I; 1ST test sequence) |  |  |  |  |  |  |  |
| A.3.2 | Tests for flameproofness (Group I; 1ST test sequence) |  |  |  |  |  |  |  |
| A.3.3 | Flammability (Group I; 1ST test sequence) |  |  |  |  |  |  |  |
| A.3.1.6 | Resistance to oils and greases (Group I; 2ND test sequence) |  |  |  |  |  |  |  |
| A.3.2 | Tests for flameproofness (Group I; 2ND test sequence) |  |  |  |  |  |  |  |
| A.3.3 | Flammability (Group I; 2ND test sequence) |  |  |  |  |  |  |  |

Sheet 3 of 52

| Reference No: |  |  |  |  |  |  | IEC 60079-1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Clause |  | Results |  |  |  |  |  |  |
|  |  | Pass | Fail | N/A | Comments |  |  | ExCB Review |
| A.3.1.6 | Resistance to hydraulic liquids (Group I; 3RD test sequence) |  |  |  |  |  |  |  |
| A.3.2 | Tests for flameproofness (Group I; 3RD test sequence) |  |  |  |  |  |  |  |
| A.3.3 | Flammability (Group I; 3RD test sequence) |  |  |  |  |  |  |  |
| A.3.1.3 | Thermal endurance to heat (Group II; test sequence) |  |  |  |  |  |  |  |
| A.3.1.4 | Thermal endurance to cold (Group II; test sequence) |  |  |  |  |  |  |  |
| A.3.2 | Tests for flameproofness (Group II; test sequence) |  |  |  |  |  |  |  |
| A.3.3 | Flammability (Group II; test sequence) |  |  |  |  |  |  |  |
| A.3.1.7 | Resistance to chemical agents (Group II; test sequence) |  |  |  |  |  |  |  |
| A.3.1.5 | Resistance to light |  |  |  |  |  |  |  |
|  | Tests for cable entries and stopping boxes |  |  |  | according to national regulations: |  |  |  |
| B.1 | Scope (Breathing devices and draining devices) |  |  |  |  |  |  |  |
| B.2 | General requirements |  |  |  |  |  |  |  |
| B.3 | Materials |  |  |  |  |  |  |  |
| B.4 | Dimensions |  |  |  |  |  |  |  |
| B.5 | Elements with measurable paths |  |  |  |  |  |  |  |
| B.6 | Additional requirements for crimped ribbon elements of breathing and draining devices |  |  |  |  |  |  |  |
| B.7 | Elements with non-measurable paths |  |  |  |  |  |  |  |
| B.8 | Device that can be dismantled |  |  |  |  |  |  |  |
| B.9 | Mounting arrangements of the elements |  |  |  |  |  |  |  |
| B.10 | Mechanical strength |  |  |  |  |  |  |  |
| B.11 | Tests of flameproof enclosure with breathing/draining devices |  |  |  |  |  |  |  |
| B.11.1 | Test of ability of the enclosure to withstand pressure |  |  |  |  |  |  |  |
| B.11.2 | Thermal tests |  |  |  |  |  |  |  |
| B.11.3 | Test for non-transmission of an internal ignition |  |  |  |  |  |  |  |

Sheet 4 of 52

| Reference No: |  |  |  |  |  |  | IEC 60079-1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Clause |  | Results |  |  |  |  |  |  |
|  |  | Pass | Fail | N/A | Comments |  |  | ExCB Review |
| B.12 | Breathing devices and draining devices when used as Ex components |  |  |  |  |  |  |  |
| B.12.1 | Mounting arrangements of the elements and component |  |  |  |  |  |  |  |
| B.12.2 | Type tests for breathing and draining devices used as Ex components |  |  |  |  |  |  |  |
| B.12.2.1 | Tests of the ability of the breathing and draining device to withstand pressure |  |  |  |  |  |  |  |
| B.12.2.2 | Thermal tests |  |  |  |  |  |  |  |
| B.12.2.3 | Test for non-transmission of an internal ignition |  |  |  |  |  |  |  |
| B.12.3 | Marking |  |  |  |  |  |  |  |
| B.12.4 | Component certificate |  |  |  |  |  |  |  |

Sheet 5 of 52

| Section 2 EVALUATION RECORD To be completed by ExTL IEC 60079-1 1990 Plus Amendment 1 Electrical apparatus for explosive gas atmospheres - Part 1: Construction and verification test of flameproof enclosures of electrical apparatus |  |
| --- | --- |
| Report |  |
| Reference No. |  |
| Tested by ( + signature) Approved by (+ signature) Date of issue Content |  |
| This report is based on a standard format prepared by CSA International |  |
| Client |  |
| Name Address |  |
| Evaluation Record Form No. EXTR600791/Version1a/DS Copyright reserved to the bodies participating in the IECEx Scheme |  |
| Testing laboratory |  |
| Name Address Testing location |  |
| Test item Standard IEC 60079-1 Description. Trademark. Model and/or type reference Manufacturer |  |

Sheet 6 of 52

| Test Item (continued) Rating(s) Type of transformers (where relevant) Application Protection index Other characteristics Rated ambient temperature ta (OC) | stationary / portable / hand-held IP |
| --- | --- |
| Other information |  |

Sheet 7 of 52

General remarks This evaluation record is not valid unless appended to an IECEx Test Report issued by an ExCB, in accordance with IECEx 02. This evaluation record shall not be reproduced except in full without the written approval of the testing laboratory. The test results presented in this evaluation record relate only to the item tested.

Copy of marking plate

Sheet 8 of 52

| ## MANUFACTURER'S DOCUMENTS |  |  |  |  |
| --- | --- | --- | --- | --- |
| Item | Subject | Drawing  | Rev. | Date |

Sheet 9 of 52

# Evaluation Record

| IEC 60079-1 |  | SECTION ONE  GENERAL 1/2 | # APPLIED |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| CLAUSE |  | REQUIREMENTS | # YES | NO | ENCL. |  |
| 1 |  | Scope |  |  |  |  |
| Scope |  |  |  |  |  |  |
| The electrical apparatus defined below (see overview) and in 2/2 is feasible for coverage in accordance with the specific requirements addressed by this part of IEC 60079: |  |  |  |  |  |  |
|  | YES |  |  |  |  |  |
|  | NO |  |  |  |  |  |
|  | Notes: |  |  |  |  |  |
|  | (Refer to 2/2 for considerations for ambient temperatures below  20 C or above 60 C) |  |  |  |  |  |
| Form sheets not included in this report (reasons as indicated): |  |  |  |  |  |  |
|  | Annex A (normative); not applicable since no flameproof enclosure non-metallic parts are incorporated |  |  |  |  |  |
|  | Annex B (normative); not applicable since no breathing or draining devices are involved |  |  |  |  |  |
| Overview: |  |  |  |  |  |  |

Sheet 10 of 52 	(IEC 60079-1, 1990, Amendments 1 & 2)

| ## IEC 60079-1 |  |  | ## SECTION ONE  GENERAL 2/2 |  |  | APPLIED |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ##### CLAUSE |  |  | REQUIREMENTS |  |  | YES | NO | ENCL. |  |  |
| 2 |  |  | Definitions |  |  |  |  |  |  |  |
| 3 |  |  | Grouping and temperature classification |  |  |  |  |  |  |  |
| Note: The type of protection covered by this part of IEC 60079 is dependent on the specific requirements addressed in the forms which follow. |  |  |  |  |  |  |  |  |  |  |
| Item: |  |  |  |  |  |  |  |  |  |  |
| Definitions |  |  |  |  |  |  |  |  |  |  |
|  | All definitions covered in Clause 2 apply, as noted for the specific requirements herein |  |  |  |  |  |  |  |  |  |
| Grouping and temperature classification (refer to forms for IEC 60079-0) |  |  |  |  |  |  |  |  |  |  |
|  | Group I apparatus |  |  |  | Group II apparatus, in which case requirements for specific sub-divisions are applied |  |  |  |  |  |
| Ambient temperatures below 20 C or above 60 C: |  |  |  |  |  |  |  |  |  |  |
|  | measures accounting for higher explosion pressures and the possibility of brittle failure at low temperatures: |  |  |  |  |  |  |  |  |  |
|  | measures accounting for decrease of the maximum safe gap at high temperatures: |  |  |  |  |  |  |  |  |  |

Sheet 11 of 52 	(IEC 60079-1, 1990, Amendments 1 & 2)

| ###### IEC 60079-1 |  |  |  |  | SECTION TWO  CONSTRUCTION 1/12 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | APPLIED |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ###### CLAUSE |  |  |  |  | REQUIREMENTS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | YES |  | NO |  |  | ENCL. |  |  |  |  |  |  |
| 4.1 |  |  |  |  | ###### Flameproof joints - General requirements |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4.2.1 |  |  |  |  | ###### Flameproof joints - Width of non-threaded joints |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Item: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Flameproof joints - General requirements |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Additional safety factors/tests applied to special joints: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | labyrinth (Fig. 1) |  |  |  |  |  |  |  | serrated (Fig. 2) |  |  |  |  |  |  |  |  |
| Joint surfaces: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | protected against corrosion (method) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Note: Paint or similar material is not normally permitted. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Non-threaded joints other than spigot joints (refer to Tables I, IIA, IIB, IIC): |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Joint components |  |  |  |  |  |  | Shape (X) |  |  |  |  |  |  |  | Encl. volume (cm3) |  | Width L (mm) |  |  |  |  |  |  | Distance l (mm) |  |  | Diameter (mm) |  |  |  |  | Gap i (mm) |  |  |  |  |  |
|  |  |  |  |  |  |  |  | cylindrical |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | conical |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | flat |  |  |  | R |  |  | M |  |  |  | R | M |  | R |  |  | M |  | R | M |  |  |  |  |
|  | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | R = required; M = measured/actual (see also considerations for Clauses 4.2.1, 4.2.2, 4.2.3, 4.2.5, 4.2.6 and 4.2.6.1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Flameproof joints - Width of non-threaded joints (cylindrical parts and conical surfaces) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Joint of cylindrical metallic parts (refer to table above): |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | press-fitted bushing |  |  |  |  |  |  |  |  | other ( |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ) |  |  |  |
|  |  |  | reduction of joint width to 5 mm applied, based on:- |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | enclosure volume, |  |  |  |  |  |  |  |  |  |  |  | cm3 Required: volume  2000 cm3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| and |  |  |  |  |  |  |  |  |  |  |  | non-reliance on interference fit to prevent part being displaced in type test; i.e. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| and |  |  |  |  |  |  |  |  |  |  |  | compliance with impact test requirements; refer to forms for IEC 60079-0, Test for resistance to impact |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| and |  |  |  |  |  |  |  |  |  |  |  | diameter of press-fitted part, |  |  |  |  |  |  |  |  |  |  |  |  |  |  | mm Required: diameter  60 mm |  |  |  |  |  |  |  |  |  |  |  |
| Joint including conical surfaces (refer to table above): |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | gap is uniform through-out the conical part (verified by |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ) |  |  |  |
|  |  |  | cone angle  5 ( |  |  |  |  |  |  | ) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Note: The width of the joint and the gap normal to the surfaces shall comply with the relevant dimensions in Tables I, IIA, IIB and IIC. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Sheet 12 of 52 	(IEC 60079-1, 1990, Amendments 1 & 2)

| ###### IEC 60079-1 |  |  |  |  |  | SECTION TWO  CONSTRUCTION 2/12 |  |  |  |  |  |  |  |  |  |  |  |  |  | APPLIED |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ###### CLAUSE |  |  |  |  |  | REQUIREMENTS |  |  |  |  |  |  |  |  |  |  |  |  |  | YES |  | NO |  | ENCL. |  |  |  |  |
| 4.2.2 |  |  |  |  |  | Flameproof joints - Surface roughness |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4.2.3 |  |  |  |  |  | ###### Flameproof joints - Gap |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4.2.4 |  |  |  |  |  | ###### Flameproof joints - Spigot joints |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Item: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Flameproof joints - Surface roughness |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Average roughness (Ra): (Ref. ISO 468) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  6.3 µm ( |  |  |  | µm); verified by |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Flameproof joints - Gap |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Intentional gap between flanged joint surfaces of parts, |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | NO |  |  |  |  |  | YES; purpose: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | Required: gap between joint surfaces shall at no point exceed the recorded maximum value |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Provision for checking gaps of flanged joints (parts, |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ): |  |
|  |  |  | directly |  |  |  |  |  | indirectly (by calculation or measurement) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | method of checking: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Required for Group I apparatus having doors and covers designed to be opened (Fig. 3) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Flameproof joints - Spigot joints |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Refer to Tables I, IIA, IIB, IIC: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Joint components |  |  |  |  |  |  | Encl. volume (cm3) |  | Parts comprising width of joint, L |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Width L (mm) |  |  |
|  |  |  |  |  |  |  |  |  |  | Cylindrical part |  |  |  |  |  |  | *cylindrical |  | Plane part |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | Width d [*L] (mm) |  | Width a (mm) |  | Gap i (mm) |  |  |  | part only | Width c (mm) |  | Width b (mm) |  | Gap i (mm) |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | M |  | M |  | R |  | M |  | (X) | M |  | M |  | R |  | M | R |  |  |
|  | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Notes: (i) R = required; M = measured/actual. (ii) Width L is determined by width c + width d. (iii) Required for IIC: c  6 mm and d   L. (iv) See also considerations for Clauses 4.2.6.2 (holes in joints, where a + b = distance l ) and 4.2.2. (v) *Width L determined by cylindrical part only:  No gap requirements for plane part for Groups I, IIA and IIB  Required for Group IIC  gap i of plane part  gap i for cylindrical part |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Gasket in plane part: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | gap i of plane part measured in each case after compression of gasket, and |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | minimum width (d or *L) of cylindrical part maintained in each case before and after compression of gasket |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Metallic or metal-clad compressible gasket in plane part (gasket type; |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ): |  |
|  |  |  | Group IIC; gap i between each surface of plane part and gasket measured in each case after compression |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Sheet 13 of 52 	(IEC 60079-1, 1990, Amendments 1 & 2)

| ###### IEC 60079-1 |  |  |  |  |  |  | SECTION TWO  CONSTRUCTION 3/12 |  |  |  |  |  |  |  |  |  |  |  | APPLIED |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ###### CLAUSE |  |  |  |  |  |  | REQUIREMENTS |  |  |  |  |  |  |  |  |  |  |  | YES | NO |  |  | ENCL. |  |  |  |  |
| 4.2.5 |  |  |  |  |  |  | Flameproof joints - Flanged joints for Group IIC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4.2.6 |  |  |  |  |  |  | Flameproof joints - Holes in joints |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4.2.6.1 |  |  |  |  |  |  | Flameproof joints - Holes in joints/Flanged joints |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4.2.6.2 |  |  |  |  |  |  | Flameproof joints - Holes in joints/Spigot joints |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4.3 |  |  |  |  |  |  | Flameproof joints - Threaded joints |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Item: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Flameproof joints - Flanged joints for Group IIC (apparatus for use in acetylene) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | gap i  0.040 mm for L  9.5 mm and enclosure volume  500 cm3 (otherwise flanged joints are not permitted) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | refer to 1/12, table for non-threaded joints other than spigot joints (Page |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ) |  |  |  |
|  |  |  | measures to prevent ignition of surrounding atmosphere by projection of debris/dust/carbon through joints: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | e.g. provision of gaskets, angled or labyrinth joints, deflectors or screens |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Flameproof joints - Holes in joints |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Joint surface interrupted by holes for clamping bolts etc. (parts, |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ): |  |
|  |  |  | distance l  6 mm for width L < 12.5 mm |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | distance l  8 mm for width L >12.5 mm and < 25 mm |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | distance l  9 mm for width L  25 mm |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Note: Some countries may require distance l to exceed the above values |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Flameproof joints – Holes in joints/Flanged joints |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Measurement of distance l for parts, |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | : |  |
|  |  |  | between hole and inside of enclosure |  |  |  |  |  |  |  |  |  |  |  |  | between hole and outside of enclosure |  |  |  |  |  |  |  |  |  |  |  |
| Flameproof joints – Holes in joints/Spigot joints |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| For parts, |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | : |  |  |  |  |  |  |
|  |  |  | distance l comprises width a of cylindrical part + width b of plane part, based on:- |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | at chamfer, f  1 mm and |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | gap of cylindrical part  0.2 mm (Groups I, IIA) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| or |  |  |  |  |  |  |  |  |  | gap of cylindrical part  0.15 mm (Group IIB) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| or |  |  |  |  |  |  |  |  |  | gap of cylindrical part  0.1 mm (Group IIC) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | distance l is based on width b of the plane part alone |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Flameproof joints - Threaded joints |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Groups I, IIA and IIB enclosures: (as an alternative refer to tables for Group IIC enclosures) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Joint components |  |  |  |  |  |  |  |  |  |  | Threads engaged |  |  | Axial engagement for enclosure volume: |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  100 cm3 |  |  |  |  |  |  | > 100 cm3 |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | Required | Actual |  | Required |  |  | Actual (mm) |  |  |  | Required |  |  | Actual (mm) |  |  |
|  | 1 |  |  |  |  |  |  |  |  |  |  |  5 |  |  |  5 mm |  |  |  |  |  |  |  8 mm |  |  |  |  |  |
|  | 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 |  |  |  |  |  |  |  |  |  |  |  5 |  |  |  5mm |  |  |  |  |  |  |  8 mm |  |  |  |  |  |
|  | 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Sheet 14 of 52 	(IEC 60079-1, 1990, Amendments 1 & 2)

| ###### IEC 60079-1 |  |  |  |  |  |  | SECTION TWO  CONSTRUCTION 4/12 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | APPLIED |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ###### CLAUSE |  |  |  |  |  |  | REQUIREMENTS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | YES |  | NO |  |  | ENCL. |  |  |  |  |  |  |  |  |  |
| 4.3 |  |  |  |  |  |  | Flameproof joints - Threaded joints (continued) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4.4 |  |  |  |  |  |  | Flameproof joints - Gaskets and O-rings |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Item: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Flameproof joints - Threaded joints (continued) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Group IIC enclosures: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Joint components |  |  |  |  |  |  | Pitch (mm) Required:  0.7 mm &  3 mm |  |  | *Class of fit (X) |  |  |  | Threads engaged |  |  |  |  |  | Engagement length for enclosure volume: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | Medium |  |  |  |  |  |  |  |  |  |  100 cm3 |  |  |  |  |  100 cm3 |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | Fine |  |  |  | Required |  | Actual |  |  |  | Required |  | Actual (mm) |  |  | Required |  |  |  |  | Actual (mm) |  |  |  |  |  |
|  |  | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  5 |  |  |  |  |  |  5 mm |  |  |  |  |  8 mm |  |  |  |  |  |  |  |  |  |  |
|  |  | 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  5 |  |  |  |  |  |  5 mm |  |  |  |  |  8 mm |  |  |  |  |  |  |  |  |  |  |
|  |  | 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Notes: (i) *Class of fit: medium, 0.7 pitch  2 mm; fine, 2  pitch  3 mm; ref. ISO 965-1 and 965-3. (ii) Cylindrical threaded joints which do not conform with the ISO standard are permitted if the test for non-transmission of an internal ignition is passed. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | ###### ref. Tests for non-transmission of an internal ignition |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Group IIC enclosures (alternative): |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Joint components |  |  |  |  |  |  | Pitch (mm) Required:  1.27 mm |  |  | Threads engaged |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | Tapered |  |  |  |  |  | Parallel |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 5H4h |  |  |  |  |  | 6H6g |  |  |  |  |  | 7H8g |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | Required |  |  |  | Actual |  | Required |  |  |  | Actual |  | Required |  |  | Actual |  |  | Required |  |  |  | Actual |  |  |  |  |
|  | 1 |  |  |  |  |  |  |  |  |  |  5 |  |  |  |  |  |  6 |  |  |  |  |  |  7 |  |  |  |  |  |  8 |  |  |  |  |  |  |  |  |
|  | 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 |  |  |  |  |  |  |  |  |  |  5 |  |  |  |  |  |  6 |  |  |  |  |  |  7 |  |  |  |  |  |  8 |  |  |  |  |  |  |  |  |
|  | 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Flameproof joints - Gaskets and O-rings |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Gasket of compressible material: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | ###### Joint components |  |  |  |  |  |  |  |  |  | ###### Gasket |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ###### In flame path: |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | ###### affixed to component: |  |  |  | ###### Material |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ###### Width L |  |  | ###### Distance I |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | ###### (X) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ###### NO | ###### YES |  | ###### NO | ###### YES |  |  |  |
|  | ###### 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | ###### 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | ###### 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | ###### 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Notes: (i) Not applicable to the entry of conductors and cables or to transparent elements of luminaires. (ii) Joints of bushings and light-transmitting parts may contain a gasket of metal or compressible non-flammable material (compressible material as determined by ISO 1210, with a metal sheath). (iii) The gasket shall be applied as a supplement to but not included in the flameproof joint. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Closure of flameproof joints per the requirements of applicable Tables I, IIA, IIB, IIC; for the joints of |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ( |  |  |  |  | components, |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ): |  |  |
|  |  |  |  |  | ###### gap i maintained |  |  |  |  | ###### width L maintained |  |  |  |  |  |  |  |  |  | ###### distance l maintained |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Sheet 15 of 52 	(IEC 60079-1, 1990, Amendments 1 & 2)

| ###### IEC 60079-1 |  |  |  |  |  | SECTION TWO  CONSTRUCTION 5/12 |  |  |  |  |  |  |  | APPLIED |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ###### CLAUSE |  |  |  |  |  | REQUIREMENTS |  |  |  |  |  |  |  | YES |  |  | NO |  | ENCL. |  |  |  |  |
| 4.5 |  |  |  |  |  | Flameproof joints - Cemented joints |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5 |  |  |  |  |  | Operating rods (spindles) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Item: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Flameproof joints - Cemented joints |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Refer to forms for IEC 60079-0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | strength of enclosure is not dependent upon cement/sealing material |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Shortest path through cemented joint (inside to outside enclosure), |  |  |  |  |  |  |  |  |  |  |  |  |  |  | mm for:- |  |  |  |  |  |  |  |  |
|  |  |  |  | enclosure volume  10 cm3 Required: path  3mm |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 10 cm3 enclosure volume  100 cm3 Required: path  6mm |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | enclosure volume  100 cm3 Required: path  10mm |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Joints exempted from compliance with path requirements (ref. 1/12 – 3/12) based on:- |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | parts cemented directly into enclosure wall; details: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | parts cemented into metallic frame as a replaceable unit; details: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Operating rods (spindles) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Dimensions (refer to Tables, I, IIA, IIB, IIC): |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Joint components |  |  |  |  |  |  |  | Enclosure volume (cm3) | Joint width L (mm) |  |  | Rod diameter (mm) |  |  |  |  | Joint width L (mm) [  L] |  | Diametrical clearance i (mm) |  |  |  |
|  |  |  |  |  |  |  |  |  |  | R | M |  |   L |  |  |   L |  | R |  | R | M |  |  |
|  | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| R = required; M = measured/actual |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Enlargement of diametrical clearance by wear in normal use: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | not applicable; reason: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | means to counteract enlargement: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Sheet 16 of 52 	(IEC 60079-1, 1990, Amendments 1 & 2)

| ###### IEC 60079-1 |  |  |  |  |  |  |  | SECTION TWO  CONSTRUCTION 6/12 |  |  |  |  |  |  |  |  |  | APPLIED |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ###### CLAUSE |  |  |  |  |  |  |  | REQUIREMENTS |  |  |  |  |  |  |  |  |  | YES |  |  |  | NO |  |  | ENCL. |  |  |  |  |  |
| 6 |  |  |  |  |  |  |  | Shafts and bearings |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7 |  |  |  |  |  |  |  | Light-transmitting parts |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Item: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Shafts and bearings |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Flameproof gland: |  |  |  |  |  |  |  |  |  | plain fixed |  |  |  |  | labyrinth |  |  |  |  |  |  |  | floating |  |  |  |  |  |  |  |
| Gland material: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ( |  |  | non-sparking) |  |  |  |  |  |  |  |
| Bearings: |  |  |  |  |  |  |  |  |  | sleeve |  |  |  |  |  | rolling-element |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Design features ensuring against wear of the gland by decentralization or wear of the bearing: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Refer to Tables I, IIA, IIB, IIC: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Gland/shaft joint for: |  |  |  |  |  |  |  |  | Enclosure volume (cm3) | Shaft dia. (mm) | Joint between gland and shaft |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Stator-rotor radial clearance, a (mm) |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | Length of flame path L (mm) |  |  |  |  | Diametrical clearance i (mm) |  |  |  | Maximum radial clearance, m (mm) |  |  | Minimum radial clearance, k (mm) |  | Radial movement g (mm) |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | R |  | M |  |  | R | M |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Notes: (i) R = required; M = measured. (ii) Sleeve bearings are not permitted for rotating electrical machines for Group IIC. (iii) Required:  k (ref. Fig. 18)  0.075 mm for Groups I, IIA, IIB and  0.05 mm for Group IIC  L  shaft diameter in a flameproof gland associated with a sleeve bearing (L need not exceed 25 mm)  a  g (ref. Fig. 19) for a rotating machine having sleeve bearings and using a plain fixed gland or labyrinth gland; alternatively (ref. Fig. 20), the gland shall be of non-sparking material  m   value of required i for shaft glands equipped with rolling-element bearings  L = uninterrupted length of a plain gland  i  0.1 mm for a plain gland |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Light-transmitting parts |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Part: |  |  |  |  |  |  |  |  |  |  |  | Transparent material: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | fixed directly into enclosure as an integral part by cement or sealing material |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | clamped directly into enclosure (with gasket) |  |  |  |  |  |  |  |  |  |  |  | clamped directly into enclosure (without gasket) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | sealed or cemented into a frame clamped into enclosure so that window is replaceable as a unit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | refer to 4/12, Gaskets and O-rings |  |  |  |  |  |  |  |  |  |  |  | refer to 5/12, Cemented joints |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | means preventing mechanical stress; details: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Relevant tests: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | thermal stability |  |  |  |  |  |  | impact |  |  |  |  | thermal shock |  |  |  |  |  |  |  | insulation resistance |  |  |  |  |  |  |  |
|  |  |  | refer to forms for IEC 60079-0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | refer to |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Sheet 17 of 52 	(IEC 60079-1, 1990, Amendments 1 & 2)

| ###### IEC 60079-1 |  |  |  |  |  |  | SECTION TWO  CONSTRUCTION 7/12 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | APPLIED |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ###### CLAUSE |  |  |  |  |  |  | REQUIREMENTS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | YES |  |  | NO |  |  |  |  | ENCL. |  |  |  |  |  |  |  |
| 8 |  |  |  |  |  |  | Breathing and draining devices |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 9 |  |  |  |  |  |  | Fasteners |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Item: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Breathing and draining devices |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Manufacturer and designation (if certified, details) |  |  |  |  |  |  |  |  |  |  |  |  |  | Certified (YES/NO) | Permeable element |  |  |  |  |  |  |  |  | Location |  |  |  |  |  |  |  |  |  |  |  | Can be taken apart (NO/YES) | Subject to clogging (NO/YES) |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | crimped ribbon | pressed wire | sintered |  |  | foam |  |  | other |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | (X) | (X) | (X) |  |  | (X) |  |  | (X) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | refer to Annex B  Breathing devices and draining devices |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Fasteners |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Parts fastened |  |  |  |  | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Parts fastened |  |  |  |  |  | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Number of fasteners, |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Number of fasteners, |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | *removable screw |  |  |  |  |  |  |  | *removable stud |  |  |  |  |  |  |  |  |  |  |  | *removable screw |  |  |  |  |  |  |  |  | *removable stud |  |  |  |  |  |  |  |  |  |  |
|  |  |  | in enclosure wall |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | in enclosure wall |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | diameter (D) of hole |  |  |  |  |  |  |  |  |  |  | mm |  |  |  |  |  |  |  |  | diameter (D) of hole |  |  |  |  |  |  |  |  |  |  |  |  | mm |  |  |  |  |  |  |
|  |  |  | screw/stud engages blind hole |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | screw/stud engages blind hole |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | **enclosure through-hole blinded by screwed plug |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | **enclosure through-hole blinded by screwed plug |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 3mm  metal thickness surrounding hole  D3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 3mm  metal thickness surrounding hole  D3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | free space at bottom of hole with fastener tight |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | free space at bottom of hole with fastener tight |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | permanently attached screw/stud; |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | permanently attached screw/stud; |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | welded |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | welded |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | riveted |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | riveted |  |  |  |  |  |  |  |  |  |  |
|  |  |  | secured against vibration; method: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | secured against vibration; method: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Notes: (i) *For fastener details refer to forms for IEC 60079-0, Fasteners. (ii) **For details of screwed plug used to blind an enclosure through-hole refer to 3/12 and 4/12, Threaded joints. (iii) For Group I enclosures refer to forms for IEC 60079-0, Fasteners (Special fasteners) where fasteners are used for securing doors, covers or blanking plates to the flameproof enclosure. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Sheet 18 of 52 	(IEC 60079-1, 1990, Amendments 1 & 2)

| ###### IEC 60079-1 |  |  | SECTION TWO  CONSTRUCTION 8/12 |  |  |  |  |  |  | APPLIED |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ###### CLAUSE |  |  | REQUIREMENTS |  |  |  |  |  |  | YES | NO |  | ENCL. |  |  |  |  |
| 10 |  |  | Mechanical strength of the enclosure |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 11 |  |  | Connection of conductors and cables |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Item: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Mechanical strength of the enclosure |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Single compartment enclosure: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | sub-divided by the disposition of internal parts of the apparatus |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | means to prevent pressure piling: |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Multiple enclosure compartments: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | isolated compartments |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | communicating compartments |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | means to prevent pressure piling: |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Mechanical strength of enclosure increased to allow for pressure piling; details: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Per SECTION THREE  VERIFICATIONS AND TESTS, 3/5, Overpressure test: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | no deformation/damage weakening enclosure or enlarging any joint so as to exceed required gap i |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Note: The above consideration applies to each compartment where two or more flameproof enclosures are incorporated together. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Liquid in enclosure: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | NO |  |  |  | YES (type, |  |  |  |  |  |  |  |  |  |  | ); |  |
|  |  |  |  |  | risk of producing explosive mixture more dangerous than allowed for enclosure: |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  | NO |  |  |  | YES |  |  |  |  |
| Connection of conductors and cables |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | means preventing disturbance of conductor terminations: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | provision for conduit connection (conduit size(s), |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ); |  |
|  |  |  |  |  | threaded for engagement of at least 5 threads |  |  |  |  |  |  |  |  |  |  |  |  |

Sheet 19 of 52 	(IEC 60079-1, 1990, Amendments 1 & 2)

| ###### IEC 60079-1 |  |  |  | SECTION TWO  CONSTRUCTION 9/12 |  |  |  |  |  |  |  | APPLIED |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ###### CLAUSE |  |  |  | REQUIREMENTS |  |  |  |  |  |  |  | YES |  | NO |  | ENCL. |  |  |
| 11.1 |  |  |  | ###### Connection of conductors and cables |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 11.2 |  |  |  | ###### Connection of conductors and cables; conduit connection |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 11.3, 11.3.1, 11.3.2 |  |  |  | ###### Connection of conductors and cables; indirect entry |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Item: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Connection of conductors and cables |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Indirect entry; means: |  |  |  |  | terminal box |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | plug and socket connection |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Direct entry, inside main enclosure |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Means preventing disturbance of conductor terminations: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Connection of conductors and cables; conduit connection |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Provision for conduit connection; conduit size(s), |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | threaded for engagement of at least 5 threads |  |  |  |  |  |  |  |  |  |  |  |  |
| Connection of conductors and cables; indirect entry |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Terminal box: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | flameproof terminal box; details: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | Certified ( |  |  |  |  |  |  |  |  |  |  | ) |  |
|  |  | Note: The requirements of 11/12 for cable/conductor entry are applicable. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | terminal box having other type of protection ( |  |  |  |  |  |  |  |  |  | ); details: |  |  |  |  |  |  |  |
|  |  |  |  |  |  | Certified ( |  |  |  |  |  |  |  |  |  |  | ) |  |
|  |  |  |  |  |  | refer to forms for specific type of protection, IEC 60079- |  |  |  |  |  |  |  |  |  |  |  |  |
| Connections between external conductors and cables and the circuits inside the flameproof enclosure: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | through bushings inserted in enclosure wall separating the chambers; details: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | refer to 1/12 – 4/12 for details of flameproof joints between components and enclosure |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | through sealing glands (alternative to bushings); details: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | refer to 1/12 – 4/12 for details of flameproof joints between components and enclosure |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Sheet 20 of 52 	(IEC 60079-1, 1990, Amendments 1 & 2)

| ###### IEC 60079-1 |  |  |  |  | SECTION TWO  CONSTRUCTION 10/12 |  |  |  |  |  | APPLIED |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ###### CLAUSE |  |  |  |  | REQUIREMENTS |  |  |  |  |  | YES | NO | ENCL. |  |  |  |
| 11.3.3, 11.3.3.1, 11.3.3.2, 11.3.3.3 |  |  |  |  | ###### Connection of conductors and cables; indirect entry (plug and socket connection) |  |  |  |  |  |  |  |  |  |  |  |
| Item: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Connection of conductors and cables; indirect entry (plug and socket connection) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Plug and socket; |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Certified ( |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ) |
|  | refer to (e.g. drawings): |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | refer to forms for IEC 60079-0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | flameproof properties of the apparatus enclosure are maintained with the plug and socket separated; details: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | refer to 1/12 – 4/12 for details of flameproof joints between plug/socket and enclosure |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | contacts in intrinsically-safe circuits; |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | contacts for earthing or bonding; |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | contact enclosure required to be flameproof “d” |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | plug and socket fixed together by special fasteners, in which case the requirements below do not apply |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | refer to forms for IEC 60079-0 |  |  |  |  |  |  |  |  |
| Chamber volume existing at the moment of separation of contacts in ignition-capable circuits, |  |  |  |  |  |  |  |  |  |  |  |  |  |  | cm3 |  |
| Plug/socket flameproof enclosure joint; details: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Note: Joint width and gap are determined by the volume existing at the moment of separation of contacts in ignition-capable circuits. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | refer to 1/12 (table for non-threaded joints) |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | refer to: |  |  |  |  |  |  |  |  |
| Means to maintain flameproof protection in the event of connector internal explosion: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | with plug/socket connected together; |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | at the moment of separation of contacts in ignition-capable circuits; |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Sheet 21 of 52 	(IEC 60079-1, 1990, Amendments 1 & 2)

| ###### IEC 60079-1 |  |  |  |  | SECTION TWO  CONSTRUCTION 11/12 |  |  |  |  |  |  |  |  |  |  |  | APPLIED |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ###### CLAUSE |  |  |  |  | REQUIREMENTS |  |  |  |  |  |  |  |  |  |  |  | YES | NO | ENCL. |  |
| 11.4 |  |  |  |  | ###### Connection of conductors and cables; direct entry |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Item: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Connection of conductors and cables; direct entry |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Entry method applied to:- |  |  |  |  |  |  |  |  | cable |  |  |  |  |  |  | conductors |  |  |  |  |
|  |  |  |  |  |  |  |  |  | flameproof terminal box for indirect entry (ref. 9/12) |  |  |  |  |  |  |  |  |  |  |  |
| Method: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | refer to forms for IEC 60079-0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | refer to 1/12 – 4/12 for details of flameproof joints between components and enclosure |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | packing gland; details: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | sealing material/application; details: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | cable sealed into main enclosure; details: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| cable; |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  | Certified ( |  |  |  |  |  |  |  |  |  |  | ) |
|  |  | length of attached cable, |  |  |  |  |  |  |  |  | m Required:  1m |  |  |  |  |  |  |  |  |  |
| Minimum width of seal, |  |  |  |  |  |  |  |  |  | mm (packing material compressed): Ref. Figures 21, 22, 23. |  |  |  |  |  |  |  |  |  |  |
|  |  minimum width of flame path required ( |  |  |  |  |  |  |  |  |  |  |  |  |  | mm) for enclosure volume Ref. Tables I, IIA, IIB, IIC |  |  |  |  |  |
| Conduit attachment; enclosure entry means for conductors/cable : |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | compound-filled box; details: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | compound-filled recess; details: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Note: For entry means not integral with enclosure refer to applicable requirements of 1/12-4/12. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Sheet 22 of 52 	(IEC 60079-1, 1990, Amendments 1 & 2)

| IEC 60079-1 |  |  | SECTION TWO  CONSTRUCTION 12/12 |  |  |  |  | APPLIED |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CLAUSE |  |  | REQUIREMENTS |  |  |  |  | YES | NO | ENCL. |  |
| 12 |  |  | Marking |  |  |  |  |  |  |  |  |
| Item: |  |  |  |  |  |  |  |  |  |  |  |
| Note: For major marking content refer to forms for IEC 60079-0, Marking. This sheet covers specific marking for the type of protection. Marking |  |  |  |  |  |  |  |  |  |  |  |
| Cover(s) giving access to enclosure interior and not interlocked with a disconnector: |  |  |  |  |  |  |  |  |  |  |  |
|  | refer to forms for IEC 60079-0 |  |  |  |  |  |  |  |  |  |  |
|  | label(s) affixed, indicating that cover(s) shall not be opened with the apparatus energized; wording: |  |  |  |  |  |  |  |  |  |  |
|  | label material: |  |  |  |  |  |  |  |  |  |  |
|  | label fixing method: |  |  |  |  |  |  |  |  |  |  |
|  | documentation; ref. |  |  |  |  |  |  |  |  |  |  |
| Indication of special conditions for safe use: |  |  |  |  |  |  |  |  |  |  |  |
| refer to ANNEX B  BREATHING DEVICES AND DRAINING DEVICES, Test for non-transmission of an internal ignition, Group IIC; devices with measurable paths (required additional tests) and devices with non-measurable paths (Method B) |  |  |  |  |  |  |  |  |  |  |  |
|  | apparatus is marked with the sign X, indicating special conditions noted in the certificate: |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | ethyl nitrate is excluded |  |  |  |  |
|  |  |  |  |  |  |  | carbon disulfide is excluded (for enclosure volume  100 cm3) |  |  |  |  |

Sheet 23 of 52 	(IEC 60079-1, 1990, Amendments 1 & 2)

| ###### IEC 60079-1 |  |  | SECTION THREE  VERIFICATIONS AND TESTS 1/5 |  |  |  |  |  |  |  |  | APPLIED |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ###### CLAUSE |  |  | REQUIREMENTS |  |  |  |  |  |  |  |  | YES | NO |  | ENCL. |  |
| 13 |  |  | General |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 |  |  | Type tests |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14.1 |  |  | Tests of ability of the enclosure to withstand pressure |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14.1.1 |  |  | Determination of explosion pressure |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Item: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| General |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Refer to forms for IEC 60079-0 for prerequisite verifications and tests |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type tests (with reference to given order) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Conducted in order |  |  |  |  | other; |  |  |  |  |  |  |  |  |  |  |
| Tests of ability of the enclosure to withstand pressure |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Enclosure tested with:- |  |  |  |  |  | all internal apparatus in place |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | internal apparatus simulated by |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | all or part of the internal apparatus removed; details: |  |  |  |  |  |  |  |  |  |  |
|  | Notes: 1. Tests with internal apparatus removed shall consider the possibility of removal according to features of design and shall represent the most severe conditions as determined by the testing laboratory. 2. The reported results of the following tests shall consider enclosure damage, permanent deformation liable to weaken any enclosure parts or permanent enlargement of enclosure joints. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Determination of explosion pressure (refer to 2/5, Reference pressure table) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Pressure measurement by (pressure sensing device/circuit used to measure maximum smoothed pressure): |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Note: Smoothed pressure can be achieved by inserting a 5 kHz  10% filter in the pressure signal circuit. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Ignition source: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | high voltage sparking plug ( |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ) |
|  | low-energy ignition source ( |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ) |
|  | contained ignition-capable switching device ( |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ) |
| Detachable gaskets in place: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | YES |  |  |  |  | NO; reason: |  |  |  |  |  |  |  |  |  |  |
| Rotating electrical machines: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | running tests at |  |  |  |  | rpm, based on maximum rated speed of |  |  |  |  |  |  |  | rpm |  |  |
|  | running tests waived (reason): |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | tests at rest waived (reason): |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| continued on 2/5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Sheet 24 of 52 	(IEC 60079-1, 1990, Amendments 1 & 2)

| ###### IEC 60079-1 |  |  |  | SECTION THREE  VERIFICATIONS AND TESTS 2/5 |  |  |  |  |  |  |  |  |  |  | APPLIED |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ###### CLAUSE |  |  |  | REQUIREMENTS |  |  |  |  |  |  |  |  |  |  | YES |  | NO | ENCL. |  |  |  |  |
| 14.1.1 |  |  |  | Determination of explosion pressure (continued) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Item: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Determination of explosion pressure (continued) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reference pressure: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Test No | Ignition source location |  |  |  |  | Test mixture |  | Rotating machine |  |  |  | Pressure |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  | (bar) | Measured at: |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | (a – f) | C |  |  |  |  |  |  |  |  |  |  | ignition end | opposite end |  |  |
|  |  |  |  |  |  |  |  |  | ON |  | OFF |  |  |  |  |  |  |  | (X) | (X) |  |  |
|  |  | (1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | (2) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | (3) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | (4) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | (5) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Notes (explosive test mixture, concentration in air based on volumetric ratio at atmospheric pressure; ref. Table IV): (a) Group I; 9.8  0.5 % methane (b) Group IIA; 4.6  0.3 % propane (c) Group IIB; 8.0  0.5 % ethylene (pressure piling excl.) (d) Group IIC; 31  1.0 % hydrogen (e) Group IIC; 14.0  0.5 % acetylene (f) Group IIB; 24  1 % hydrogen-methane (85/15  1 %) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Damage/deformation: |  |  |  |  |  | NO |  |  |  |  |  | YES; Test No |  |  |  |  |  |  |  |  |  |  |
| Joint enlargement: |  |  |  |  |  | NO |  |  |  |  |  | YES; Test No |  |  |  |  |  |  |  |  |  |  |

Sheet 25 of 52 	(IEC 60079-1, 1990, Amendments 1 & 2)

| ###### IEC 60079-1 |  |  |  |  | SECTION THREE  VERIFICATIONS AND TESTS 3/5 |  |  |  |  |  |  |  |  |  |  |  |  |  | APPLIED |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ###### CLAUSE |  |  |  |  | REQUIREMENTS |  |  |  |  |  |  |  |  |  |  |  |  |  | YES |  |  | NO |  | ENCL. |  |  |  |  |
| 14.1.2 |  |  |  |  | Overpressure test |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Item: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Overpressure test |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test method: |  |  |  |  |  |  | static |  |  |  |  | dynamic |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Static test: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Test No | Compartment |  |  |  |  |  | Damage or deformation |  |  |  |  | Joint enlargement |  |  |  | Subjected to routine testing |  |  |  | Volume (cm3) |  | Test pressure |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | (bar) |  | Duration (s) | Based on reference pressure: (bar) |  |  |
|  |  |  |  |  |  |  |  | NO |  | YES |  |  | NO |  | YES |  | YES |  | NO |  |  |  |  |  |  |  |  |  |
|  | Required 1. For enclosures of volume  10 cm3, which are not subjected to routine tests, applied test pressure = 4  max. reference pressure; otherwise 3.5 bar  applied test pressure = 1.5  max. reference pressure. Refer to 2/5 for reference test pressures. 3. Duration of test pressure application, 10 (+2 - 0) s. 4. Applied test pressure of 10 bar (Groups I, IIA, IIB) or 15 bar (Group IIC) where reference pressure measurement and dynamic method are not practicable. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Dynamic test: (ref. 1/5 for ignition source) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Test No | Reference pressure test (location No) |  | Compartment |  |  |  |  |  |  | Damage or deformation |  |  |  |  | Joint enlargement |  |  |  | Explosive mixture |  |  |  |  | Test pressure |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Test mixture per reference pressure test |  |  | Initial pressure (bar) |  | (bar) | Based on reference pressure: (bar) |  |  |
|  |  |  |  |  |  |  |  |  |  |  | NO |  |  | YES |  | NO |  | YES |  | (a – f) |  | C |  |  |  |  |  |  |
| Required 1. Maximum test pressure = 1.5  reference pressure. 2. Rate of rise of pressure shall closely approximate that obtained during determination of reference pressure. 3. Where reference pressure measurement is not practicable, the test shall be made by explosive mixture at 1.5  atmospheric pressure. 4. For Group IIC the test shall be made three times with each explosive test mixture. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Sheet 26 of 52 	(IEC 60079-1, 1990, Amendments 1 & 2)

| ###### IEC 60079-1 |  |  |  |  | SECTION THREE  VERIFICATIONS AND TESTS 4/5 |  |  |  |  |  |  |  | APPLIED |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ###### CLAUSE |  |  |  |  | REQUIREMENTS |  |  |  |  |  |  |  | YES |  | NO |  |  | ENCL. |  |  |  |  |  |
| 14.2 |  |  |  |  | Test for non-transmission of an internal ignition |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Item: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test for non-transmission of an internal ignition |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | enclosure placed in test chamber (volume |  |  |  |  |  |  |  |  |  |  | ) |  |  |  |  |  |  |  |  |  |  |
| Note: For tests per Group IIC, Second method the volume ratio of the test chamber to the enclosure should be at least 5 to 1. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Test No |  | Ignition source (1/5) |  |  | Explosive test mixture (Notes 1, 2) in test chamber and inside enclosure | Test conditions for Group II Group I enclosure tested in its normal condition per Note 4 |  |  |  |  |  |  |  |  |  |  |  |  |  | Flame transmission (NO/YES) |  |  |
|  |  |  |  |  |  |  | IIA, IIB |  |  |  | IIC; First method (increased joint gap incl. bearings, operating rods) |  |  | IIA, IIB (Note 2) or IIC; First method |  |  |  |  | IIC Second method (Note 4) |  |  |  |  |
|  |  |  |  |  |  |  | Increased joint gap (Note 2) |  |  | Normal condition (Note 4) | cylindrical (incl. spigot, width L) | flanged ( 0.1mm) and cylindrical (incl. spigot, width L) | flanged only |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | Reduction of length of thread engagement (Note 5) |  |  |  |  | Test mixture pressure vs atmospheric pressure |  |  |  |  |
|  |  |  | Location (or location No for reference pressure test) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | 1.42 iC (mm) Group IIA |  | 1.85 iC (mm) Group IIB |  | iC +  iT (mm) | iC +  iC (mm) | 1.5 iT (mm) | none (taper) |  |  |  |  | 1.5  | 1.2(iCiE)  (Note 6) |  |  |  |
|  |  |  |  |  |  | (a – h) |  |  |  | (X) |  |  |  | (X) |  | (X) | (X) |  | (X) |  |  |  |  |
| Notes 1. Explosive mixture, concentration in air based on volumetric ratio at atmospheric pressure; ref. Table IV: (a) Group I; 12.5  0.5 % methane-hydrogen (58/42  1 %) (b) Group IIA; 55  1.0 % hydrogen (c) Group IIB; 37  1.0 % hydrogen (d) Group IIC; 27  1.0 % hydrogen (e) Group IIC; 7.5  1 % acetylene 2. Alternative test mixtures for Groups IIA and IIB where damage by test mixture b or c is possible; ref. Cl. 14.2.1.2: (f) Group IIA; 4.2  0.1 % propane (g) Group IIB; 6.5  0.5 % ethylene (h) Group IIB; 19  1 % hydrogen-methane (85/15) 3. iC = the maximum constructional gap, as specified in the manufacturer's drawings iE = the test gap iT = the maximum gap permitted in Table I, IIA, IIB or IIC as applicable 4. Where no artificial gap is created, the test gap shall be 80 to 100 % of the specified maximum constructional gap, but not exceeding iT. 5. Required reduction of length of thread engagement (Groups IIA and IIB where an alternative test mixture is used; Group IIC, first method): - by  of manufacturer's specification for joints conforming to ISO fit - by  of manufacturer's specification for joints worse than ISO fit - - no reduction of taper-threaded joints 6. For Group IIC, second method (tested in normal condition without an artificial gap), test mixture pressure shall be 1.2(iCiE)  atmospheric pressure if the maximum test gap is less than 80 % of the maximum specified constructional gap. 7. For Group IIC, in the case of limited production, each product (having joints within manufacturing tolerances) shall be tested five times, using the test mixture per Note 1 (d) or 1 (e) at atmospheric pressure. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Sheet 27 of 52 	(IEC 60079-1, 1990, Amendments 1 & 2)

| ###### IEC 60079-1 |  |  | SECTION THREE  VERIFICATIONS AND TESTS 5/5 |  |  |  |  |  |  | APPLIED |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ###### CLAUSE |  |  | REQUIREMENTS |  |  |  |  |  |  | YES | NO | ENCL. |  |
| 15 |  |  | Routine tests |  |  |  |  |  |  |  |  |  |  |
| Item: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Routine tests (refer to 3/5, Overpressure test) |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Not required because:- |  |  |  | enclosure does not involve welded construction |  |  |  |  |  |  |  |  |
| and |  |  |  |  | volume  10 cm3 ( |  |  |  | cm3) or |  |  |  |  |
|  |  |  |  |  | volume > 10 cm3 and |  |  | static type test at 4  max. reference pressure |  |  |  |  |  |
| Routine test: |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | enclosure of welded construction Required for welded construction: routine tests on 100% of production. |  |  |  |  |  |  |  |  |  |  |  |  |
|  | based on static type test |  |  |  |  |  |  |  |  |  |  |  |  |
|  | based on dynamic type test; method:- |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | explosion test using appropriate mixture per Table IV, inside and outside enclosure, at 1.5  atmospheric pressure or |  |  |  |  |  |  |  |  |
|  |  |  |  |  | explosion test using mixture for Tests for non-transmission of an internal ignition (4/5), inside and outside enclosure, at atmospheric pressure, preceded by dynamic type test or |  |  |  |  |  |  |  |  |
|  |  |  |  |  | static type test at pressure  2 bar, preceded by dynamic type test |  |  |  |  |  |  |  |  |
|  | static test at relative pressure (reference pressure measurement and dynamic method not practicable):- |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 10 bar (Group I, IIA or IIB) |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 15 bar (Group C) |  |  |  |  |  |  |  |  |
| Method/conditions of routine tests subject to agreement between the testing laboratory and the manufacturer: |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Notes: 1. Empty enclosures may be tested. 2. Each of the parts forming the enclosure may be tested separately with similar stressing compared to the complete enclosure. 3. Consideration shall be given to the influence by the enclosed apparatus on rise of pressure during an internal explosion if the routine test is dynamic. 4. Observations shall be made as to structural damage or permanent deformation of the enclosure that could affect its flameproof properties as a result of the tests. 5. In the case of welded enclosures, observations shall also be made as to the presence of holes or cracks reducing the flameproof properties of the enclosure or any part of it. |  |  |  |  |  |  |  |  |  |  |  |  |

Sheet 28 of 52 	(IEC 60079-1, 1990, Amendments 1 & 2)

| ###### IEC 60079-1 |  |  |  |  | Amendment 1, 1993-08 ANNEX A  NON-METALLIC PARTS OF FLAMEPROOF ENCLOSURES 1/12 |  |  |  |  |  |  | APPLIED |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ###### CLAUSE |  |  |  |  | REQUIREMENTS |  |  |  |  |  |  | YES |  |  | NO |  | ENCL. |  |  |  |  |  |
| A.1 |  |  |  |  | Scope |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| A.2 |  |  |  |  | Special requirements for construction |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| A.3.1.1 |  |  |  |  | Ambient temperature during tests |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| A.3.1.2 |  |  |  |  | ###### Sequence of tests |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Item: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Scope (Annex A does not apply to sealing rings of cable entries or non-metallic parts on which the type of protection does not depend.) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Non-metallic enclosure; details: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Non-metallic enclosure part(s); details: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | *volume  3000 cm3 |  |  |  |  |  | *volume  3000 cm3 and |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | *surface area of (each) non-metallic part  500 cm2 except, |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | *surface area of light-transmitting part (luminaire)  8000 cm2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| *Note: For information purposes only (ref. Appendix A) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ###### Special requirements for construction |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Resistance to tracking/creepage distances for non-metallic material supporting live bare parts: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Grade of insulating material |  |  |  | Between (conductive parts): |  |  | Working voltage U |  |  | Distance (mm) |  |  |  | grooves |  | CTI |  |  |
|  |  |  |  | Component |  |  |  |  |  |  |  |  |  | Measured |  | Required |  |  | ribs |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | (X) | (X) |  |  |  |
| Required (i) CTI and creepage distances on internal surfaces shall comply with IEC 60079-7 (ref. IEC 60079-7, Clause 4.4 and Tables 2 and 3). (ii) Surface ribs  3 mm high,  1 mm thick, or grooves  3mm deep,  3mm wide if taken into account for reduced creepage distance. (iii) Group I; CTI  400 M (ref. IEC 60112) for insulating materials subject to arc-capable electrical stresses due to rated current  16 A. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Ambient temperature during tests |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Where applicable, upper and lower test ambients comply with the following criteria: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| max. service ambient, |  |  |  |  |  |  |  |  | C Required: upper test ambient = max. service ambient + 10 to 15 K |  |  |  |  |  |  |  |  |  |  |  |  |  |
| min. service ambient, |  |  |  |  |  |  |  |  | C Required: lower test ambient = max. service ambient  5 to 10 K |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ###### Sequence of tests |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | tests were conducted per sequences as follows, using a total of |  |  |  |  |  |  |  |  |  |  | samples Required: two each |  |  |  |  |  |  |  |  |  |
|  |  | ###### samples of the non-metallic enclosure or parts identified above were tested |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | other samples tested: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Sheet 29 of 52 	(IEC 60079-1, 1990, Amendments 1 & 2)

| ###### IEC 60079-1 |  |  |  |  |  | Amendment 1, 1993-08 ANNEX A  NON-METALLIC PARTS OF FLAMEPROOF ENCLOSURES 2/12 |  |  |  |  |  |  |  |  |  |  |  | APPLIED |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ###### CLAUSE |  |  |  |  |  | REQUIREMENTS |  |  |  |  |  |  |  |  |  |  |  | YES |  | NO | ENCL. |  |
| A.3.1.3 |  |  |  |  |  | Thermal endurance to heat |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| A.3.1.4 |  |  |  |  |  | Thermal endurance to cold |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | Mechanical tests |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | GROUP I APPARATUS (1ST test sequence) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Item: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ###### Thermal endurance to heat |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Maximum service temperature  75 C: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | continuous storage for |  |  |  |  |  |  |  |  |  | weeks Required: four weeks |  |  |  |  |  |  |  |  |  |  |  |
|  | relative humidity, |  |  |  |  |  |  |  |  |  | % Required: 90  5% |  |  |  |  |  |  |  |  |  |  |  |
|  | temperature, |  |  |  |  |  |  | C Required: 80 C  (20  2) C above max. service temperature |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Maximum service temperature  75 C: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | continuous storage for |  |  |  |  |  |  |  |  |  | weeks at |  |  |  |  | C and |  |  | % relative humidity |  |  |  |
| Required: two weeks at (95  2) C and (90  5)% relative humidity |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| and |  |  |  |  |  |  |  |  |  |  | weeks at |  |  |  |  | C |  |  |  |  |  |  |
| Required: two weeks at (20  2) C above max. service temperature and prevailing humidity |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Observations : |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ###### Thermal endurance to cold |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | continuous storage for |  |  |  |  |  |  |  |  |  |  | h at |  |  | C |  |  |  |  |  |  |  |
| Required: 24 h at lower test ambient |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Observations: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Mechanical tests |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Applicable test(s): |  |  |  |  |  |  |  |  |  | conducted on component part, assembled to enclosure |  |  |  |  |  |  |  |  |  |  |  |  |
|  | refer to forms for IEC 60079-0, Test for resistance to impact |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | refer to forms for IEC 60079-0, Drop test (portable apparatus) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | refer to: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Results: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| *Note: Special conditions shall be stated if any of the conditioned enclosure samples do not withstand the applicable Mechanical tests. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Sheet 30 of 52 	(IEC 60079-1, 1990, Amendments 1 & 2)

| ###### IEC 60079-1 |  |  |  |  |  |  | Amendment 1, 1993-08 ANNEX A  NON-METALLIC PARTS OF FLAMEPROOF ENCLOSURES 3/12 |  |  |  |  |  |  |  |  |  |  |  |  | APPLIED |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ###### CLAUSE |  |  |  |  |  |  | REQUIREMENTS |  |  |  |  |  |  |  |  |  |  |  |  | YES |  |  |  |  | NO |  |  |  |  |  | ENCL. |  |  |  |  |  |  |  |  |
| A.3.2 |  |  |  |  |  |  | ###### Tests for flameproofness |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ###### A.3.3 |  |  |  |  |  |  | ###### Flammability |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | GROUP I APPARATUS (1ST test sequence, continued) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Item: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Tests for flameproofness |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | The following tests were conducted in the order given on enclosures subjected to tests per 2/12 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test of ability of the enclosure to withstand pressure |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | refer to SECTION THREE  VERIFICATIONS AND TESTS, 1/5 through 3/5 (Pages |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ) |  |
|  |  | ###### refer to: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test of erosion by flame (to be considered where enclosure flameproof joints have at least one face of plastics material) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | ###### not applicable (i.e. flameproof joints do not involve plastic material; details: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ) |  |  |
|  |  | not applicable (i.e. volume  100 cm3 and material withstands Flammability test below) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | static gap (flanged joint/plane part of spigot joint) set to |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | mm Required: 0.1mm  value  0.15mm |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| or |  |  |  |  |  |  |  |  | gap set to maximum permitted value ( 0.15 mm); |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | mm |  |  |  |  |  |  |
|  |  | explosive test mixture; 9.8  0.5 methane in air (test ambient, |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | C) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | ignitions by (ignition source) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (ignition source location, |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ) |  |  |
| Required: 50 ignitions of the explosive mixture per Table IV for determination of explosion pressure |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test for non-transmission of an internal ignition |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | refer to SECTION THREE  VERIFICATIONS AND TESTS, 4/5 (Page |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ) |  |  |  |  |  |  |  |  |  |
| Flammability |  |  |  |  |  |  |  |  | conducted on test specimens conditioned in accordance with 2/12 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Component |  |  |  |  |  |  |  |  |  | Test specimens |  |  |  |  |  | Test method |  |  |  |  |  |  | Worst case result after flame applications to each specimen (NO/YES) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | number of pieces | number of sets | cut from enclosure | moulded |  | cut from plates | ISO 1210 |  | first alternative |  | second alternative 125133 mm specimens |  |  | ISO 1210 |  |  |  | Alternative methods |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | burning  15 s |  |  | specimen burnt completely | flaming  10 s |  |  |  | flaming  50s total for 10 applications |  | specimen burnt up to holding clamp | cotton layer ignited | glowing  30 s |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  | (X) | (X) |  | (X) | (X) |  | (X) |  | (X) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Notes (i) Test pieces moulded as individual pieces or plates from which the test pieces are cut shall be produced under conditions equivalent to those used to produce the actual enclosure and the conditions shall be recorded in the manufacturer's documentation. (ii) One of the alternative test methods shall be applied if the test per ISO 1210 results in distortion of the test piece out of the flame. (iii) The first alternative test method shall be in accordance with IEC 60707 (Method FV: Flame – Vertical specimen). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Sheet 31 of 52 	(IEC 60079-1, 1990, Amendments 1 & 2)

| ###### IEC 60079-1 |  |  |  |  |  | Amendment 1, 1993-08 ANNEX A  NON-METALLIC PARTS OF FLAMEPROOF ENCLOSURES 4/12 |  |  |  |  |  |  |  | APPLIED |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ###### CLAUSE |  |  |  |  |  | REQUIREMENTS |  |  |  |  |  |  |  | YES | NO | ENCL. |  |  |  |
| A.3.1.6 |  |  |  |  |  | ###### Resistance to chemical agents (oils and greases) |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | ###### Mechanical tests |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | GROUP I APPARATUS (2ND test sequence) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Item: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ###### Resistance to chemical agents (oils and greases) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ###### Enclosure samples were conditioned as follows: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | ###### sealed against intrusion of test liquids (method); |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | immersed for |  |  |  |  |  |  |  | h in No. 2 oil (ref. ISO 1817) at |  |  |  |  | C |  |  |  |  |  |
|  | Required: (24  2) h at 50 C |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | samples removed from liquid bath and stored for |  |  |  |  |  |  |  |  |  |  | h in laboratory atmosphere (ambient, |  |  |  |  |  | C) |  |
|  | Required: samples carefully wiped and stored for 24 h |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Observations: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Mechanical tests |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Applicable test(s): |  |  |  |  |  |  |  |  |  | conducted on component part, assembled to enclosure |  |  |  |  |  |  |  |  |  |
|  | refer to forms for IEC 60079-0, Test for resistance to impact |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | refer to forms for IEC 60079-0, Drop test (portable apparatus) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | refer to: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Results: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Note: Special conditions shall be stated if any of the conditioned enclosure samples do not withstand the applicable Mechanical tests. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Sheet 32 of 52 	(IEC 60079-1, 1990, Amendments 1 & 2)

| ###### IEC 60079-1 |  |  |  |  |  |  | Amendment 1, 1993-08 ANNEX A  NON-METALLIC PARTS OF FLAMEPROOF ENCLOSURES 5/12 |  |  |  |  |  |  |  |  |  |  |  |  | APPLIED |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ###### CLAUSE |  |  |  |  |  |  | REQUIREMENTS |  |  |  |  |  |  |  |  |  |  |  |  | YES |  |  |  |  | NO |  |  |  |  | ENCL. |  |  |  |  |  |  |  |  |  |
| A.3.2 |  |  |  |  |  |  | ###### Tests for flameproofness |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ###### A.3.3 |  |  |  |  |  |  | ###### Flammability |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | GROUP I APPARATUS (2ND test sequence, continued) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Item: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Tests for flameproofness |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | The following tests were conducted in the order given on enclosures subjected to tests per 4/12 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test of ability of the enclosure to withstand pressure |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | refer to SECTION THREE  VERIFICATIONS AND TESTS, 1/5 through 3/5 (Pages |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ) |  |
|  |  | ###### refer to: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test of erosion by flame (to be considered where enclosure flameproof joints have at least one face of plastics material) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | ###### not applicable (i.e. flameproof joints do not involve plastic material; details: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ) |  |  |
|  |  | not applicable (i.e. volume  100 cm3 and material withstands Flammability test below) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | static gap (flanged joint/plane part of spigot joint) set to |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | mm Required: 0.1mm  value  0.15mm |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| or |  |  |  |  |  |  |  |  | gap set to maximum permitted value ( 0.15 mm); |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | mm |  |  |  |  |  |  |  |
|  |  | explosive test mixture; 9.8  0.5 methane in air (test ambient, |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | C) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | ignitions by (ignition source) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (ignition source location, |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ) |  |  |
| Required: 50 ignitions of the explosive mixture per Table IV for determination of explosion pressure |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test for non-transmission of an internal ignition |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | refer to SECTION THREE  VERIFICATIONS AND TESTS, 4/5 (Page |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ) |  |  |  |  |  |  |  |  |  |  |
| Flammability |  |  |  |  |  |  |  |  | conducted on test specimens conditioned in accordance with 4/12 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Component |  |  |  |  |  |  |  |  |  | Test specimens |  |  |  |  |  | Test method |  |  |  |  |  |  | Worst case result after flame applications to each specimen (NO/YES) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | number of pieces | number of sets | cut from enclosure | moulded |  | cut from plates | ISO 1210 |  | first alternative |  | second alternative 125133 mm specimens |  |  | ISO 1210 |  |  |  | Alternative methods |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | burning  15 s |  |  | specimen burnt completely | flaming  10 s |  |  | flaming  50s total for 10 applications |  |  | specimen burnt up to holding clamp | cotton layer ignited | glowing  30 s |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  | (X) | (X) |  | (X) | (X) |  | (X) |  | (X) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Notes (i) Test pieces moulded as individual pieces or plates from which the test pieces are cut shall be produced under conditions equivalent to those used to produce the actual enclosure and the conditions shall be recorded in the manufacturer's documentation. (ii) One of the alternative test methods shall be applied if the test per ISO 1210 results in distortion of the test piece out of the flame. (iii) The first alternative test method shall be in accordance with IEC 60707 (Method FV: Flame – Vertical specimen). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Sheet 33 of 52 	(IEC 60079-1, 1990, Amendments 1 & 2)

| ###### IEC 60079-1 |  |  |  |  |  | Amendment 1, 1993-08 ANNEX A  NON-METALLIC PARTS OF FLAMEPROOF ENCLOSURES 6/12 |  |  |  |  |  | APPLIED |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ###### CLAUSE |  |  |  |  |  | REQUIREMENTS |  |  |  |  |  | YES | NO | ENCL. |  |  |  |  |
| A.3.1.6 |  |  |  |  |  | ###### Resistance to chemical agents (hydraulic liquids) |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | ###### Mechanical tests |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | GROUP I APPARATUS (3RD test sequence) |  |  |  |  |  |  |  |  |  |  |  |  |
| Item: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ###### Resistance to chemical agents (hydraulic liquids) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ###### Enclosure samples were conditioned as follows: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | ###### sealed against intrusion of test liquids (method); |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | immersed for |  |  |  |  |  |  |  | h in an aqueous solution of ethylene glycol in 35% water by volume |  |  |  |  |  |  |  |  |  |
|  | Required: (24  2) h |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | samples were removed from liquid bath and stored for |  |  |  |  |  |  |  |  |  | h in laboratory atmosphere (ambient, |  |  |  |  | C) |  |  |
|  | Required: samples carefully wiped and stored for 24 h |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Observations: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Mechanical tests |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Applicable test(s): |  |  |  |  |  |  |  |  | conducted on component part, assembled to enclosure |  |  |  |  |  |  |  |  |  |
|  | refer to forms for IEC 60079-0, Test for resistance to impact |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | refer to forms for IEC 60079-0, Drop test (portable apparatus) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | refer to: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Results: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| *Note: Special conditions shall be stated if any of the conditioned enclosure samples do not withstand the applicable Mechanical tests. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Sheet 34 of 52 	(IEC 60079-1, 1990, Amendments 1 & 2)

| ###### IEC 60079-1 |  |  |  |  |  |  | Amendment 1, 1993-08 ANNEX A  NON-METALLIC PARTS OF FLAMEPROOF ENCLOSURES 7/12 |  |  |  |  |  |  |  |  |  |  |  |  | APPLIED |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ###### CLAUSE |  |  |  |  |  |  | REQUIREMENTS |  |  |  |  |  |  |  |  |  |  |  |  | YES |  |  |  |  | NO |  |  |  |  | ENCL. |  |  |  |  |  |  |  |  |  |
| A.3.2 |  |  |  |  |  |  | ###### Tests for flameproofness |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ###### A.3.3 |  |  |  |  |  |  | ###### Flammability |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | GROUP I APPARATUS (3RD test sequence, continued) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Item: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Tests for flameproofness |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | The following tests were conducted in the order given on enclosures subjected to tests per 6/12 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test of ability of the enclosure to withstand pressure |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | refer to SECTION THREE  VERIFICATIONS AND TESTS, 1/5 through 3/5 (Pages |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ) |  |
|  |  | ###### refer to: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test of erosion by flame (to be considered where enclosure flameproof joints have at least one face of plastics material) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | ###### not applicable (i.e. flameproof joints do not involve plastic material; details: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ) |  |  |
|  |  | not applicable (i.e. volume  100 cm3 and material withstands Flammability test below) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | static gap (flanged joint/plane part of spigot joint) set to |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | mm Required: 0.1mm  value  0.15mm |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| or |  |  |  |  |  |  |  |  | gap set to maximum permitted value ( 0.15 mm); |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | mm |  |  |  |  |  |  |  |
|  |  | explosive test mixture; 9.8  0.5 methane in air (test ambient, |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | C) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | ignitions by (ignition source) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (ignition source location) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Required: 50 ignitions of the explosive mixture per Table IV for determination of explosion pressure |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test for non-transmission of an internal ignition |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | refer to SECTION THREE  VERIFICATIONS AND TESTS, 4/5 (Page |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ) |  |  |  |  |  |  |  |  |  |  |
| Flammability |  |  |  |  |  |  |  |  | conducted on test specimens conditioned in accordance with 6/12 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Component |  |  |  |  |  |  |  |  |  | Test specimens |  |  |  |  |  | Test method |  |  |  |  |  |  | Worst case result after flame applications to each specimen (NO/YES) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | number of pieces | number of sets | cut from enclosure | moulded |  | cut from plates | ISO 1210 |  | first alternative |  | second alternative 125133 mm specimens |  |  | ISO 1210 |  |  |  | Alternative methods |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | burning  15 s |  |  | specimen burnt completely | flaming  10 s |  |  | flaming  50s total for 10 applications |  |  | specimen burnt up to holding clamp | cotton layer ignited | glowing  30 s |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  | (X) | (X) |  | (X) | (X) |  | (X) |  | (X) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Notes (i) Test pieces moulded as individual pieces or plates from which the test pieces are cut shall be produced under conditions equivalent to those used to produce the actual enclosure and the conditions shall be recorded in the manufacturer's documentation. (ii) One of the alternative test methods shall be applied if the test per ISO 1210 results in distortion of the test piece out of the flame. (iii) The first alternative test method shall be in accordance with IEC 60707 (Method FV: Flame – Vertical specimen). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Sheet 35 of 52 	(IEC 60079-1, 1990, Amendments 1 & 2)

| ###### IEC 60079-1 |  |  |  |  | Amendment 1, 1993-08 ANNEX A  NON-METALLIC PARTS OF FLAMEPROOF ENCLOSURES 8/12 |  |  |  |  |  |  |  |  |  |  |  |  | APPLIED |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ###### CLAUSE |  |  |  |  | REQUIREMENTS |  |  |  |  |  |  |  |  |  |  |  |  | YES |  | NO | ENCL. |  |
| A.3.1.3 |  |  |  |  | Thermal endurance to heat |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| A.3.1.4 |  |  |  |  | Thermal endurance to cold |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Mechanical tests |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | GROUP II APPARATUS (test sequence) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Item: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ###### Thermal endurance to heat |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Maximum service temperature  75 C: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | continuous storage for |  |  |  |  |  |  |  |  |  | weeks Required: four weeks |  |  |  |  |  |  |  |  |  |  |  |
|  | relative humidity, |  |  |  |  |  |  |  |  |  | % Required: 90  5% |  |  |  |  |  |  |  |  |  |  |  |
|  | temperature, |  |  |  |  |  |  | C Required: 80 C  (20  2) C above max. service temperature |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Maximum service temperature  75 C: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | continuous storage for |  |  |  |  |  |  |  |  |  | weeks at |  |  |  |  | C and |  |  | % relative humidity |  |  |  |
| Required: two weeks at (95  2) C and (90  5)% relative humidity |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| and |  |  |  |  |  |  |  |  |  |  | weeks at |  |  |  |  | C |  |  |  |  |  |  |
| Required: two weeks at (20  2) C above max. service temperature and prevailing humidity |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Observations : |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ###### Thermal endurance to cold |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | continuous storage for |  |  |  |  |  |  |  |  |  |  | h at |  |  | C |  |  |  |  |  |  |  |
| Required: 24 h at lower test ambient |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Observations: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Mechanical tests |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Applicable test(s): |  |  |  |  |  |  |  |  |  | conducted on component part, assembled to enclosure |  |  |  |  |  |  |  |  |  |  |  |  |
|  | refer to forms for IEC 60079-0, Test for resistance to impact |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | refer to forms for IEC 60079-0, Drop test (portable apparatus) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | refer to: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Results: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| *Note: Special conditions shall be stated if any of the conditioned enclosure samples do not withstand the applicable Mechanical tests. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Sheet 36 of 52 	(IEC 60079-1, 1990, Amendments 1 & 2)

| ###### IEC 60079-1 |  |  |  |  |  |  | Amendment 1, 1993-08 ANNEX A  NON-METALLIC PARTS OF FLAMEPROOF ENCLOSURES 9/12 |  |  |  |  |  |  |  |  |  |  |  |  | APPLIED |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ###### CLAUSE |  |  |  |  |  |  | REQUIREMENTS |  |  |  |  |  |  |  |  |  |  |  |  | YES |  |  |  |  | NO |  |  |  |  |  | ENCL. |  |  |  |  |  |  |  |  |
| A.3.2 |  |  |  |  |  |  | ###### Tests for flameproofness |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ###### A.3.3 |  |  |  |  |  |  | ###### Flammability |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | GROUP II APPARATUS (test sequence, continued) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Item: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Tests for flameproofness |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | The following tests were conducted in the order given on enclosures subjected to tests per 8/12 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test of ability of the enclosure to withstand pressure |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | refer to SECTION THREE  VERIFICATIONS AND TESTS, 1/5 through 3/5 (Pages |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ) |  |
|  |  | ###### refer to: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test of erosion by flame (to be considered where enclosure flameproof joints have at least one face of plastics material) Any machining required to increase gaps for purposes of the Test for non-transmission of an internal ignition shall have been done. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | ###### not applicable (i.e. flameproof joints do not involve plastic material; details: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ) |  |  |
|  |  | not applicable (i.e. volume  100 cm3 and material withstands Flammability test below) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | static gap (flanged joint/plane part of spigot joint) set to |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | mm Required: 0.1mm  value  0.15mm |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| or |  |  |  |  |  |  |  |  | gap set to maximum permitted value ( 0.15 mm); |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | mm |  |  |  |  |  |  |
|  |  | explosive test mixture; 9.8  0.5  methane in air (test ambient, |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | C) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | ignitions by (ignition source) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (ignition source location, |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ) |  |  |
| Required: 50 ignitions of the explosive mixture per Table IV for determination of explosion pressure, except 25 ignitions with each explosive mixture for Group IIC enclosures for use in both hydrogen and acetylene |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test for non-transmission of an internal ignition |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | refer to SECTION THREE  VERIFICATIONS AND TESTS, 4/5 (Page |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ) |  |  |  |  |  |  |  |  |  |
| Flammability |  |  |  |  |  |  |  |  | conducted on test specimens conditioned in accordance with 8/12 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Component |  |  |  |  |  |  |  |  |  | Test specimens |  |  |  |  |  | Test method |  |  |  |  |  |  | Worst case result after flame applications to each specimen (NO/YES) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | number of pieces | number of sets | cut from enclosure | moulded |  | cut from plates | ISO 1210 |  | first alternative |  | second alternative 125133 mm specimens |  |  | ISO 1210 |  |  |  | Alternative methods |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | burning  15 s |  |  | specimen burnt completely | flaming  10 s |  |  |  | flaming  50s total for 10 applications |  | specimen burnt up to holding clamp | cotton layer ignited | glowing  30 s |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  | (X) | (X) |  | (X) | (X) |  | (X) |  | (X) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Notes (i) Test pieces moulded as individual pieces or plates from which the test pieces are cut shall be produced under conditions equivalent to those used to produce the actual enclosure and the conditions shall be recorded in the manufacturer's documentation. (ii) One of the alternative test methods shall be applied if the test per ISO 1210 results in distortion of the test piece out of the flame. (iii) The first alternative test method shall be in accordance with IEC 60707 (Method FV: Flame – Vertical specimen). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Sheet 37 of 52 	(IEC 60079-1, 1990, Amendments 1 & 2)

| ###### IEC 60079-1 |  |  | Amendment 1, 1993-08 ANNEX A  NON-METALLIC PARTS OF FLAMEPROOF ENCLOSURES 10/12 | APPLIED |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ###### CLAUSE |  |  | REQUIREMENTS | YES | NO | ENCL. |  |
| A.3.1.7 |  |  | ###### Resistance to chemical agents |  |  |  |  |
|  |  |  | GROUP II APPARATUS (test sequence, continued) |  |  |  |  |
| Item: |  |  |  |  |  |  |  |
| Resistance to chemical agents Note: No requirements are currently prescribed. |  |  |  |  |  |  |  |
|  | The following tests (based on rationale as noted) were conducted in the order given: |  |  |  |  |  |  |

Sheet 38 of 52 	(IEC 60079-1, 1990, Amendments 1 & 2)

| ###### IEC 60079-1 |  |  |  | Amendment 1, 1993-08 ANNEX A  NON-METALLIC PARTS OF FLAMEPROOF ENCLOSURES 11/12 |  |  |  | APPLIED |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ###### CLAUSE |  |  |  | REQUIREMENTS |  |  |  | YES |  | NO |  | ENCL. |  |  |  |  |
| A.3.1.5 |  |  |  | Resistance to light |  |  |  |  |  |  |  |  |  |  |  |  |
| Item: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ###### Resistance to light |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | not applicable (i.e. enclosure is protected from light; method: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ) |  |
|  |  | not applicable (i.e. plastic enclosure part(s) protected from light; details: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ) |  |
|  |  | not applicable (i.e. Group I apparatus other than a luminaire) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ###### Impact bending strength (ISO 179): |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Component |  |  |  | Specimen No | Before exposure |  |  | Period of exposure (h) |  | After exposure |  |  |  |  |  |
|  |  |  |  |  |  | impact energy absorbed (kJ/m2) | break |  |  |  | impact energy absorbed (exposed side) (kJ/m2) |  | break |  |  |  |
|  |  |  |  |  |  |  | YES | NO |  |  |  |  | YES | NO |  |  |
|  |  |  |  |  | 1 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 2 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 3 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 4 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 5 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 6 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 1 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 2 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 3 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 4 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 5 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 6 |  |  |  |  |  |  |  |  |  |  |  |
| Notes (i) The test shall be made on six test bars of standard size 50 mm  6 mm  4 mm according to ISO 179. (ii) Test bars shall be produced under conditions equivalent to those used to produce the actual enclosure and the conditions shall be stated in the test report. (iii) The test shall be made in accordance with ISO 4892 (i.e. exposure chamber using xenon lamp and sunlight simulating filter system, at a black panel temperature of (55  3) C). (iv) Exposure time shall be 1000 h. (v) Impact bending strength (on exposed side) shall be  50% of corresponding value measured on unexposed test piece. (vi) Where the impact bending strength cannot be determined prior to exposure (i.e. no rupture), not more than three of the exposed test bars may break. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | test bars were made under conditions equivalent to those used to produce the actual enclosure; details: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Sheet 39 of 52 	(IEC 60079-1, 1990, Amendments 1 & 2)

| ###### IEC 60079-1 |  |  | Amendment 1, 1993-08 ANNEX A  NON-METALLIC PARTS OF FLAMEPROOF ENCLOSURES 12/12 | APPLIED |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ###### CLAUSE |  |  | REQUIREMENTS | YES | NO | ENCL. |  |
|  |  |  | ###### Tests for cable entries and stopping boxes |  |  |  |  |
| Item: |  |  |  |  |  |  |  |
| Tests for cable entries and stopping boxes Note: No requirements are currently prescribed (ref. APPENDIX A, Cl. A5). |  |  |  |  |  |  |  |
|  | The following tests were conducted according to national regulations in the order given: |  |  |  |  |  |  |

Sheet 40 of 52 	(IEC 60079-1, 1990, Amendments 1 & 2)

| ###### IEC 60079-1 |  |  |  |  | Amendment 1, 1993-08 ANNEX B  BREATHING DEVICES AND DRAINING DEVICES 1/11 |  |  |  |  | APPLIED |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ###### CLAUSE |  |  |  |  | REQUIREMENTS |  |  |  |  | YES | NO | ENCL. |  |
| B.1 |  |  |  |  | ###### Scope |  |  |  |  |  |  |  |  |
| B.2 |  |  |  |  | General requirements |  |  |  |  |  |  |  |  |
| B.3 |  |  |  |  | Materials |  |  |  |  |  |  |  |  |
| B.4 |  |  |  |  | Dimensions |  |  |  |  |  |  |  |  |
| Item: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ###### Scope |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Purpose: |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | breathing |  |  |  |  |  | draining |  | sound transmission |  |  |  |  |
|  | other; |  |  |  |  |  |  |  |  |  |  |  |  |
| Feasibility for conditions of use: |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | device is not used to relieve pressure in the event of an internal explosion and |  |  |  |  |  |  |  |  |  |  |  |
|  |  | device is not used with pressure lines containing gas which is capable of forming an explosive mixture with air and is at pressure > 1.1  atmospheric |  |  |  |  |  |  |  |  |  |  |  |
| General requirements Note: Breathing and draining devices are not intended to withstand continuous burning on their surfaces. |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | device incorporating permeable element other than the identified types; details: |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Note: Openings for breathing or draining shall not be produced by deliberate enlargement of gaps of flanged joints. |  |  |  |  |  |  |  |  |  |  |  |
|  |  | manufacturer supplies instructions that care shall be taken to ensure that the device is not liable to become inoperative in service (e.g. accumulation of dust or paint); details/refer to: |  |  |  |  |  |  |  |  |  |  |  |
| Materials |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Composition limits; details/reference specification: |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | used with acetylene ( |  |  |  |  | element does not comprise more than 60  of copper by mass) |  |  |  |  |  |  |
| Dimensions |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Details/refer to: |  |  |  |  |  |  |  |  |  |  |  |  |  |

Sheet 41 of 52 	(IEC 60079-1, 1990, Amendments 1 & 2)

| ###### IEC 60079-1 |  |  |  | Amendment 1, 1993-08 ANNEX B  BREATHING DEVICES AND DRAINING DEVICES 2/11 |  |  |  |  |  |  | APPLIED |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ###### CLAUSE |  |  |  | REQUIREMENTS |  |  |  |  |  |  | YES |  | NO |  | ENCL. |  |  |  |  |  |  |  |
| B.5 |  |  |  | Elements with measurable paths |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| B.6 |  |  |  | Additional requirements for crimped ribbon elements of breathing and draining devices |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Item: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| # Elements with measurable paths |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| # Interstices and measurable lengths of path: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | # Between (parts): |  |  |  |  |  |  |  |  | Enclosure volume # (cm3) |  | # Width L # (mm) |  | # Gap i # (mm) |  | # Comply with Table (YES/NO) |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | # I | # IIA | # IIB | # IIC |  |  |  |
| # Note: Interstices and measurable lengths of path need not comply with the values given in Tables I, IIA, IIB, IIC provided that the elements pass the tests of 5/11 through 11/11. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| # Joint between device and flameproof enclosure: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | refer to SECTION TWO  CONSTRUCTION 1/12 through 4/12 as applicable |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| # Additional requirements for crimped ribbon elements of breathing and draining devices |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ###### Material: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | cupro-nickel |  |  |  | stainless steel |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Note: Aluminium, titanium, magnesium and their alloys shall not be used. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Measurable paths specified in drawings: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | NO (refer to 3/11, Elements with non-measurable paths) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | YES; details/reference specification defining upper and lower tolerance limits: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | tolerance limits are monitored in production; method: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type tests (refer to 7/11 and 8/11, Test for non-transmission of an internal ignition): |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | conducted on samples manufactured with the largest permitted gap dimensions; sample details: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Sheet 42 of 52 	(IEC 60079-1, 1990, Amendments 1 & 2)

| ###### IEC 60079-1 |  |  |  | Amendment 1, 1993-08 ANNEX B  BREATHING DEVICES AND DRAINING DEVICES 3/11 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | APPLIED |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ###### CLAUSE |  |  |  | REQUIREMENTS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | YES |  |  |  | NO |  |  |  | ENCL. |  |  |  |  |
| B.7 |  |  |  | Elements with non-measurable paths |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Item: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| # Elements with non-measurable paths |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Notes: 1. Elements are classified according to their density as well as their pore size. 2. For functional reasons, it may also be necessary to state the fluid permeability and the porosity. 3. The appropriate specifications shall be in accordance with a recognized standard method (i.e. as indicated below) for the particular material and manufacturing method. 4. Reference documentation shall clearly identify the metal element in accordance with specifications given below. 5. Test sample pore size shall be not less than 85 % of the specified maximum pore size and the Mechanical strength test and Tests of flameproof enclosure with breathing and draining devices shall be applied to the same sample. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ###### Additional requirements for elements with non-measurable paths, of breathing and draining devices: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ###### Sintered metal elements |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ###### Specifications identified in documentation (refer to |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ): |
|  | stainless steel |  |  |  |  |  |  |  |  | 90/10 copper-tin bronze |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | maximum pore size (ISO 4003), |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | m |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | density (ISO 2738), |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | g/cm3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | minimum thickness, |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | mm |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | functional aspects (ISO 4022 and ISO 2738):- |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | open porosity, |  |  |  |  |  |  |  |  |  |  | % |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | fluid permeability, |  |  |  |  |  |  |  |  |  |  |  | m2 |  |  |
| ###### Pressed metal wire elements |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ###### Specifications identified in documentation (refer to |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ): |
|  | stainless steel wire |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | maximum pore size (ISO 4003), |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | m |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | density (ISO 2738), |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | g/cm3 (based on the information given below) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (mass, |  |  |  |  |  | g; length of wire braid, |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ; mesh size, |  |  |  |  |  |  |  |  |  |  | ) |
| ratio between mass of filter and identical volume of same solid metal, |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Required: 0.4  ratio  0.6 |  |  |  |  |  |  |
|  | dimensions (incl. tolerances), |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | original wire diameter, |  |  |  |  |  |  |  |  |  |  |  |  |  | mm |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | functional aspects (ISO 4022 and ISO 2738):- |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | open porosity, |  |  |  |  |  |  |  |  |  |  | % |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | fluid permeability, |  |  |  |  |  |  |  |  |  |  |  |  | m2 |  |
| Note: The element shall be rigid (i.e. wire braid, compressed into a dye to form a homogeneous matrix). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Metal foam elements |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Specifications identified in documentation (refer to |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ): |
|  | nickel-chrome alloy ( |  |  |  |  |  |  |  |  |  |  |  | % chromium by mass) Required: at least 15% chromium |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | maximum pore size (ISO 4003), |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | m |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | minimum thickness, |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | mm |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | minimum density, |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | g/cm3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | functional aspects (ISO 4022 and ISO 2738):- |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | open porosity, |  |  |  |  |  |  |  |  |  |  | % |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | fluid permeability, |  |  |  |  |  |  |  |  |  |  |  | m2 |  |  |
| Note The element shall be produced by coating a reticulated polyurethane foam with nickel, removing the polyurethane by thermal decomposition and converting the nickel into a nickel-chrome alloy (e.g. by gaseous diffusion) and compressing the material as necessary. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Sheet 43 of 52 	(IEC 60079-1, 1990, Amendments 1 & 2)

| ###### IEC 60079-1 |  |  | Amendment 1, 1993-08 ANNEX B  BREATHING DEVICES AND DRAINING DEVICES 4/11 |  |  |  |  |  |  | APPLIED |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ###### CLAUSE |  |  | REQUIREMENTS |  |  |  |  |  |  | YES |  | NO | ENCL. |  |  |
| B.8 |  |  | Device that can be dismantled |  |  |  |  |  |  |  |  |  |  |  |  |
| B.9 |  |  | Mounting arrangements of the elements |  |  |  |  |  |  |  |  |  |  |  |  |
| B.10 |  |  | Mechanical strength |  |  |  |  |  |  |  |  |  |  |  |  |
| Item: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| # Device that can be dismantled |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | ###### device is designed to avoid reduction/enlargement of openings during re-assembly; details: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ###### Mounting arrangements of the elements |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ###### Integral assembly: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | fixed (as below) in a suitable mounting component, replaceable as a unit; method of attachment to enclosure: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | joint between device and flameproof enclosure: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | refer to SECTION TWO  CONSTRUCTION (1/12 through 4/12 as applicable) |  |  |  |  |  |  |  |  |  |  |
|  | ###### fixed (as below) directly into the enclosure to form an integral part of the enclosure |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | sintered |  |  |  | welded |  |  | brazed |  |  |  |  |  |  |  |
|  | cemented, in which case |  |  |  | strength of enclosure or mounting component is not dependent on cement and |  |  |  |  |  |  |  |  |  |  |
| shortest path through cemented joint inside to outside enclosure ( |  |  |  |  |  |  |  |  |  |  | mm) for:- |  |  |  |  |
|  |  |  |  |  | enclosure volume  10 cm3 Required: path  3mm |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 10 cm3 < enclosure volume  100 cm3 Required: path  6mm |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | enclosure volume > 100 cm3 Required: path  10mm |  |  |  |  |  |  |  |  |  |  |
|  | other method of fixing: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Mounted so as to form a flameproof joint with the enclosure: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | clamping ring |  |  |  | (other means) |  |  |  |  |  |  |  |  |  |  |
|  | from within enclosure ( |  |  |  | screws/clamping rings are accessible only from inside the enclosure) |  |  |  |  |  |  |  |  |  |  |
|  | from outside enclosure ( |  |  |  | refer to SECTION TWO  CONSTRUCTION, 7/12, Fasteners) |  |  |  |  |  |  |  |  |  |  |
|  | joint between element and flameproof enclosure: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | refer to SECTION TWO  CONSTRUCTION (1/12 through 4/12 as applicable) |  |  |  |  |  |  |  |  |  |  |
|  | Note: Exception that the requirement for surface roughness of the element can be waived if the element passes the type test. (ref. 7/11 and 8/11, Tests for non-transmission of an internal ignition) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Mechanical strength Note: Applicable to the device and its guard, if any, mounted in the intended manner. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | refer to forms for IEC 60079-0, Test for resistance to impact |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Results: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Sheet 44 of 52 	(IEC 60079-1, 1990, Amendments 1 & 2)

| ###### IEC 60079-1 |  |  | Amendment 1, 1993-08 ANNEX B  BREATHING DEVICES AND DRAINING DEVICES 5/11 |  |  |  |  |  |  |  |  |  |  |  | APPLIED |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ###### CLAUSE |  |  | REQUIREMENTS |  |  |  |  |  |  |  |  |  |  |  | YES |  | NO |  |  | ENCL. |  |  |  |  |  |
| B.11 |  |  | Tests of flameproof enclosure with breathing and draining devices |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| B.11.1 |  |  | Test of ability of the enclosure to withstand pressure |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| B.11.2 |  |  | Thermal tests |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Item: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| # Tests of flameproof enclosure with breathing and draining devices |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | the tests following were conducted in the order given on a sample(s) subjected to the preceding impact test |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ###### Devices with non-measurable paths: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | ###### pore size of test sample, |  |  |  |  |  |  | ###### m ( |  |  |  | ######  85 % of the specified maximum pore size) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ###### Test of ability of the enclosure to withstand pressure |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Refer to: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | SECTION THREE  VERIFICATIONS AND TESTS, 1/5 and 2/5, Determination of explosion pressure |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | breathing/draining device(s) replaced by solid plug(s); method: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | SECTION THREE  VERIFICATIONS AND TESTS, 3/5, Overpressure test |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | thin flexible membrane fitted to inner surfaces of device(s); method: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Observed permanent damage or deformation of breathing/draining device(s): |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | NO |  |  |  |  | YES (details) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | type of protection affected: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  | NO |  |  |  |  | YES |  |  |  |  |  |  |  |
| Thermal tests |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Ignition source: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | high voltage sparking plug ( |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ) |  |
|  | low energy ignition source ( |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ) |  |
|  | contained ignition-capable switching device ( |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ) |  |
| Test set-up: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | sample device fitted to enclosure and placed in test chamber (volume |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ) |  |  |  |  |  |  |
|  | same test mixture throughout (volume ratio, test chamber/enclosure to which the device is fitted: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | / |  |  |  |
| continued on 6/11 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Sheet 45 of 52 	(IEC 60079-1, 1990, Amendments 1 & 2)

| ###### IEC 60079-1 |  |  |  |  |  |  | Amendment 1, 1993-08 ANNEX B  BREATHING DEVICES AND DRAINING DEVICES 6/11 |  |  |  |  |  |  |  |  |  |  |  |  |  | APPLIED |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ###### CLAUSE |  |  |  |  |  |  | REQUIREMENTS |  |  |  |  |  |  |  |  |  |  |  |  |  | YES |  | NO |  |  | ENCL. |  |  |  |  |  |
| B.11.2 |  |  |  |  |  |  | Thermal tests (continued) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Item: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Thermal tests (continued) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Devices for Groups I, IIA, IIB, IIC: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Test No |  | Ex component for multiple use in any single flameproof enclosure |  | Ignition source location i.e. other than for an Ex component sample mounted on test rig position producing the highest peak explosion pressure and rate of rise of pressure at the face of the device and considering the device location which gives the most unfavourable results where two or more identical devices are fitted |  |  |  |  |  |  |  |  | Ex component sample (mounted on test rig) |  |  |  | Test mixture |  |  |  | test gas flow through the device and the enclosure | ventilation or sampling system operated per mfr.'s documentation |  | Test mixture maintained (external to device) after each test for at least (min) sufficient time to reveal any continuous burning on device face; e.g.  10 min |  |  | External surface temperature of device (C) |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Spark plug location |  |  | (4.2  0.1) % C3H8, in air applied for all Groups | (7.5  1) % C2H2, in air device for use in acetylene |  | at atmospheric pressure and temperature |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 50 mm from inside of test rig end plate |  | at test rig inlet |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | (X) |  |  |  |  |  |  |  |  |  |  | (X) | (X) |  | (X) | (X) | (X) |  | (X) | (X) | (X) |  |  |  |  |  |  |  |  |
|  | Notes 1. For devices for hydrogen only, tests with mixtures including acetylene are not required. 2. The surface temperature of the device should be at test ambient at the start of each test. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Continuous burning: |  |  |  |  |  |  |  |  |  | NO |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | YES; Test No |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Flame transmission: |  |  |  |  |  |  |  |  |  | NO |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | YES; Test No |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Maximum external surface temperature of the device: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | C at ambient, |  |  |  |  |  | C |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (surface temperatures measured by |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ) |  |
| (maximum external surface temperature rise of the device, |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | K)  1.2 = |  |  |  |  |  |  | K |  |  |  |  |
| Note: The measured external surface temperature rise of the device shall be multiplied by a safety factor of 1.2 for the determination of the temperature class of the electrical apparatus. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Sheet 46 of 52 	(IEC 60079-1, 1990, Amendments 1 & 2)

| ###### IEC 60079-1 |  |  |  |  | Amendment 1, 1993-08 ANNEX B  BREATHING DEVICES AND DRAINING DEVICES 7/11 |  |  |  |  |  |  |  |  | APPLIED |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ###### CLAUSE |  |  |  |  | REQUIREMENTS |  |  |  |  |  |  |  |  | YES |  | NO |  | ENCL. |  |  |  |  |  |  |  |
| B.11.3 |  |  |  |  | Test for non-transmission of an internal ignition |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Item: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test for non-transmission of an internal ignition |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | sample device fitted to enclosure and placed in test chamber (volume, |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ) |  |  |  |  |
| Note: The volume ratio of the test chamber to the enclosure should be at least 5 to 1 for precompressed test mixtures (8/11). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Devices for Groups I, IIA, IIB: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Test No |  | Ignition source (as for Thermal tests, 5/11) close to the inner surface of the device and then (if necessary) in one or more places producing the highest peak explosion pressure and rate of rise of pressure at the device face |  |  | Ex component sample (mounted on test rig) |  |  | Normal condition (Note 2) |  |  | Alternative for Groups IIA, IIB devices with measurable paths i.e. where damage by hydrogen/air test mixtures is possible |  |  |  |  |  |  |  |  |  |  | Flame transmission (NO/YES) |  |  |
|  |  |  |  |  |  |  | Spark plug location |  | Explosive test mixture (same mixture throughout) |  |  |  |  |  | Increased joint gap |  |  |  | Reduction of length of thread engagement (Note 3) |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  | Group I; 12.5  0.5 % methane-hydrogen (58/42  1 %) in air | Group IIA; 55  1.0 % hydrogen in air | Group IIB; 37  1.0 % hydrogen in air | Group IIA; 4.2  0.1 % propane in air | Group IIB; 6.5  0.5 % ethylene in air | Group IIB; 19  1 % hydrogen-methane (85/15) in air |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | 50 mm from inside of test rig end plate | at test rig inlet |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Location i.e. other than for an Ex component sample mounted on test rig |  |  |  |  |  |  |  |  |  |  |  | 1.42 iC (mm) Group IIA |  | 1.85 iC (mm) Group IIB |  | none (taper-threaded) |  |  |  |  |  |  |
|  |  |  |  |  |  | (X) | (X) | (X) | (X) | (X) | (X) | (X) | (X) | (X) |  |  |  |  | (X) | (X) |  | (X) |  |  |  |
|  |  |  | close to the inner surface of the device |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Notes 1. iC = the maximum constructional gap, as specified in the manufacturer's drawings iT = the maximum gap permitted in Table I, IIA or IIB as applicable 2. Where no artificial gap is created, the test gap shall be 80 to 100 % of the specified maximum constructional gap, but not exceeding iT. 3. Required reduction of length of thread engagement: - by  of manufacturer's specification for joints conforming to ISO fit - by  of manufacturer's specification for joints worse than ISO fit - no reduction of taper-threaded joints continued on 8/11 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Sheet 47 of 52 	(IEC 60079-1, 1990, Amendments 1 & 2)

| ###### IEC 60079-1 |  |  |  | Amendment 1, 1993-08 ANNEX B  BREATHING DEVICES AND DRAINING DEVICES 8/11 |  |  |  |  |  |  |  | APPLIED |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ###### CLAUSE |  |  |  | REQUIREMENTS |  |  |  |  |  |  |  | YES |  | NO |  |  | ENCL. |  |  |  |  |  |  |
| B.11.3 |  |  |  | ###### Test for non-transmission of an internal ignition (continued) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Item: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test for non-transmission of an internal ignition (continued) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Devices for Group IIC: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Test No | Ignition source (as for Thermal tests, 5/11) close to the inner surface of the device and then (if necessary) in one or more places producing the highest peak explosion pressure and rate of rise of pressure at the device face |  |  | Ex component sample (mounted on test rig) |  |  | Measurable paths |  |  |  |  |  |  |  | Measurable or non- measurable paths |  |  |  |  | Flame transmission (NO/YES) |  |  |
|  |  |  |  |  |  | Spark plug location |  | Reduction of length of thread engagement |  |  |  |  |  |  |  | Method A |  |  | Method B (Note 2) |  |  |  |  |
|  |  |  |  |  |  |  |  | Increased joint gap |  |  |  |  |  |  | (""; ISO fit) or (""; worse than ISO fit) or ("none"; taper) |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | 27  1.0 % H2 in air (test mixture at atmospheric pressure) | 7.5  1 % C2H2 in air (test mixture at atmospheric pressure) | cylindrical (incl. spigot, width L) | flanged ( 0.1mm) and cylindrical (incl. spigot, width L) |  | flanged only |  |  | 27  1.0 % H2 in air (1.5 bar precompression of test mixture) |  | 7.5  1 % C2H2 in air (1.5 bar precompression of test mixture) | (40  1 %) H2, (20  1 %) O2, bal. N2 (test mixture at atmospheric pressure) | (10  1 %) C2H2, (24  1 %) O2, bal. N2 (test mixture at atmospheric pressure) |  |  |  |
|  |  |  |  |  |  | 50 mm from inside of test rig end plate | at test rig inlet |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Location i.e. other than for an Ex component sample mounted on test rig |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | iC +  iT (mm) | iC +  iC (mm) |  | 1.5 iT (mm) |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | (X) | (X) | (X) | (X) | (X) |  |  |  |  |  |  | (X) |  | (X) | (X) | (X) |  |  |  |
|  |  | close to the inner surface of the device |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | close to the inner surface of the device |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Notes 1. For devices for hydrogen only, tests with mixtures including acetylene are not required. 2. In the case of Method B the apparatus shall be marked with sign X indicating exclusion of ethyl nitrate and where the device is for use with enclosure volume > 100 cm3, indicating exclusion of carbon disulfide. 3. The test chamber and the enclosure shall be filled with the explosive test mixture. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Sheet 48 of 52 	(IEC 60079-1, 1990, Amendments 1 & 2)

| ###### IEC 60079-1 |  |  | Amendment 2, 1998-05 ANNEX B  BREATHING DEVICES AND DRAINING DEVICES 9/11 |  |  |  |  |  | # APPLIED |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ###### CLAUSE |  |  | REQUIREMENTS |  |  |  |  |  | YES |  | NO | ENCL. |  |
| B.12 |  |  | Breathing devices and draining devices when used as Ex Components |  |  |  |  |  |  |  |  |  |  |
| B.12.1 |  |  | Mounting arrangements of the elements and component |  |  |  |  |  |  |  |  |  |  |
| Item: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Breathing devices and draining devices when used as Ex Components |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Applied requirements of B.1 to B.8; refer to applicable as follows: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Scope, General requirements, Materials and Dimensions; 1/11 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Elements with measurable paths; 2/11 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Additional requirements for crimped ribbon elements of breathing and draining devices; 2/11 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Elements with non-measurable paths; 3/11 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Device that can be dismantled; 4/11 |  |  |  |  |  |  |  |  |  |  |  |  |
| Mounting arrangements of the elements and component |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ###### Fixed into a suitable mounting part to form the mounting component: |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | sintered |  |  |  | welded |  | brazed |  |  |  |  |  |  |
|  | cemented, in which case |  |  |  | strength of enclosure or mounting component is not dependent on cement and |  |  |  |  |  |  |  |  |
| shortest path through cemented joint inside to outside enclosure ( |  |  |  |  |  |  |  |  |  | mm) for:- |  |  |  |
|  |  |  |  |  | enclosure volume  10 cm3 Required: path  3mm |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 10 cm3 < enclosure volume  100 cm3 Required: path  6mm |  |  |  |  |  |  |  |  |
|  |  |  |  |  | enclosure volume > 100 cm3 Required: path  10mm |  |  |  |  |  |  |  |  |
|  | other method of fixing: |  |  |  |  |  |  |  |  |  |  |  |  |
| Mounting part: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Means of securing mounting component as a replaceable unit: |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | refer to SECTION TWO  CONSTRUCTION, 7/12, Fasteners |  |  |  |  |  |  |  |  |  |  |  |  |
| Joint between mounting component and flameproof enclosure: |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | refer to SECTION TWO  CONSTRUCTION, 1/12 through 4/12 as applicable |  |  |  |  |  |  |  |  |  |  |  |  |

Sheet 49 of 52 	(IEC 60079-1, 1990, Amendments 1 & 2)

| ###### IEC 60079- |  |  | Amendment 2, 1998-05 ANNEX B  BREATHING DEVICES AND DRAINING DEVICES 10/11 |  |  |  |  | # APPLIED |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ###### CLAUSE |  |  | REQUIREMENTS |  |  |  |  | YES | NO |  | ENCL. |  |
| B.12.2 |  |  | Type tests for breathing and draining devices used as Ex components |  |  |  |  |  |  |  |  |  |
| B.12.2.1 |  |  | Test of the ability of the breathing and draining device to withstand pressure |  |  |  |  |  |  |  |  |  |
| B.12.2.2 |  |  | Thermal tests |  |  |  |  |  |  |  |  |  |
| B.12.2.3 |  |  | Test for non-transmission of an internal ignition |  |  |  |  |  |  |  |  |  |
| Item: |  |  |  |  |  |  |  |  |  |  |  |  |
| Type tests for breathing and draining devices used as Ex components |  |  |  |  |  |  |  |  |  |  |  |  |
| Sample device, attached on end (base section) of test rig per Figure B.1 for the sequence of tests following: |  |  |  |  |  |  |  |  |  |  |  |  |
|  | subjected to impact test; refer to forms for IEC 60079-0, Test for resistance to impact |  |  |  |  |  |  |  |  |  |  |  |
| Notes: 1. The sample device shall be mounted in the same manner as it would normally be mounted on a flameproof enclosure. 2. The impact test may be performed separate from the test rig if the sample is mounted on a plate that forms the end part of the test rig enclosure. 3. For devices with non-measurable paths, test sample pore size shall be not less than 85 % of the specified maximum pore size. 4. Devices excluded from certification as Ex components due to test failure may be used as an integral part of a flameproof enclosure provided they meet the applicable test requirements of 4/11 through 8/11 when tested with the specific enclosure. |  |  |  |  |  |  |  |  |  |  |  |  |
| Test of the ability of the breathing and draining device to withstand pressure |  |  |  |  |  |  |  |  |  |  |  |  |
| Reference test pressure: |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1,200 kPa (Gp I) |  |  |  | 1,350 kPa (Gp IIA) |  | 2,500 kPa (Gp IIB) |  |  | 4,000 kPa (Gp IIC) |  |  |
| Overpressure test: |  |  |  |  |  |  |  |  |  |  |  |  |
|  | thin flexible membrane fitted to inner surfaces (method) |  |  |  |  |  |  |  |  |  |  |  |
|  | 1.5 × reference pressure applied for 1 min. Required: Each component to be submitted to a routine test |  |  |  |  |  |  |  |  |  |  |  |
|  | 4 × reference pressure applied for 1 min. (alternatively) |  |  |  |  |  |  |  |  |  |  |  |
| Observed permanent damage or deformation of the device: |  |  |  |  |  |  |  |  |  |  |  |  |
|  | NO |  |  |  | YES (details) |  |  |  |  |  |  |  |
| Thermal tests (refer to 5/11 and 6/11, Thermal tests) |  |  |  |  |  |  |  |  |  |  |  |  |
| Evidence of thermal or mechanical damage or deformation affecting flame arresting properties: |  |  |  |  |  |  |  |  |  |  |  |  |
|  | NO |  |  |  | YES (details) |  |  |  |  |  |  |  |
| Test for non-transmission of an internal ignition |  |  |  |  |  |  |  |  |  |  |  |  |
| Test rig set-up: |  |  |  |  |  |  |  |  |  |  |  |  |
|  | one section (Group I and Group IIA) |  |  |  |  |  | two sections (Group IIB and Group IIC) |  |  |  |  |  |
| Refer to: |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 7/11, Group IIC devices with measurable paths and devices for Groups I, IIA, IIB |  |  |  |  |  |  |  |  |  |  |  |
|  | 8/11, Group IIC; devices with measurable paths (required additional tests) and devices with non-measurable paths |  |  |  |  |  |  |  |  |  |  |  |

Sheet 50 of 52 	(IEC 60079-1, 1990, Amendments 1 & 2)

| ###### IEC 60079- |  |  |  | Amendment 2, 1998-05 ANNEX B  BREATHING DEVICES AND DRAINING DEVICES 11/11 |  |  | # APPLIED |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ###### CLAUSE |  |  |  | REQUIREMENTS |  |  | YES | NO | ENCL. |  |  |
| B.12.3 |  |  |  | Marking |  |  |  |  |  |  |  |
| B.12.4 |  |  |  | Component certificate |  |  |  |  |  |  |  |
| Item: |  |  |  |  |  |  |  |  |  |  |  |
| Marking (refer to forms for IEC 60079-0, Marking) |  |  |  |  |  |  |  |  |  |  |  |
|  |  | device is certified in the name of the original component manufacturer |  |  |  |  |  |  |  |  |  |
|  |  | Note: The original component manufacturer is responsible to ensure that all future component devices are made in accordance with the certified type. |  |  |  |  |  |  |  |  |  |
| Component certificate (this section confirmed by the certifying authority) |  |  |  |  |  |  |  |  |  |  |  |
| Certificate shows: |  |  |  |  |  |  |  |  |  |  |  |
|  |  | manufacturer's name |  |  |  |  |  |  |  |  |  |
|  |  | identifying drawings |  |  |  |  |  |  |  |  |  |
|  |  | specifications |  |  |  |  |  |  |  |  |  |
|  |  | limiting reference pressure |  |  |  |  |  |  |  |  |  |
|  |  | overpressure test details and whether routine testing is required |  |  |  |  |  |  |  |  |  |
|  |  | maximum recorded surface temperature obtained during the type test (i.e. thermal test) |  |  |  |  |  |  |  |  |  |
|  |  | the Group |  |  |  |  |  |  |  |  |  |
| Certificate requires: |  |  |  |  |  |  |  |  |  |  |  |
|  |  | that each component or package of components is accompanied by a copy of the certificate and |  |  |  |  |  |  |  |  |  |
|  |  | the manufacturer's declaration stating:- |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | compliance with the certified conditions |  |  |  |  |  |
|  |  |  |  |  |  | confirmation of material, bubble pore size, density (where applicable) |  |  |  |  |  |
|  |  |  |  |  |  | special mounting instructions (if any) |  |  |  |  |  |
| Notes/details re certificate contents: (optional) |  |  |  |  |  |  |  |  |  |  |  |

Sheet 51 of 52 	(IEC 60079-1, 1990, Amendments 1 & 2)

| ## IEC 60079-1 |  | COMMENTS |  |  |
| --- | --- | --- | --- | --- |
| #### Clause |  |  | Page |  |

Sheet 52 of 52 	(IEC 60079-1, 1990, Amendments 1 & 2)