| Reference No: |  |  |  |  |  |  |  |  |  |  | #### IEC 60079-1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ## Field of application This form is intended to be used when assessing and testing electrical apparatus for explosive atmospheres according to the standard specified in the adjoining column. The test results are valid for the tested items only. Reproduction This report must not be reproduced other than in its entirety except with the prior written approval of |  | Standard: # IEC 60079-1: 2001 Fourth Edition Electrical apparatus for explosive gas atmospheres – Part 1: Flameproof enclosures "d" |  |  |  |  | ## Instructions for use to issuing ExCBs The comment columns can be used to e.g. specify test values, applied alternative requirements/tests or to refer to enclosed additional pages with comments. Abbreviations: N/A = Not applicable (for the assessment/test of the equipment and its design documentation) The applicant is the body/person who has requested the assessment/test and to which the results, as specified in this report, are provided. The short descriptions in the column for "Clause" do not replace the complete text in the standard. |  |  |  |  |
|  |  |  |  |  |  |  | # This test report may only be used for the # IECEx Scheme and must be accompanied by an ExTR cover page issued by an ExCB. |  |  |  |  |
| ## Product name |  |  |  |  | # Model/Type designation |  |  |  |  |  |  |
| Serial No | Code (e.g. Ex II T ) |  |  |  |  | Ambient temperature [min-max] |  |  |  |  |  |
| Applicant [name and postal address] |  |  |  |  | Manufactured by [name and postal address] |  |  |  |  |  |  |
| Test sample received [date] |  |  |  |  |  |  |  |  |  |  |  |
| ## Product description |  |  |  |  |  |  |  |  |  |  |  |
| The equipment complies with the standard [yes/no] | ## Prepared by |  |  |  |  |  |  |  | Date [year-month-day] |  |  |
|  | [signature] |  |  |  |  |  |  |  |  |  |  |
| Sheet 1 of 67 IEC 60079-1: 2001 |  |  |  |  |  |  |  |  |  |  |  |

| Reference No: |  |  |  |  |  |  |  | IEC 60079-1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Clause |  |  | Results |  |  |  |  |  |
|  |  |  | Pass | Fail | N/A | Comments |  | ExCB Review |
| 1 | Scope |  |  |  |  |  |  |  |
| 2 See also DS 2010/006A | Normative references |  |  |  |  |  |  |  |
| 3 | Definitions |  |  |  |  |  |  |  |
| 4 | Apparatus grouping and temperature classification |  |  |  |  |  |  |  |
| 5 | Flameproof joints |  |  |  |  |  |  |  |
| 5.1 | General requirements |  |  |  |  |  |  |  |
| 5.2 | Non-threaded joints |  |  |  |  |  |  |  |
| 5.2.1 | Width of joints (L) |  |  |  |  |  |  |  |
| 5.2.2 | Gap (i) |  |  |  |  |  |  |  |
| 5.2.3 | Spigot joints |  |  |  |  |  |  |  |
| 5.2.4 | Holes in joint surfaces |  |  |  |  |  |  |  |
| 5.2.4.1 | Flanged joints with holes outside the enclosure (see figures 4 and 6) |  |  |  |  |  |  |  |
| 5.2.4.2 | Flanged joints with holes inside the enclosure (see figure 5) |  |  |  |  |  |  |  |
| 5.2.4.3 | Spigot joints where, to the edges of the holes, the joint consists of a cylindrical part and a plane part (see figure 7) |  |  |  |  |  |  |  |
| 5.2.4.4 | Spigot joints where, to the edges of the holes, the joint consists only of the plane part (see figures 8 and 9), in so far as plane joints are permitted (see 5.2.7) |  |  |  |  |  |  |  |
| 5.2.5 | Conical joints |  |  |  |  |  |  |  |
| 5.2.6 | Joints with partial cylindrical surfaces (not permitted for Group IIC) |  |  |  |  |  |  |  |
| 5.2.7 | Additional requirements for joints of electrical apparatus of Group IIC |  |  |  |  |  |  |  |
| 5.2.8 | Serrated joints |  |  |  |  |  |  |  |
| Sheet 2 of 67 IEC 60079-1: 2001 |  |  |  |  |  |  |  |  |

| Reference No: |  |  |  |  |  |  |  | IEC 60079-1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Clause |  |  | Results |  |  |  |  |  |
|  |  |  | Pass | Fail | N/A | Comments |  | ExCB Review |
| 5.3 | Threaded joints |  |  |  |  |  |  |  |
| 5.4 | Gaskets (including O-rings) |  |  |  |  |  |  |  |
| 5.5 | Apparatus using capillaries |  |  |  |  |  |  |  |
| 6 | Cemented joints |  |  |  |  |  |  |  |
| 6.1 | General |  |  |  |  |  |  |  |
| 6.2 | Mechanical strength |  |  |  |  |  |  |  |
| 6.3 | Width of cemented joints |  |  |  |  |  |  |  |
| 7 | Operating rods |  |  |  |  |  |  |  |
| 7.1 | Diameter of operating rod |  |  |  |  |  |  |  |
| 7.2 | Diametrical clearance |  |  |  |  |  |  |  |
| 8 | Supplementary requirements for shafts and bearings |  |  |  |  |  |  |  |
| 8.1 | Joints of shafts |  |  |  |  |  |  |  |
| 8.1.1 | Cylindrical joints |  |  |  |  |  |  |  |
| 8.1.2 | Labyrinth joints |  |  |  |  |  |  |  |
| 8.1.3 | Joints with floating glands |  |  |  |  |  |  |  |
| 8.2 | Bearings |  |  |  |  |  |  |  |
| 8.2.1 | Sleeve Bearings |  |  |  |  |  |  |  |
| 8.2.2 | Rolling-element bearings |  |  |  |  |  |  |  |
| 9 | Light-transmitting parts |  |  |  |  |  |  |  |
| 10 | Breathing and draining devices which form part of a flameproof enclosure |  |  |  |  |  |  |  |
| 10.1 | Openings for breathing or draining |  |  |  |  |  |  |  |
| 10.2 | Composition limits |  |  |  |  |  |  |  |
| 10.3 | Dimensions |  |  |  |  |  |  |  |
| Sheet 3 of 67 IEC 60079-1: 2001 |  |  |  |  |  |  |  |  |

| Reference No: |  |  |  |  |  |  |  | IEC 60079-1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Clause |  |  | Results |  |  |  |  |  |
|  |  |  | Pass | Fail | N/A | Comments |  | ExCB Review |
| 10.4 | Elements with measurable paths |  |  |  |  |  |  |  |
| 10.5 | Elements with non-measurable paths |  |  |  |  |  |  |  |
| 10.6 | Removable devices |  |  |  |  |  |  |  |
| 10.7 | Mounting arrangements of the elements |  |  |  |  |  |  |  |
| 10.8 | Mechanical strength |  |  |  |  |  |  |  |
| 10.9 | Breathing devices and draining devices when used as Ex components |  |  |  |  |  |  |  |
| 10.9.1 | Mounting arrangements of the elements and components |  |  |  |  |  |  |  |
| 10.9.2 | Type tests for breathing and draining devices used as Ex components |  |  |  |  |  |  |  |
| 10.9.2.1 | Test of the ability of the breathing and draining device to withstand pressure |  |  |  |  |  |  |  |
| 10.9.2.1.1 | Test procedure |  |  |  |  |  |  |  |
| 10.9.2.1.2 | Acceptance criteria |  |  |  |  |  |  |  |
| 10.9.2.2 | Thermal Tests |  |  |  |  |  |  |  |
| 10.9.2.2.1 | Test procedure |  |  |  |  |  |  |  |
| 10.9.2.2.2 | Acceptance criteria |  |  |  |  |  |  |  |
| 10.9.2.3 | Test for non-transmission of an internal ignition |  |  |  |  |  |  |  |
| 10.9.2.3.1 | Test procedure |  |  |  |  |  |  |  |
| 10.9.2.3.2 | Acceptance criteria |  |  |  |  |  |  |  |
| 10.9.3 | Marking |  |  |  |  |  |  |  |
| 10.9.4 | Component certificate |  |  |  |  |  |  |  |
| 11 | Fasteners, associated holes and closing devices |  |  |  |  |  |  |  |
| Sheet 4 of 67 IEC 60079-1: 2001 |  |  |  |  |  |  |  |  |

| Reference No: |  |  |  |  |  |  |  | IEC 60079-1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Clause |  |  | Results |  |  |  |  |  |
|  |  |  | Pass | Fail | N/A | Comments |  | ExCB Review |
| 11.1 | Type of fastener |  |  |  |  |  |  |  |
| 11.2 | Plastic material or light alloys |  |  |  |  |  |  |  |
| 11.3 | Yield stress |  |  |  |  |  |  |  |
| 11.4 | Studs |  |  |  |  |  |  |  |
| 11.5 | Fasteners through walls |  |  |  |  |  |  |  |
| 11.6 | Blind holes |  |  |  |  |  |  |  |
| 11.7 | Screws into blind holes |  |  |  |  |  |  |  |
| 11.8 | Closing of through holes |  |  |  |  |  |  |  |
| 11.9 | Closure of apertures and compliance of blanking elements |  |  |  |  |  |  |  |
| 11.9.1 | Closing device removable from outside |  |  |  |  |  |  |  |
| 11.9.2 | Tool used to remove closing device |  |  |  |  |  |  |  |
| 11.9.3 | Special removal technique |  |  |  |  |  |  |  |
| 11.10 | Separate fastening arrangements for threaded doors/covers |  |  |  |  |  |  |  |
| 12 See also DS 2012/004 | Materials and mechanical strength of enclosures; materials inside the enclosures |  |  |  |  |  |  |  |
| 12.1 | Tests prescribed by Clauses 14 to 16 |  |  |  |  |  |  |  |
| 12.2 | Assembly of multiple flameproof enclosures |  |  |  |  |  |  |  |
| 12.3 | Intercommunicating enclosure compartments |  |  |  |  |  |  |  |
| 12.4 | Use of cast iron |  |  |  |  |  |  |  |
| 12.5 | Use of liquids |  |  |  |  |  |  |  |
| 12.6 | Insulating materials for Group I apparatus |  |  |  |  |  |  |  |
| 13 | Entries for flameproof enclosures |  |  |  |  |  |  |  |
| 13.1 | Cable entries |  |  |  |  |  |  |  |
| 13.2 | Conduit entries |  |  |  |  |  |  |  |
| Sheet 5 of 67 IEC 60079-1: 2001 |  |  |  |  |  |  |  |  |

| Reference No: |  |  |  |  |  |  |  | IEC 60079-1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Clause |  |  | Results |  |  |  |  |  |
|  |  |  | Pass | Fail | N/A | Comments |  | ExCB Review |
| 13.2.1 | Permitted for Group II only |  |  |  |  |  |  |  |
| 13.2.2 | Joint widths and gaps |  |  |  |  |  |  |  |
| 13.2.3 | Requirements for sealing device |  |  |  |  |  |  |  |
| 13.3 | Plugs and sockets and cable couplers |  |  |  |  |  |  |  |
| 13.3.1 | Construction & mounting |  |  |  |  |  |  |  |
| 13.3.2 | Flameproof joints of contact parts |  |  |  |  |  |  |  |
| 13.3.3 | Flameproof properties in the event of internal explosion |  |  |  |  |  |  |  |
| 13.3.4 | Exemption & warning label |  |  |  |  |  |  |  |
| 13.4 | Bushings |  |  |  |  |  |  |  |
| 13.4.1 | Flameproof & cemented joints |  |  |  |  |  |  |  |
| 13.4.2 | Bushings outside flameproof enclosures |  |  |  |  |  |  |  |
| 13.4.3 | Bushings specific to flameproof enclosures |  |  |  |  |  |  |  |
| 13.4.4 | Bushings not specific to one flameproof enclosure |  |  |  |  |  |  |  |
| 14 | Verification and tests |  |  |  |  |  |  |  |
| 15 | Type tests Test sequence & test method |  |  |  |  |  |  |  |
| 15.1 | Tests of ability of the enclosure to withstand pressure |  |  |  |  |  |  |  |
| 15.1.1 | General |  |  |  |  |  |  |  |
| 15.1.2 | Determination of explosion pressure (reference pressure) |  |  |  |  |  |  |  |
| 15.1.2.1 | Test procedure |  |  |  |  |  |  |  |
| 15.1.2.2 | Rotating electrical machines |  |  |  |  |  |  |  |
| 15.1.2.3 | Pressure-piling |  |  |  |  |  |  |  |
| Sheet 6 of 67 IEC 60079-1: 2001 |  |  |  |  |  |  |  |  |

| Reference No: |  |  |  |  |  |  |  | IEC 60079-1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Clause |  |  | Results |  |  |  |  |  |
|  |  |  | Pass | Fail | N/A | Comments |  | ExCB Review |
| 15.1.2.4 | Apparatus intended for use in a single gas |  |  |  |  |  |  |  |
| 15.1.3 | Overpressure test |  |  |  |  |  |  |  |
| 15.1.3.1 | Overpressure test: first method (static) |  |  |  |  |  |  |  |
| 15.1.3.2 | Overpressure test: second method (dynamic) |  |  |  |  |  |  |  |
| 15.2 | Test for non-transmission of an internal ignition |  |  |  |  |  |  |  |
| 15.2.1 | Electrical apparatus of Groups I, IIA and IIB |  |  |  |  |  |  |  |
| 15.2.1.1 | Test sample and test gas |  |  |  |  |  |  |  |
| 15.2.1.2 | Increasing of gaps for test |  |  |  |  |  |  |  |
| 15.2.1.3 | Number of tests and acceptance criterion |  |  |  |  |  |  |  |
| 15.2.2 | Electrical apparatus of Group IIC |  |  |  |  |  |  |  |
| 15.2.2.1 | First method |  |  |  |  |  |  |  |
| 15.2.2.2 | Second method |  |  |  |  |  |  |  |
| 15.2.2.3 | Single constructions |  |  |  |  |  |  |  |
| 15.3 | Test for sealing of stopping boxes with setting compound |  |  |  |  |  |  |  |
| 15.4 | Tests of flameproof enclosures with breathing and draining devices |  |  |  |  |  |  |  |
| 15.4.1 | Tests of ability of the enclosure to withstand pressure |  |  |  |  |  |  |  |
| 15.4.1.1 | Replacement of breathing and draining devices |  |  |  |  |  |  |  |
| 15.4.1.2 | Over pressure test |  |  |  |  |  |  |  |
| 15.4.2 | Thermal tests |  |  |  |  |  |  |  |
| 15.4.2.1 | Test procedure |  |  |  |  |  |  |  |
| Sheet 7 of 67 IEC 60079-1: 2001 |  |  |  |  |  |  |  |  |

| Reference No: |  |  |  |  |  |  |  | IEC 60079-1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Clause |  |  | Results |  |  |  |  |  |
|  |  |  | Pass | Fail | N/A | Comments |  | ExCB Review |
| 15.4.2.2 | Acceptance criterion |  |  |  |  |  |  |  |
| 15.4.3 | Tests for non-transmission of an internal ignition |  |  |  |  |  |  |  |
| 15.4.3.1 | Test procedure |  |  |  |  |  |  |  |
| 15.4.3.2 | Non-transmission test for breathing and draining devices |  |  |  |  |  |  |  |
| 15.4.3.2.1 | Method A |  |  |  |  |  |  |  |
| 15.4.3.2.2 | Method B |  |  |  |  |  |  |  |
| 15.4.3.3 | Acceptance criterion |  |  |  |  |  |  |  |
| 16 | Routine tests |  |  |  |  |  |  |  |
| 16.1 | General |  |  |  |  |  |  |  |
| 16.1.1 | Routine overpressure test – first method |  |  |  |  |  |  |  |
| 16.1.2 | Routine test – second method |  |  |  |  |  |  |  |
| 16.1.3 | Routine test – empty enclosure & parts of enclosure |  |  |  |  |  |  |  |
| 16.2 | Routine tests – where not required |  |  |  |  |  |  |  |
| 16.3 | Routine tests – acceptance criterion |  |  |  |  |  |  |  |
| 17 | Switchgear for Group I |  |  |  |  |  |  |  |
| 17.1 | Means of isolation |  |  |  |  |  |  |  |
| 17.1.1 | Fitted inside Ex d enclosure |  |  |  |  |  |  |  |
| 17.1.2 | Fitted inside another enclosure |  |  |  |  |  |  |  |
| 17.1.3 | Plug, socket or cable co coupler - Compliance with 13.3 |  |  |  |  |  |  |  |
| 17.2 | Doors or covers |  |  |  |  |  |  |  |
| 17.2.1 | Quick-acting doors or covers |  |  |  |  |  |  |  |
| 17.2.1.1 | Retention of properties |  |  |  |  |  |  |  |
| 17.2.1.2 | Closure of isolator |  |  |  |  |  |  |  |
| Sheet 8 of 67 IEC 60079-1: 2001 |  |  |  |  |  |  |  |  |

| Reference No: |  |  |  |  |  |  |  | IEC 60079-1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Clause |  |  | Results |  |  |  |  |  |
|  |  |  | Pass | Fail | N/A | Comments |  | ExCB Review |
| 17.2.2 | Doors or covers fixed by screws |  |  |  |  |  |  |  |
| 17.2.3 | Threaded doors or covers |  |  |  |  |  |  |  |
| 18 | Lampholders and Iampcaps |  |  |  |  |  |  |  |
| 18.1 | Device preventing lamps working loose |  |  |  |  |  |  |  |
| 18.2 | Holders and caps for lamps with cylindrical caps |  |  |  |  |  |  |  |
| 18.2.1 | Holders and caps for tubular fluorescent lamps |  |  |  |  |  |  |  |
| 18.2.2 | Other holders |  |  |  |  |  |  |  |
| 18.3 | Holders for lamps with threaded caps |  |  |  |  |  |  |  |
| 18.3.1 | Resistant to corrosion |  |  |  |  |  |  |  |
| 18.3.2 | Contact separation |  |  |  |  |  |  |  |
| 18.3.3 | E26/E27 and E39/E40 threaded lampholders |  |  |  |  |  |  |  |
| 19 | Non-metallic enclosures and non-metallic parts of enclosures |  |  |  |  |  |  |  |
| 19.1 | Permitted non-metallic enclosures |  |  |  |  |  |  |  |
| 19.2 | Special constructional requirements |  |  |  |  |  |  |  |
| 19.2.1 | Resistance to tracking and creepage distances on internal surfaces of the enclosure walls |  |  |  |  |  |  |  |
| 19.3 | Supplementary requirements for type tests |  |  |  |  |  |  |  |
| 19.3.1 | Tests for flameproofness |  |  |  |  |  |  |  |
| Sheet 9 of 67 IEC 60079-1: 2001 |  |  |  |  |  |  |  |  |

| Reference No: |  |  |  |  |  |  |  |  | IEC 60079-1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Clause |  |  |  | Results |  |  |  |  |  |
|  |  |  |  | Pass | Fail | N/A | Comments |  | ExCB Review |
| 19.3.1.1 | Test procedure |  |  |  |  |  |  |  |  |
| 19.3.1.2 | Tests of ability of the enclosure to withstand pressure |  |  |  |  |  |  |  |  |
| 19.3.1.3 | Test of erosion by flame |  |  |  |  |  |  |  |  |
| 19.3.1.4 | Test for non-transmission of an internal ignition |  |  |  |  |  |  |  |  |
| 19.3.2 | Flammability |  |  |  |  |  |  |  |  |
| 19.3.2.1 | Test procedure |  |  |  |  |  |  |  |  |
| 19.3.2.2 |  | Results |  |  |  |  |  |  |  |
| 19.3.2.2.1 |  | First alternative test method |  |  |  |  |  |  |  |
| 19.3.2.2.2 |  | Second alternative test method |  |  |  |  |  |  |  |
| 19.3.2.2.3 |  | Additional explosion tests |  |  |  |  |  |  |  |
| 19.4 |  | Test report |  |  |  |  |  |  |  |
| Annex A (Normative) |  | Additional requirements for crimped ribbon elements of breathing and draining devices |  |  |  |  |  |  |  |
| A1 |  | Crimped ribbon |  |  |  |  |  |  |  |
| A2 |  | Path dimensions |  |  |  |  |  |  |  |
| A3 |  | Annex B requirements |  |  |  |  |  |  |  |
| A4 |  | Type tests |  |  |  |  |  |  |  |
| Annex B (Normative) |  | Additional requirements for elements, with non-measurable paths, of breathing and draining devices |  |  |  |  |  |  |  |
| B1 |  | Sintered metal elements |  |  |  |  |  |  |  |
| B1.1 |  | Construction |  |  |  |  |  |  |  |
| B1.2 |  | Bubble test pore size |  |  |  |  |  |  |  |
| B1.3 |  | Density |  |  |  |  |  |  |  |
| Sheet 10 of 67 IEC 60079-1: 2001 |  |  |  |  |  |  |  |  |  |

| Reference No: |  |  |  |  |  |  |  | IEC 60079-1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Clause |  |  | Results |  |  |  |  |  |
|  |  |  | Pass | Fail | N/A | Comments |  | ExCB Review |
| B1.4 | Open porosity and/or fluid permeability |  |  |  |  |  |  |  |
| B1.5 | Identification |  |  |  |  |  |  |  |
| B2 | Pressed metal wire elements |  |  |  |  |  |  |  |
| B2.1 | Construction |  |  |  |  |  |  |  |
| B2.2 | Specifications |  |  |  |  |  |  |  |
| B2.3 | Bubble test pore size |  |  |  |  |  |  |  |
| B2.4 | Density |  |  |  |  |  |  |  |
| B2.5 | Open porosity and or fluid permeability |  |  |  |  |  |  |  |
| B2.6 | Identification |  |  |  |  |  |  |  |
| B3 | Metal foam elements |  |  |  |  |  |  |  |
| B3.1 | Construction |  |  |  |  |  |  |  |
| B3.2 | Chromium content |  |  |  |  |  |  |  |
| B3.3 | Bubble test pore size |  |  |  |  |  |  |  |
| B3.4 | Density |  |  |  |  |  |  |  |
| B3.5 | Open porosity and/or fluid permeability |  |  |  |  |  |  |  |
| B3.6 | Identification |  |  |  |  |  |  |  |
| Annex C (Normative) | Additional requirements for flameproof cable entries |  |  |  |  |  |  |  |
| Sheet 11 of 67 IEC 60079-1: 2001 |  |  |  |  |  |  |  |  |

| Reference No: |  |  |  |  |  |  |  | IEC 60079-1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Clause |  |  | Results |  |  |  |  |  |
|  |  |  | Pass | Fail | N/A | Comments |  | ExCB Review |
| C1 | Compliance with IEC 60079-0 |  |  |  |  |  |  |  |
| C2 | Constructional requirements |  |  |  |  |  |  |  |
| C2.1 | Sealing methods |  |  |  |  |  |  |  |
| C2.1.1 | Cable entries with elastomeric sealing rings |  |  |  |  |  |  |  |
| C2.1.1.1 | Minimum uncompressed axial height |  |  |  |  |  |  |  |
| C2.1.1.2 | Cable entries with only one specific elastomeric sealing ring |  |  |  |  |  |  |  |
| C2.1.2 | Cable entries sealed with setting compound |  |  |  |  |  |  |  |
| C2.2 | Threaded cable entries |  |  |  |  |  |  |  |
| ## C3 | Type tests |  |  |  |  |  |  |  |
| C3.1 | Sealing test |  |  |  |  |  |  |  |
| C3.1.1.1 | Cable entries with sealing ring |  |  |  |  |  |  |  |
| C3.1.2 | Cable entries sealed with setting compound |  |  |  |  |  |  |  |
| C3.2 | Test of mechanical strength |  |  |  |  |  |  |  |
| C3.2.1 | Cable entries with a screwed compression element |  |  |  |  |  |  |  |
| C3.2.2 | Cable entries with a compression element fixed by screws |  |  |  |  |  |  |  |
| C3.2.3 | Cable entries with setting compound |  |  |  |  |  |  |  |
| C3.2.4 | Acceptance criteria |  |  |  |  |  |  |  |
| Sheet 12 of 67 IEC 60079-1: 2001 |  |  |  |  |  |  |  |  |

| ##### Section 2 EVALUATION RECORD To be completed by ExTL # IEC 60079-1 : 2001 Electrical apparatus for explosive gas atmospheres Part 1: Flameproof enclosures "d" |  |
| --- | --- |
| ### Report |  |
| Report No. |  |
| Tested by ( + signature) Approved by (+ signature) Date of issue |  |
| Client |  |
| Name Address |  |
| Testing laboratory |  |
| Name Address Testing location |  |
| Test item Standard: IEC 60079-1 Description. Trademark. Model and/or type reference Manufacturer |  |
| Sheet 13 of 67 IEC 60079-1: 2001 |  |

| Test Item (continued) Rating(s) Type of transformers (where relevant) Application Degree of Protection Other characteristics Ambient temperature range, ta (C)……………………………… | stationary / portable / hand-held IP |
| --- | --- |
| Other information |  |
| Sheet 14 of 67 IEC 60079-1: 2001 |  |

| General remarks This evaluation record is not valid unless appended to an IECEx Assessment and Test Report issued by an ExCB, in accordance with IECEx 02. This evaluation record shall not be reproduced except in full without the written approval of the testing laboratory. The test results presented in this evaluation record relate only to the item tested. |  |
| --- | --- |
| Copy of marking plate |  |
| Sheet 15 of 67 IEC 60079-1: 2001 |  |

| ## MANUFACTURER'S DOCUMENTS |  |  |  |  |
| --- | --- | --- | --- | --- |
| Item | Subject | Drawing No. | Rev. | Date |
| Sheet 16 of 67 IEC 60079-1: 2001 |  |  |  |  |

# Evaluation Record

| ## IEC 60079-1 |  | # SECTION ONE  GENERAL 1/2 | ENCL. |  |
| --- | --- | --- | --- | --- |
| CLAUSE |  | REQUIREMENTS |  |  |
| 1 |  | ## Scope |  |  |
| Scope |  |  |  |  |
| The electrical apparatus defined below (see overview) and in 2/2 is feasible for coverage in accordance with the specific requirements addressed by this part of IEC 60079: |  |  |  |  |
|  | YES |  |  |  |
|  | NO |  |  |  |
|  | Notes: |  |  |  |
|  | (Refer to 2/2 for considerations for ambient temperatures below  20C or above 60C) |  |  |  |
| Form sheets not included in this report (reasons as indicated): |  |  |  |  |
|  | Annex A (normative); |  |  |  |
|  | Annex B (normative); |  |  |  |
|  | Annex C (normative); |  |  |  |
| Overview: |  |  |  |  |
| Sheet 17 of 67 IEC 60079-1: 2001 |  |  |  |  |

| ## IEC 60079-1 |  |  |  | SECTION ONE  GENERAL 2/2 |  |  |  | ENCL. |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ##### CLAUSE |  |  |  | REQUIREMENTS |  |  |  |  |  |  |
| 2 |  |  |  | ## Normative References |  |  |  |  |  |  |
| 3 |  |  |  | ## Definitions |  |  |  |  |  |  |
| 4 |  |  |  | Apparatus Grouping and temperature classification |  |  |  |  |  |  |
| Note: The type of protection covered by this part of IEC 60079 is dependent on the specific requirements addressed in the forms which follow. |  |  |  |  |  |  |  |  |  |  |
| Item: |  |  |  |  |  |  |  |  |  |  |
| Normative References |  |  |  |  |  |  |  |  |  |  |
|  | The following normative documents were referenced in this text. |  |  |  |  |  |  |  |  |  |
| Standard |  |  | Definition |  |  |  | Version |  |  |  |
| Definitions |  |  |  |  |  |  |  |  |  |  |
|  | All definitions covered in Clause 3 apply, as noted for the specific requirements herein |  |  |  |  |  |  |  |  |  |
| Apparatus Grouping and temperature classification (refer to forms for IEC 60079-0) |  |  |  |  |  |  |  |  |  |  |
|  | Group I apparatus |  |  |  |  | Group II apparatus, in which case requirements for specific sub-divisions are applied |  |  |  |  |
| Ambient temperatures below 20 C or above 60 C: |  |  |  |  |  |  |  |  |  |  |
|  | measures accounting for higher explosion pressures and the possibility of brittle failure at low temperatures: |  |  |  |  |  |  |  |  |  |
|  | measures accounting for decrease of the maximum safe gap at high temperatures: |  |  |  |  |  |  |  |  |  |
| Sheet 18 of 67 IEC 60079-1: 2001 |  |  |  |  |  |  |  |  |  |  |

| ###### IEC 60079-1 |  |  |  |  | SECTION TWO  CONSTRUCTION 1/25 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ENCL. |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ###### CLAUSE |  |  |  |  | REQUIREMENTS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.1 |  |  |  |  | Flameproof Joints - General requirements |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.2 |  |  |  |  | Non-threaded Joints |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.2.1 |  |  |  |  | Non-threaded Joints - Width of joints (L) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Item: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Flameproof Joints - General requirements |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Additional safety factors/tests applied to special joints: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | labyrinth (Fig. 19) |  |  |  |  |  |  |  | serrated (Fig. 10.1) |  |  |  |  |  |  |  |  |
| Note: Additional measures may be necessary in order to pass the non-transimission test of 15.2 Joint surfaces: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | protected against corrosion (method) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Note: Paint or powder coat is not permitted. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Non-threaded Joints - Width of joints (L) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Non-threaded joints other than spigot, conical and partially cylindrical surfaces (refer to Tables 1 & 2): |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Joint components |  |  |  |  |  |  | Shape |  |  |  |  | Encl. volume (cm3) |  | Width L (mm) |  |  |  |  |  |  | Distance l (mm) |  |  | Diameter (mm) |  |  |  |  | Gap i (mm) |  |  |  |  |  |
|  |  |  |  |  |  |  |  | cylindrical |  |  | flat |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | (x) |  |  | (x) |  |  |  | R |  |  | M |  |  |  | R | M |  | R |  |  | M |  | R | M |  |  |  |  |
|  | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | R = required; M = measured/actual (see also considerations for Clauses 5.2.1, 5.2.2, 5.2.4, 5.2.4.1, 5.2.4.2 and 5.2.7) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Note: Flanged joint not permitted for apparatus of IIC acetylene unless i  0.04mm, L  9.5mm & volume  500cm3, Refer clause 5.2.7 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Joint of cylindrical metallic parts (refer to table above): |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | press-fitted bushing |  |  |  |  |  | other ( |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ) |  |  |
|  |  |  | reduction of joint width to 5 mm applied, based on:- |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| and |  |  |  |  |  |  |  |  | enclosure volume, |  |  |  |  |  |  |  |  |  |  |  | cm3 Required: volume  2000 cm3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  | non-reliance on interference fit to prevent part being displaced in type test; i.e. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| and |  |  |  |  |  |  |  |  | compliance with impact test requirements; refer to forms for IEC 60079-0, Test for resistance to impact |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| and |  |  |  |  |  |  |  |  | diameter of press-fitted part, |  |  |  |  |  |  |  |  |  |  |  |  |  |  | mm Required: diameter  60 mm |  |  |  |  |  |  |  |  |  |  |  |
| Sheet 19 of 67 IEC 60079-1: 2001 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| ###### IEC 60079-1 |  |  |  |  |  |  |  | # SECTION TWO  CONSTRUCTION 2/25 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ENCL. |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ###### CLAUSE |  |  |  |  |  |  |  | REQUIREMENTS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.2.2 |  |  |  |  |  |  |  | Gap (i) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.2.3 |  |  |  |  |  |  |  | Spigot joints |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Item: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Gap (i) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Flameproof joints - Surface roughness |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Average roughness (Ra): (Ref. ISO 468) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  6.3 µm ( |  |  |  | µm); verified by |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Intentional gap between flanged joint surfaces of parts, |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | NO |  |  |  |  |  | YES; purpose: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  | Required: gap between joint surfaces shall at no point exceed the recorded maximum value |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Provision for checking gaps of flanged joints (parts, |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ): |  |  |  |
|  |  |  |  |  | directly |  |  |  |  |  | indirectly (by calculation or measurement) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | method of checking: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Required for Group I apparatus having doors and covers designed to be opened (Fig. 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Spigot joints |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Refer to Tables 1 and 2: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Joint components |  |  |  |  |  |  |  |  | Encl. volume (cm3) |  | Parts comprising width of joint, L |  |  |  |  |  |  |  |  |  |  |  |  |  | Width L (mm) |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | Cylindrical part |  |  |  |  |  | *cylindrical |  | Plane part |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | Width d [*L] (mm) |  | Width a (mm) |  | Gap i (mm) |  |  | part only | Width c (mm) |  | Width b (mm) | Gap i (mm) |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | M |  | M |  | R |  | M | (X) | M |  | M | R |  | M | R |  |  |  |  |  |  |
|  | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Notes: (i) R = required; M = measured/actual. (ii) Width L is determined by width c + width d. (iii) Required: c  6mm (IIC), c  3mm (I,IIA,IIB); d  ½ L (IIC); f  1mm. (iv) See also considerations for Clauses 5.2.4.3 (holes in joints, where a + b = distance l). (v) *Width L determined by cylindrical part only:  No gap requirements for plane part |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Gasket in plane part (refer clause 5.4): |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | gap i and width L of of plane part maintained after compression of gasket, and |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | minimum width (d or *L) of cylindrical part maintained before and after compression of gasket |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Metallic or non-flammable compressible gasket in plane part (gasket type; |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ): |  |  |  |  |
|  |  |  |  | gap i between each surface of plane part and gasket measured in each case after compression |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Sheet 20 of 67 IEC 60079-1: 2001 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| ###### IEC 60079-1 |  |  |  |  | SECTION TWO  CONSTRUCTION 3/25 |  |  |  |  |  |  | ENCL. |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ###### CLAUSE |  |  |  |  | REQUIREMENTS |  |  |  |  |  |  |  |  |  |  |
| 5.2.4 |  |  |  |  | ## Holes in joint surfaces |  |  |  |  |  |  |  |  |  |  |
| 5.2.4.1 |  |  |  |  | Flanged joints with holes outside the enclosure |  |  |  |  |  |  |  |  |  |  |
| 5.2.4.2 |  |  |  |  | Flanged joints with holes inside the enclosure |  |  |  |  |  |  |  |  |  |  |
| 5.2.4.3 |  |  |  |  | Spigot joints (............cylindrical part and a plane part) |  |  |  |  |  |  |  |  |  |  |
| 5.2.4.4 |  |  |  |  | Spigot joints (............plane part only) |  |  |  |  |  |  |  |  |  |  |
| Item: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Holes in joints Surface |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Joint surface interrupted by holes for clamping bolts etc. (parts, |  |  |  |  |  |  |  |  |  |  |  |  | ): |  |  |
|  |  | distance l  6 mm for width L < 12.5 mm |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | distance l  8 mm for width L >12.5 mm and < 25 mm |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | distance l  9 mm for width L  25 mm |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Flanged joints with holes outside the enclosure |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Measurement of distance l for parts, |  |  |  |  |  |  |  |  |  |  |  |  |  | : |  |
|  |  | between hole and inside of enclosure |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Flanged joints with holes inside the enclosure |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Measurement of distance l for parts, |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | between hole and outside of enclosure |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Spigot joints (............cylindrical part and a plane part) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| For parts, |  |  |  |  |  |  |  |  |  |  |  |  |  | : |  |
|  |  | distance l comprises width a of cylindrical part + width b of plane part, based on:- |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | at chamfer, f  1 mm and |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | gap of cylindrical part  0.2 mm (Groups I, IIA) |  |  |  |  |  |  |  |
| or |  |  |  |  |  |  |  | gap of cylindrical part  0.15 mm (Group IIB) |  |  |  |  |  |  |  |
| or |  |  |  |  |  |  |  | gap of cylindrical part  0.1 mm (Group IIC) |  |  |  |  |  |  |  |
|  |  | distance l is based on width b of the plane part alone |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Spigot joints (............plane part only) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | distance / is based on the width of the plane part between the inside or outside of the enclosure and a hole |  |  |  |  |  |  |  |  |  |  |  |  |  |
| or |  |  |  |  |  |  |  | gap of cylindrical part  0.2 mm (Groups I, IIA) |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | gap of cylindrical part  0.15 mm (Group IIB) |  |  |  |  |  |  |  |
| or |  |  |  |  |  |  |  | gap of cylindrical part  0.04 mm (Group IIC) enclosure volume < 500cm3 (Group IIC) distance L > 9.5 mm (Group IIC) |  |  |  |  |  |  |  |
|  |  | distance l is based on the width b of the plane part. |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Sheet 21 of 67 IEC 60079-1: 2001 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| ###### IEC 60079-1 |  |  |  |  | SECTION TWO  CONSTRUCTION 4/25 |  |  |  |  |  |  |  |  |  |  |  | ENCL. |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ###### CLAUSE |  |  |  |  | REQUIREMENTS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.2.5 |  |  |  |  | ## Conical joints |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.2.6 |  |  |  |  | Joints with partial cylindrical surfaces (not permitted for Group IIC) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Item: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Conical joints |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Joint components |  |  |  |  |  | Encl. volume (cm3) |  | Width L (mm) |  |  |  | Distance l (mm) |  | Diameter (mm) |  |  | Gap i (mm) |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  | R |  | M |  | R | M | R | M |  | R | M |  |  |  |  |
|  | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | R = required; M = measured/actual |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Joint including conical surfaces (refer to table above): |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | gap is uniform through-out the conical part (verified by |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ) |  |  |
|  |  |  | cone angle  5 ( |  |  |  |  | ) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Note: The width of the joint and the gap normal to the surfaces shall comply with the relevant dimensions in Tables I, IIA, IIB and IIC. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Joints with partial cylindrical surfaces (not permitted for Group IIC) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Joint components |  |  |  |  |  | Encl. volume (cm3) |  |  | Width L (mm) |  |  | Distance l (mm) |  | Diameter (mm) |  |  | Gap i (mm) |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | R |  | M | R | M | R | M |  | R | M |  |  |  |  |
|  | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Sheet 22 of 67 IEC 60079-1: 2001 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| ###### IEC 60079-1 |  |  |  |  |  | SECTION TWO  CONSTRUCTION 5/25 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ENCL. |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ###### CLAUSE |  |  |  |  |  | REQUIREMENTS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.2.7 |  |  |  |  |  | Additional requirements for joints of electrical apparatus of Group IIC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.2.8 |  |  |  |  |  | Serrated joints |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.3 |  |  |  |  |  | Threaded joints |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Item: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Additional requirements for joints of electrical apparatus of Group IIC (apparatus for use in acetylene) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Gap i < 0.040 mm for L> 9.5 mm and enclosure volume <500 cm3 (Otherwise, flanged joints are not permitted) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Refer to 1/25, table for non-threaded joints other than spigot joints (Page_____________ ) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Serrated joints |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Joint Components |  |  |  |  |  |  |  | Included angle (º) |  |  |  |  |  |  |  | Number of Serrations Engaged |  |  |  |  |  | Pitch T (mm) |  |  |  |  |  |  | Gap i (mm) |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | R |  |  |  | M |  |  |  | R |  |  | M |  |  | R |  |  | M |  |  |  | R |  |  | M |  |  |  |  |  |  |
|  |  |  | 1 |  |  |  |  |  |  |  | 60 |  |  |  |  |  |  |  | ≥ 5 |  |  |  |  |  | ≥ 1,25 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | R = Required M = Measured |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Apparatus shall be marked “X” |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Threaded joints Cylindrical threaded joints: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Joint components |  |  |  | Class of Fit (x) |  |  |  |  |  | Pitch |  |  |  |  |  |  |  | Threads engaged |  |  |  |  |  | Axial engagement for enclosure volume: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | Medium |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | <100 cm3 |  |  |  |  |  |  | > 100cm3 |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | Fine |  |  |  | R |  |  |  | A |  |  |  | R |  |  | A |  |  | R |  |  | A |  |  |  | R |  |  | A |  |  |  |  |  |
|  |  |  | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | > 5 |  |  |  |  |  | > 5mm |  |  |  |  |  |  | >8 mm |  |  |  |  |  |  |  |
|  |  |  | 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | > 5 |  |  |  |  |  | > 5mm |  |  |  |  |  |  | >8 mm |  |  |  |  |  |  |  |
|  |  |  | 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| R = Required A = Actual |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Notes: (i) *Class of fit: medium, 0.7 pitch  2 mm; fine, 2 mm  pitch  3 mm; ref. ISO 965-1 and 965-3. (ii) Cylindrical threaded joints which do not conform with the ISO standard are permitted if the test for non-transmission of an internal ignition is passed. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Sheet 23 of 67 IEC 60079-1: 2001 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| ###### IEC 60079-1 |  |  |  |  |  |  | SECTION TWO  CONSTRUCTION 6/25 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ###### CLAUSE |  |  |  |  |  |  | REQUIREMENTS |  |  |  |  |  |  |  |  |  |  |  |  | ENCL. |  |  |  |  |  |  |  |
| 5.3 |  |  |  |  |  |  | Threaded joints (continued) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.4 |  |  |  |  |  |  | Gaskets (including O-rings) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Item: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Ref. Tests for non-transmission of an internal ignition |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Joint components |  |  |  |  |  |  |  |  | Pitch (mm) Required:  0.7 mm |  |  |  |  |  |  |  |  |  | Threads engaged |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  | Thread Form |  | Class of Fit (X) |  |  |  | Axial Engagement |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Medium |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Fine |  |  |  | R |  |  |  | M |  |  |  |  |
| 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Flameproof joints – Taper Threaded joints |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Joint components |  |  |  |  |  |  |  | Pitch (mm) Required:  0.9 |  |  |  |  | Threads engaged |  |  |  | Threads provided on each part |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  | Required |  | Actual |  | Required |  | Actual |  |  |  |  |  |  |  |  |
| 1 |  |  |  |  |  |  |  |  |  |  |  |  |  5 |  |  |  | > 6 |  |  |  |  |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  |  |  |  |  |  |  |  |  |  |  |  |  5 |  |  |  | > 6 |  |  |  |  |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Gaskets (including O-rings) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Gasket of compressible material: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ###### Joint components |  |  |  |  |  |  |  |  |  |  | Gasket |  |  |  |  |  |  |  |  |  | ###### In flame path: |  |  |  |  |  |  |
| 1 |  |  |  |  |  |  |  |  |  |  | ###### affixed to component: |  |  | ###### Material |  |  |  |  |  |  | Width L |  |  | Distance I |  |  |  |
| 2 |  |  |  |  |  |  |  |  |  |  | ###### (X) |  |  |  |  |  |  |  |  |  | ###### NO | ###### YES |  | ###### NO | ###### YES |  |  |
| 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Notes: |  |  | (i) | Not applicable to the entry of conductors and cables (See 13.1) or to transparent elements of luminaires. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | (ii) | Joints of bushings and light-transmitting parts may contain a gasket of metal or compressible non-flammable material (compressible material as determined by ISO 1210, with a metal sheath). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | (iii) | The gasket shall be applied as a supplement to but not included in the flameproof joint. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Closure of flameproof joints per the requirements of applicable Tables 1 and 2; for the joints of |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | ( |  |  | components, |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ): |  |
|  |  |  |  |  | gap i maintained |  |  |  |  |  |  | width L maintained |  |  |  |  |  | distance l maintained |  |  |  |  |  |  |  |  |  |
| Sheet 24 of 67 IEC 60079-1: 2001 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| ###### IEC 60079-1 |  |  |  |  | SECTION TWO  CONSTRUCTION 7/25 |  |  | ENCL. |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ###### CLAUSE |  |  |  |  | REQUIREMENTS |  |  |  |  |  |  |
| 5.5 |  |  |  |  | Apparatus using capillaries |  |  |  |  |  |  |
| 6 |  |  |  |  | Cemented Joints |  |  |  |  |  |  |
| 6.1 |  |  |  |  | General |  |  |  |  |  |  |
| 6.2 |  |  |  |  | Mechanical Strength |  |  |  |  |  |  |
| 6.3 |  |  |  |  | Width of Cemented Joint |  |  |  |  |  |  |
| Item: |  |  |  |  |  |  |  |  |  |  |  |
| Apparatus using capillaries |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Apparatus complies with Tables 1 or 2 . Minimum width of joint mm, Max. Gap mm, for Volume cm3 |  |  |  |  |  |  |  |  |
|  |  |  | Apparatus complies with non-transmission test refer to 15.2 of IEC 60079-1. |  |  |  |  |  |  |  |  |
| Cemented joints - General |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Joint complies with the requirements of clause 5 |  |  |  |  |  |  |  |  |
|  |  |  | Joint complies with test in 23.4.7.3 & 23.4.7.4 of IEC 60079-0 |  |  |  |  |  |  |  |  |
| Mechanical Strength |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Mechanical strength of assembly does not depend upon the addition of the cement alone through: |  |  |  |  |  |  |  |  |
|  |  |  | Details: |  |  |  |  |  |  |  |  |
|  |  |  | Cemented joint complies with Clause 15.3 of IEC 60079-1 |  |  |  |  |  |  |  |  |
| Width of cemented joints |  |  |  |  |  |  |  |  |  |  |  |
| Shortest path through cemented joint (inside to outside enclosure), |  |  |  |  |  |  | mm for:- |  |  |  |  |
|  |  |  | enclosure volume  10 cm3 Required: path  3mm |  |  |  |  |  |  |  |  |
|  |  |  | 10 cm3 enclosure volume  100 cm3 Required: path  6mm |  |  |  |  |  |  |  |  |
|  |  |  | enclosure volume  100 cm3 Required: path  10mm |  |  |  |  |  |  |  |  |
|  |  | Sheet 25 of 67 IEC 60079-1: 2001 |  |  |  |  |  |  |  |  |  |

| ###### IEC 60079-1 |  |  |  |  |  | SECTION TWO  CONSTRUCTION 8/25 |  |  |  |  |  |  | ENCL. |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ###### CLAUSE |  |  |  |  |  | REQUIREMENTS |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7 |  |  |  |  |  | Operating Rods |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7.1 |  |  |  |  |  | Diameter of Operating Rods |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7.2 |  |  |  |  |  | Diametrical Clearance |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Item: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Operating rods |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Dimensions (refer to Tables 1 and 2): |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Joint components |  |  |  |  |  | Enclosure volume (cm3) | Joint width L (mm) |  | Rod diameter (mm) |  | Joint width L (mm) [  L] |  | Diametrical clearance i (mm) |  |  |  |  |  |
|  |  |  |  |  |  |  |  | R | M |   L |   L | R |  | R | M |  |  |  |  |
|  | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| R = required; M = measured/actual |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Diametrical Clearance |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Enlargement of diametrical clearance by wear in normal use: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | not applicable; reason: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | means to counteract enlargement: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Sheet 26 of 67 IEC 60079-1: 2001 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| ###### IEC 60079-1 |  |  |  |  |  |  | SECTION TWO  CONSTRUCTION 9/25 |  |  |  |  |  |  |  |  |  |  |  |  | ENCL. |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ###### CLAUSE |  |  |  |  |  |  | REQUIREMENTS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8 |  |  |  |  |  |  | Supplementary Requirements for Shafts and bearings |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.1 |  |  |  |  |  |  | Joints of shafts |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.1.1 |  |  |  |  |  |  | ###### Cylindrical Joints |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.1.2 |  |  |  |  |  |  | Labyrinth Joints |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Item: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Cylindrical Joints |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Cylindrical/joint for: |  |  |  |  |  |  | Enclosure Volume (cm3) | Shaft dia. (mm) | Length of flame path L (mm) |  |  |  | Diametrical Clearance D-d (mm) |  |  |  | Maximum radial clearance, m (mm) |  |  |  | Minimum radial clearance, k (mm) |  |  |  |
|  |  |  |  |  |  |  |  |  |  | R |  | M |  | R |  | M |  |  |  |  |  |  |  |  |  |
|  | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Notes: (i) R = required; M = measured. (ii) Grooves for grease retention shall neither be taken into account for L nor interrupt L (ref. Fig. 18). (ii) Required: k (ref. Fig. 21)  0,05 mm |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Labyrinth Joints |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Labyrinth joint for: |  |  |  |  |  | Enclosure Volume (cm3) | Shaft dia. (mm) |  | Length of flame path L (mm) |  |  |  | Diametrical clearance i (mm) |  |  |  | Maximum radial clearance, m (mm) |  |  | Minimum radial clearance, **k (mm) |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | R |  | M |  | R |  | M |  |  |  |  |  |  |  |  |
|  |  | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Notes: (i) R = required; M = measured. (ii) Joints not complying with tables 1 and 2 may be accepted if tests of clauses 14 through 16 are satisfied. (ii) Required: k (ref. Fig. 21)  0,05 mm |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | joints do not comply with tables 1 and 2; refer to sheets for test clauses 14 – 16 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Sheet 27 of 67 IEC 60079-1: 2001 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| ###### IEC 60079-1 |  |  |  |  |  |  |  | SECTION TWO  CONSTRUCTION 10/25 |  |  |  |  |  |  |  | ENCL. |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ###### CLAUSE |  |  |  |  |  |  |  | REQUIREMENTS |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.1.3 |  |  |  |  |  |  |  | Joints with Floating Glands |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.2.1 |  |  |  |  |  |  |  | Sleeve bearings |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.2.2 |  |  |  |  |  |  |  | Rolling-element bearings |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Item: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Joints with Floating Glands |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Gland/shaft joint for: |  |  |  |  |  |  |  | Enclosure volume (cm3) | Shaft dia. (mm) | Length of flame path L (mm) |  | Diametrical clearance i (mm) |  | Maximum radial clearance, m (mm) |  | Minimum radial clearance, k (mm) |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | R | M | R | M |  |  |  |  |  |  |  |
|  | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Notes: (i) R = required; M = measured. (ii) Not permitted for Group IIC (iii) Required: A device shall prevent rotation of the gland (Fig. 20) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Sleeve bearings |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | joint face of non-sparking metal; details: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Sleeve Bearing/shaft joint for: |  |  |  |  |  |  | Enclosure volume (cm3) | Shaft dia. (mm) | Length of flame path L (mm) |  | Diametrical clearance i (mm) |  | Maximum radial clearance, m (mm) |  | Minimum radial clearance, k (mm) |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | R | M | R | M |  |  |  |  |  |  |  |
|  |  | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Notes: (i) R = required; M = measured. (ii) Sleeve bearings are not permitted for rotating electrical machines for Group IIC. (iii) Required: - k (ref. Fig. 21)  0.05 mm - L  shaft diameter in a flameproof gland associated with a sleeve bearing (L ≤ 25 mm) - for a cylindrical or labyrinth joint in a rotating machine one face of the gland shall be of non-sparking material if the stator/rotor air gap exceeds specified k and minimum thickness of the non-sparking metal shall be greater than the air gap |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Rolling element bearings |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | shaft glands equipped with rolling element bearings; details, including clearance m: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Required: m < ⅔ k per tables 1 and 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Sheet 28 of 67 IEC 60079-1: 2001 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| ###### IEC 60079-1 |  |  |  |  |  |  |  | SECTION TWO  CONSTRUCTION 11/25 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ENCL. |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ###### CLAUSE |  |  |  |  |  |  |  | REQUIREMENTS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 9 |  |  |  |  |  |  |  | Light-transmitting parts |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 10 |  |  |  |  |  |  |  | Breathing and daining dvices whch form part of a flameproof enclosure |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Item: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Light-transmitting parts |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Part: |  |  |  |  |  |  |  |  |  |  |  |  | Transparent material: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | fixed directly into enclosure as an integral part by cement or sealing material |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | clamped directly into enclosure (with gasket) |  |  |  |  |  |  |  |  |  |  |  |  |  |  | clamped directly into enclosure (without gasket) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | sealed or cemented into a frame clamped into enclosure so that window is replaceable as a unit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | refer to Cl. 5.4, Gaskets and O-rings |  |  |  |  |  |  |  |  |  |  |  |  |  |  | refer to Cl. 6, Cemented joints |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | means preventing mechanical stress; details: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Relevant tests: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | thermal stability |  |  |  |  |  |  | impact |  |  |  |  |  |  |  | thermal shock |  |  |  |  | insulation resistance |  |  |  |  |  |  |  |  |  |
|  |  |  | refer to forms for IEC 60079-0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | refer to |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Breathing and daining dvices whch form part of a flameproof enclosure |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Note: Breathing and draining devices are not intended to withstand continuous burning on their surfaces. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Manufacturer and designation (if certified, details) |  |  |  |  |  |  |  |  |  | Certified (YES/NO) | Permeable element |  |  |  |  |  |  |  | Location |  |  |  |  | Can be taken apart (NO/YES) | Subject to clogging (NO/YES) |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | crimped ribbon |  | pressed wire | sintered |  | foam |  | other |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | (X) |  | (X) | (X) |  | (X) |  | (X) |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Can withstand the pressure created by an internal explosion, refer over pressure test, clause 10.9.2.1 or 15.4.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Prevents the transmission of explosion, refer tests in clause 10.9.2.3 or 15.4.3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Sheet 29 of 67 IEC 60079-1: 2001 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| ###### IEC 60079-1 |  |  |  |  |  | SECTION TWO  CONSTRUCTION 12/25 |  |  |  | APPLIED |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ###### CLAUSE |  |  |  |  |  | REQUIREMENTS |  |  |  | YES |  |  | NO |  | ENCL. |  |  |  |  |  |
| 10.1 |  |  |  |  |  | Openings for Breathing or Draining |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 10.2 |  |  |  |  |  | Composition Limits |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 10.3 |  |  |  |  |  | Dimensions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 10.4 |  |  |  |  |  | Elements with measurable paths |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Item: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Openings for Breathing or Draining |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Note: If for technical reasons breathing or draining devices have to be provided, they should be so constructed that they are not liable to become inoperative in service (for example, because of the accumulation of dust or paint). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | device incorporating permeable element other than the identified types; details: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Note: Openings for breathing or draining shall not be produced by deliberate enlargement of gaps of flanged joints. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Composition Limits |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Composition limits; details/reference specification: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | used with acetylene ( |  |  |  |  | element does not comprise more than 60  of copper by mass) |  |  |  |  |  |  |  |  |  |  |  |  |
| Dimensions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Details/refer to: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| # Elements with measurable paths |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| # Interstices and measurable lengths of path: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | # Between (parts): |  |  |  |  |  |  |  | Enclosure volume # (cm3) |  | # Width L # (mm) | # Gap i # (mm) |  | # Comply with Table (YES/NO) |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | # 1 |  | # 2 |  |  |  |  |
| # Note: Interstices and measurable lengths of path need not comply with the values given in Tables 1 and 2 provided that the elements pass the tests of clause 14 through 16. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| # Joint between device and flameproof enclosure: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | refer to Annex A for additional requirements for crimped ribbon elements |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Sheet 30 of 67 IEC 60079-1: 2001 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| ###### IEC 60079-1 |  |  |  | SECTION TWO  CONSTRUCTION 13/25 |  |  |  |  | ENCL. |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ###### CLAUSE |  |  |  | REQUIREMENTS |  |  |  |  |  |  |  |
| 10.5 |  |  |  | Elements with non-measurable paths |  |  |  |  |  |  |  |
| 10.6 |  |  |  | Removable devices |  |  |  |  |  |  |  |
| 10.7 |  |  |  | Mounting arrangements of the elements |  |  |  |  |  |  |  |
| 10.8 |  |  |  | Mechanical Strength |  |  |  |  |  |  |  |
| Item: |  |  |  |  |  |  |  |  |  |  |  |
| # Elements with non-measurable paths |  |  |  |  |  |  |  |  |  |  |  |
| Note: For functional reasons, it may also be necessary to state the fluid permeability and the open porosity specified in accordance with the standard methods for the particular material and the particular manufacturing methods (see annex B) |  |  |  |  |  |  |  |  |  |  |  |
|  |  | ###### refer to Annex B for additional requirements for elements with non-measurable paths, of breathing and draining devices: |  |  |  |  |  |  |  |  |  |
| # Removable devices |  |  |  |  |  |  |  |  |  |  |  |
|  |  | # device is designed to avoid reduction/enlargement of openings during re-assembly; details: |  |  |  |  |  |  |  |  |  |
| ###### Mounting arrangements of the elements |  |  |  |  |  |  |  |  |  |  |  |
| Integral assembly: |  |  |  |  |  |  |  |  |  |  |  |
|  |  | fixed (as below) in a suitable mounting component, replaceable as a unit; method of attachment to enclosure: |  |  |  |  |  |  |  |  |  |
|  |  | Mounted so as to form a flameproof joint with the enclosure: |  |  |  |  |  |  |  |  |  |
| clamping ring |  |  |  |  |  | (other means) |  |  |  |  |  |
| from within enclosure ( |  |  |  |  |  | screws/clamping rings are accessible only from inside the enclosure) |  |  |  |  |  |
| from outside enclosure ( |  |  |  |  |  | refer to SECTION TWO  CONSTRUCTION, Clause 11, Fasteners) |  |  |  |  |  |
| joint between element and flameproof enclosure: |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | refer to SECTION TWO  CONSTRUCTION (Clause 5 as applicable) |  |  |  |  |
| Note: Exception that the requirement for surface roughness of the element can be waived if the element passes the type test. (ref. Clause 14 through 16, Tests for non-transmission of an internal ignition) |  |  |  |  |  |  |  |  |  |  |  |
| Mechanical strength Note: Applicable to the device and its guard, if any, mounted in the intended manner. |  |  |  |  |  |  |  |  |  |  |  |
|  |  | refer to forms for IEC 60079-0, Test for resistance to impact |  |  |  |  |  |  |  |  |  |
|  |  | Results: |  |  |  |  |  |  |  |  |  |
|  | Sheet 31 of 67 IEC 60079-1: 2001 |  |  |  |  |  |  |  |  |  |  |

| ###### IEC 60079-1 |  |  |  | SECTION TWO  CONSTRUCTION 14/25 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ###### CLAUSE |  |  |  | REQUIREMENTS |  |  |  |  |  |  |  |  |  |  |  | ENCL. |  |  |  |
| 10.9 |  |  |  | Breathing and draining devices when used as Ex components |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 10.9.1 |  |  |  | Mounting arrangements of the elements and components |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 10.9.2 |  |  |  | Type tests for breathing and draining devices used as Ex components |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 10.9.2.1 |  |  |  | Test of the ability of the breathing and draining device to withstand pressure |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 10.9.2.1.1 |  |  |  | Test Procedure |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Item: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Breathing and draining devices when used as Ex components |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Enclosure volume:__________ < 3L |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Enclosure volume: __________ > 3L, refer to 15.4 of IEC 60079-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Mounting arrangements of the elements and components |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Mounted so as to form a flameproof joint with the enclosure: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | clamping ring |  |  |  |  |  | (other means) |  |  |  |  |  |  |  |  |  |  |  |
|  |  | from within enclosure ( |  |  |  |  |  | screws/clamping rings are accessible only from inside the enclosure) |  |  |  |  |  |  |  |  |  |  |  |
|  |  | from outside enclosure ( |  |  |  |  |  | refer to SECTION TWO  CONSTRUCTION, Clause 11, Fasteners) |  |  |  |  |  |  |  |  |  |  |  |
| joint between element and flameproof enclosure: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | refer to SECTION TWO  CONSTRUCTION (Clause 5 and 6 as applicable) |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Note: Exception that the requirement for surface roughness of the element can be waived if the element passes the type test. (ref. 17/25 and 5/15, Tests for non-transmission of an internal ignition) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type tests for breathing and draining devices used as Ex components Note: Applicable to the device and its guard, if any, mounted in the intended manner. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | refer to forms for IEC 60079-0, Test for resistance to impact; results: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Device with non-measurable path: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | pore size > 85% of the specified maximum pore size ( |  |  |  |  |  |  |  |  |  |  |  |  | ) |  |  |  |  |
| ###### Test of the ability of the breathing and draining device to withstand pressure |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reference test pressure: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 1,200 kPa (Gp I) |  |  |  |  | 1,350 kPa (Gp IIA) |  |  |  | 2,500 kPa (Gp IIB) |  |  | 4,000 kPa (Gp IIC) |  |  |  |  |  |
| Overpressure test: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | thin flexible membrane fitted to inner surfaces (method): |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 1.5 × reference pressure applied for 1 min. Required: Each component to be submitted to a routine test |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 4 × reference pressure applied for 1 min. (alternatively) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Observed permanent damage or deformation of the device: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | NO |  |  |  |  | YES (details): |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Sheet 32 of 67 IEC 60079-1: 2001 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| ###### IEC 60079-1 |  |  |  | SECTION TWO  CONSTRUCTION 15/25 |  |  |  | ENCL. |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ###### CLAUSE |  |  |  | REQUIREMENTS |  |  |  |  |  |  |  |  |  |  |  |
| 10.9.2.1.2 |  |  |  | Acceptance criteria |  |  |  |  |  |  |  |  |  |  |  |
| 10.9.2.2 |  |  |  | Thermal tests |  |  |  |  |  |  |  |  |  |  |  |
| Item: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Acceptance criteria |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Deformation or damage affecting the type of protection YES/NO (details: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  | ) |  |  |
| Thermal tests: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Breather and draining devices intended for multiple flameproof enclosures |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Ignition source: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | high voltage sparking plug ( |  |  |  |  |  |  |  |  |  |  | ) |  |  |
|  |  | low energy ignition source ( |  |  |  |  |  |  |  |  |  |  | ) |  |  |
|  |  | contained ignition-capable switching device ( |  |  |  |  |  |  |  |  |  |  | ) |  |  |
|  |  | fitted at inlet and 50mm from the inside of the end-plate housing the component |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test set-up: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | sample device fitted to enclosure and placed in test chamber (volume |  |  |  |  |  |  |  | ) |  |  |  |  |  |
|  |  | same test mixture throughout (volume ratio, test chamber/enclosure to which the device is fitted: |  |  |  |  |  |  |  |  | / |  |  |  |  |
|  | Sheet 33 of 67 IEC 60079-1: 2001 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| ###### IEC 60079-1 |  |  |  |  |  |  |  | SECTION TWO – CONSTRUCTION 16/25 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ENCL. |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ###### CLAUSE |  |  |  |  |  |  |  | REQUIREMENTS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 10.9.2.2 |  |  |  |  |  |  |  | Thermal tests (continued) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Item: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Thermal tests (continued) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Devices for Groups I, IIA, IIB, IIC: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Test No |  |  | Ex component for multiple use in any single flameproof enclosure |  | Ignition source location i.e. other than for an Ex component sample mounted on test rig position producing the highest peak explosion pressure and rate of rise of pressure at the face of the device and considering the device location which gives the most unfavourable results where two or more identical devices are fitted |  |  |  |  |  |  | Ex component sample (mounted on test rig) |  |  |  | Test mixture |  |  |  | test gas flow through the device and the enclosure |  | ventilation or sampling system operated per mfr.'s documentation | Test mixture maintained (external to device) after each test for at least (min) sufficient time to reveal any continuous burning on device face; e.g.  10 min |  |  |  | External surface temperature of device (C) |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | Spark plug location |  |  | (4.2  0.1) % C3H8, in air applied for all Groups |  | (7.5  1) % C2H2, in air device for use in acetylene | at atmospheric pressure and temperature |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | 50 mm from inside of test rig end plate |  | at test rig inlet |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | (X) |  |  |  |  |  |  |  |  | (X) | (X) |  | (X) | (X) |  | (X) | (X) | (X) |  | (X) |  |  |  |  |  |  |  |  |  |  |
|  | Notes 1. For devices for hydrogen only, tests with mixtures including acetylene are not required. 2. The surface temperature of the device should be at ambient at the start of each test. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Continuous burning: |  |  |  |  |  |  |  |  |  | NO |  |  |  |  |  | YES; Test No |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Acceptance criteria |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Flame transmission: |  |  |  |  |  |  |  |  |  | NO |  |  |  |  |  | YES; Test No |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Maximum external surface temperature of the device: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | C at ambient, |  |  |  |  |  |  |  | C |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (surface temperatures measured by |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ) |  |  |
| (maximum external surface temperature rise of the device, |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | K)  1.2 = |  |  |  |  |  |  | K |  |  |  |  |
| Notes: 1. The measured external surface temperature rise of the device shall be multiplied by a safety factor of 1.2 for the determination of the temperature class of the electrical apparatus. 2. Breathing and draining dev ices which fail any of the tests of 10.9 are excluded from certification as a component device; however, they may be used as an integral part of a flameproof enclosure provided they are tested with a specific enclosure in accordance with 15.4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Sheet 34 of 67 IEC 60079-1: 2001 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| ###### IEC 60079-1 |  |  |  |  |  | SECTION TWO – CONSTRUCTION 17/25 |  |  |  |  |  |  |  |  |  |  |  |  | ENCL. |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ###### CLAUSE |  |  |  |  |  | REQUIREMENTS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 10.9.2.3.1 |  |  |  |  |  | Test for non-transmission of an internal ignition |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Item: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test for non-transmission of an internal ignition |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | sample device fitted to enclosure and placed in test chamber (volume, |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ) |  |  |
| Devices for Groups I, IIA, IIB: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Test No |  |  | Ignition source (as for Thermal tests, 15/25) close to the inner surface of the device and then (if necessary) in one or more places producing the highest peak explosion pressure and rate of rise of pressure at the device face |  |  | Ex component sample (mounted on test rig) |  |  | Normal condition (Note 2) |  |  | Alternative for Groups IIA, IIB devices with measurable paths i.e. where damage by hydrogen/air test mixtures is possible |  |  |  |  |  |  |  |  |  | Flame transmission (NO/YES) |  |  |  |  |
|  |  |  |  |  |  |  |  | Spark plug location |  | Explosive test mixture (same mixture throughout) |  |  |  |  |  | Increased joint gap |  |  |  | Reduction of length of thread engagement (Note 3) |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | Group I; 12.5  0.5 % methane-hydrogen (58/42  1 %) in air | Group IIA; 55  0.5 % hydrogen in air | Group IIB; 37  0.5 % hydrogen in air |  | Group IIA; 4.2  0.1 % propane in air | Group IIB; 6.5  0.5 % ethylene in air |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | 50 mm from inside of test rig end plate | at test rig inlet |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Location i.e. other than for an Ex component sample mounted on test rig |  |  |  |  |  |  |  |  |  |  |  | 1.42 iC (mm) Group IIA |  | 1.85 iC (mm) Group IIB |  | none (taper-threaded) | ⅓ | ½ |  |  |  |  |  |
|  |  |  |  |  |  |  | (X) | (X) | (X) | (X) | (X) | (X) |  | (X) | (X) |  |  |  |  | (X) | (X) | (X) |  |  |  |  |  |
|  |  |  |  | close to the inner surface of the device |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Notes 1. iC = the maximum constructional gap, as specified in the manufacturer's drawings iT = the maximum gap permitted in Table I, IIA or IIB as applicable 2. Where no artificial gap is created, the test gap shall be 80 to 100 % of the specified maximum constructional gap, but not exceeding iT. 3. Required reduction of length of thread engagement: - by ⅓ of manufacturer's specification for joints conforming to ISO fit - by ½ of manufacturer's specification for joints worse than ISO fit - no reduction of taper-threaded joints continued on 18/25 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Sheet 35 of 67 IEC 60079-1: 2001 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| ###### IEC 60079-1 |  |  |  |  | SECTION TWO – CONSTRUCTION 18/25 |  |  |  |  |  |  |  |  |  |  | ENCL. |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ###### CLAUSE |  |  |  |  | REQUIREMENTS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 10.9.2.3.1 |  |  |  |  | Test for non-transmission of an internal ignition (continued) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Item: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test for non-transmission of an internal ignition (continued) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Devices for Group IIC: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Test No |  | Ignition source (as for Thermal tests, 15/25) close to the inner surface of the device and then (if necessary) in one or more places producing the highest peak explosion pressure and rate of rise of pressure at the device face |  |  | Ex component sample (mounted on test rig) |  |  | Measurable paths |  |  |  |  |  | Measurable or non- measurable paths |  |  |  |  | Flame transmission (NO/YES) |  |  |  |
|  |  |  |  |  |  |  | Spark plug location |  | Reduction of length of thread engagement |  |  |  |  |  | Method A |  |  | Method B (Note 2) |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  | Increased joint gap |  |  |  |  | ("⅓"; ISO fit) or ("½"; worse than ISO fit) or ("none"; taper) |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  | 27  1.0 % H2 in air (test mixture at atmospheric pressure) | 7.5  1 % C2H2 in air (test mixture at atmospheric pressure) | cylindrical (incl. spigot, width L) | flanged ( 0.1mm) and cylindrical (incl. spigot, width L) | flanged only |  | 27  1.0 % H2 in air (1.5 bar precompression of test mixture) |  | 7.5  1 % C2H2 in air (1.5 bar precompression of test mixture) | (40  1 %) H2, (20  1 %) O2, bal. N2 (test mixture at atmospheric pressure) | (10  1 %) C2H2, (24  1 %) O2, bal. N2 (test mixture at atmospheric pressure) |  |  |  |  |
|  |  |  |  |  |  |  | 50 mm from inside of test rig end plate | at test rig inlet |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Location i.e. other than for an Ex component sample mounted on test rig |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | iC + ½ iT (mm) | iC + ½ iC (mm) | 1.5 iT (mm) |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | (X) | (X) | (X) | (X) | (X) |  |  |  |  | (X) |  | (X) | (X) | (X) |  |  |  |  |
|  |  |  | close to the inner surface of the device |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | close to the inner surface of the device |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Notes 1. For devices for hydrogen only, tests with mixtures including acetylene are not required. 2. In the case of Method B the apparatus shall be marked with sign X indicating exclusion of ethyl nitrate and where the device is for use with enclosure volume > 100 cm3, indicating exclusion of carbon disulfide. 3. The test chamber and the enclosure shall be filled with the explosive test mixture. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Sheet 36 of 67 IEC 60079-1: 2001 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| ###### IEC 60079-1 |  |  |  |  |  | # SECTION TWO - CONSTRUCTION 19/25 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ENCL. |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ###### CLAUSE |  |  |  |  |  | REQUIREMENTS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 10.9.3.2 |  |  |  |  |  | Acceptance criteria |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 10.9.3 |  |  |  |  |  | Marking |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 10.9.4 |  |  |  |  |  | Component certificate |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Item: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Acceptance criteria |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Flame transmission: |  |  |  |  |  |  |  |  |  |  |  |  | NO |  |  |  |  |  |  | YES; Test No |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Marking (refer to forms for IEC 60079-0, Marking) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | device is certified in the name of the original component manufacturer |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Note: The original component manufacturer is responsible to ensure that all future component devices are made in accordance with the certified type. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Component certificate (this section confirmed by the certifying authority) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Certificate shows: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | manufacturer's name |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | identifying drawings |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | specifications |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | limiting reference pressure |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | overpressure test details and whether routine testing is required |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | maximum recorded surface temperature obtained during the type test (i.e. thermal test) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | the Group |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Certificate requires: that each component or package of components is accompanied by a copy of the certificate and the manufacturer's declaration stating:- |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | compliance with the certified conditions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | confirmation of material, bubble pore size, density (where applicable) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | special mounting instructions (if any) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Notes/details re certificate contents: (optional) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Sheet 37 of 67 IEC 60079-1: 2001 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| IEC 60079-1 |  |  |  |  |  | # SECTION TWO - CONSTRUCTION 20/25 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | # ENCL . |  |  |  |  |  |
| CLAUSE |  |  |  |  |  | # REQUIREMENTS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 11 |  |  |  |  |  | ###### Fasteners, associated holes and closing devices |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 11.1 |  |  |  |  |  | ###### Type of fastener |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 11.2 |  |  |  |  |  | ###### Plastic materials of light alloys |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 11.3 |  |  |  |  |  | ###### Yield Stress |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 11.4 |  |  |  |  |  | ###### Studs |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 11.5 |  |  |  |  |  | ###### Fasteners |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 11.6 |  |  |  |  |  | ###### Blind holes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 11.7 |  |  |  |  |  | ###### Screws in blind holes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Item: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type of fastener |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Fasteners accessible from the outside and necessary for the assembly of the parts of a flameproof enclosure: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Group I; special fasteners complying with the requirements of IEC 60079-0. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | heads of fasteners not protected, (e.g. counter-bored holes) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | Required: The electrical apparatus shall be marked X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Group II; threads and heads in conformity with 9.2 of IEC 60079-0. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Plastic materials of light alloys (not permitted) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Not incorporated; fastener material is |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Yield stress |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Number of fasteners, |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | yield stress ≤ 240 N/mm2 Required: number of fasteners replaced for type test as per Clause 15 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | yield stress > 240 N/mm2 (Note: Type test carried out with screws/nuts provided by the manufacturer) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| and |  |  |  |  |  |  |  |  |  |  |  | 1.5 x reference pressure ( i.e. 1.5 x |  |  |  |  |  |  |  |  |  |  |  | requires higher yield stress |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| in which case, |  |  |  |  |  |  |  |  |  |  |  | yield stress ( |  |  |  |  |  |  | ) is marked or specified in certificate |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Required: Apparatus to be marked wit X if specified in certificate. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Studs |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | permanently attached; |  |  |  |  |  |  |  |  | welded |  |  |  |  |  |  | riveted |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Fasteners |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | permanently attached; |  |  |  |  |  |  |  |  | welded |  |  |  |  |  |  | riveted |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Blind holes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 3mm ≤ metal thickness surrounding hole ≥ D/3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Screws into blind holes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | screw/stud engages blind hole |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | free space at bottom of hole with fastener tight |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Notes: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Sheet 38 of 67 (IEC 60079-1: 2001) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| ###### IEC 60079-1 |  |  | SECTION TWO  CONSTRUCTION 21/25 |  |  | ENCL. |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ###### CLAUSE |  |  | REQUIREMENTS |  |  |  |  |  |
| 11.8 |  |  | Closing of through holes |  |  |  |  |  |
| 11.9 |  |  | Closure of apertures and compliance of blanking element with requirements of 11.9.1 to 11.9.3 |  |  |  |  |  |
| 11.10 |  |  | Separate fastening arrangements (for threaded doors/covers) |  |  |  |  |  |
| Item: |  |  |  |  |  |  |  |  |
| Closing of through holes |  |  |  |  |  |  |  |  |
|  | enclosure through-hole blinded by screwed plug; details: |  |  |  |  |  |  |  |
| For details of screwed plug used to blind an enclosure through-hole refer to 11.4, Studs and 5.3, Threaded joints |  |  |  |  |  |  |  |  |
| Closure of apertures and compliance of blanking element with requirements of 11.9.1 to 11.9.3 |  |  |  |  |  |  |  |  |
|  | closing device fitted from outside; details: |  |  |  |  |  |  |  |
|  | closing device fitted from inside; details: |  |  |  |  |  |  |  |
|  | closing device is provided with inside locking ring |  |  |  |  |  |  |  |
|  | closing device can only be removed with aid of tool; details: |  |  |  |  |  |  |  |
|  | tool complies with 9.2 of IEC 60079-0 |  |  |  |  |  |  |  |
|  | closing device is constructed so that insertion is by method other than that used for removal; details: |  |  |  |  |  |  |  |
|  |  |  |  |  | removal method complies wth 11.9.1 (fig. 22a) |  |  |  |
|  |  |  |  |  | removal method complies wth 11.9.2 (fig. 22b) |  |  |  |
| Separate fastening arrangements (for threaded doors/covers) Type of tool fastening: |  |  |  |  |  |  |  |  |
|  | per 9.2 of 60079-0 other method |  |  |  |  |  |  |  |
| Details: |  |  |  |  |  |  |  |  |
| Sheet 39 of 67 (IEC 60079-1: 2001) |  |  |  |  |  |  |  |  |

| ###### IEC 60079-1 |  |  |  | SECTION TWO  CONSTRUCTION 22/25 |  |  |  |  |  |  |  |  |  |  | ENCL. |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ###### CLAUSE |  |  |  | REQUIREMENTS |  |  |  |  |  |  |  |  |  |  |  |  |
| 12 |  |  |  | Materials and mechanical strength of enclosures; materials inside the enclosures |  |  |  |  |  |  |  |  |  |  |  |  |
| 12.2 |  |  |  | Assembly of multiple flameproof enclosures |  |  |  |  |  |  |  |  |  |  |  |  |
| 12.3 |  |  |  | Intercommunicating enclosure compartments |  |  |  |  |  |  |  |  |  |  |  |  |
| 12.4 |  |  |  | Use of cast iron |  |  |  |  |  |  |  |  |  |  |  |  |
| 12.5 |  |  |  | Use of liquids |  |  |  |  |  |  |  |  |  |  |  |  |
| Item: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Materials and mechanical strength of enclosures; materials inside the enclosures |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | refer to relevant tests prescribed in clauses 14 through 16 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Assembly of multiple flameproof enclosures Note: The requirements of this standard apply to each separately, with particular attention to common partitions, bushings and operating rods. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | separate interconnected (e.g. conduit) single compartment enclosures: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | multiple compartment enclosures (e.g. integral partitions): |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | other; details: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Intercommunicating enclosure compartments |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | subdivided by the disposition of internal parts |  |  |  |  |  |  |  |  |  |  |  | interconnected compartments |  |  |  |
|  |  |  |  |  |  |  |  |  | means to prevent/reduce pressure piling; details: |  |  |  |  |  |  |  |
| and/or |  |  |  |  |  |  |  |  | mechanical strength of enclosure increased to allow for pressure piling; details: |  |  |  |  |  |  |  |
| Per SECTION THREE  VERIFICATIONS AND TESTS, 3/15, Overpressure test: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | no deformation/damage weakening enclosure or enlarging any joint so as to exceed required gap i |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Note: The above consideration applies to each compartment where two or more flameproof enclosures are incorporated together. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Use of Cast Iron |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Material quality: |  |  |  |  |  |  |  |  |  |  | Required: ≥ 150 (ISO185) |  |  |  |  |  |
| ## Use of Liquids |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | No |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |
| Liquid used (type, |  |  |  |  |  |  |  |  |  |  |  |  |  | ); |  |  |
|  |  |  |  |  |  |  |  |  |  | risk of producing explosive mixture more dangerous than allowed for enclosure |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | ref. tests per clauses 14 through 16 with representative internal mixture |  |  |  |  |  |  |
| Sheet 40 of 67 (IEC 60079-1: 2001) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| ###### IEC 60079-1 |  |  | SECTION TWO  CONSTRUCTION 23/25 |  |  |  |  |  |  |  |  |  |  | ENCL. |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ###### CLAUSE |  |  | REQUIREMENTS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 12.6 |  |  | Insulating materials for Group I apparatus |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 |  |  | Entries for flameproof enclosures |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13.1 |  |  | Cable entries |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13.2 |  |  | Conduit entires |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Item: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ## Insulating materials for Group I apparatus |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ###### Switching apparatus flameproof enclosure with insulating material subjected to electrical stresses resulting from currents > 16A: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | risk of producing arc in air; CTI, |  |  |  |  |  |  |  |  |  | Required: > 400M , according to IEC 60112 |  |  |  |  |  |  |
| Entries for flameproof enclosures |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Connection means defined in manufacturer's documents; ref. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Cable entries Required: Comply with the relevant requirements of Annex C |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | integral |  |  |  | separate, threaded, and, |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | threads/ holes comply with ISO standards; i.e.: |  |  |  |  |  |  |  |  |  |  |  |  |
| or |  |  |  |  | appropriately marked to preclude confusion; i.e.: |  |  |  |  |  |  |  |  |  |  |  |  |
| or |  |  |  |  | separate, other than threaded (Ex component); i.e.: |  |  |  |  |  |  |  |  |  |  |  |  |
| Conduit entries (Note: Only permitted for apparatus of Group II.) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Joint widths and gaps comply with clause 5; ref. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Sealing device (e.g. stopping box): |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Certified ( |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ) |
|  | in flameproof enclosure, i.e.: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | immediately at entrance (method): |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| I.e. Device fixed to enclosure directly or through a coupling accessory |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | refer to (e.g. drawings): |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | refer to forms for IEC 60079-0 ( |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ) |
|  | joints comply with clause 5; ref. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | assembly tested per clause 15.2 (non-transmission of internal ignition); ref. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | setting compound (material and means of specification): |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Note: The setting compound shall be specified in the certificate either of the stopping box or of the complete electrical apparatus having the flameproof enclosure. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Sheet 41 of 67 (IEC 60079-1: 2001) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| ###### IEC 60079-1 |  |  |  |  |  | SECTION TWO  CONSTRUCTION 24/25 |  |  | ENCL. |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ###### CLAUSE |  |  |  |  |  | REQUIREMENTS |  |  |  |  |  |  |  |  |
| 13.3 |  |  |  |  |  | Plugs and sockets and cable couplers |  |  |  |  |  |  |  |  |
| Item: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Plugs and sockets and cable couplers |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | certified ( |  |  |  |  |  |  |  |  |  |  |  |  | ) |
|  | refer to (e.g. drawings): |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | refer to forms for IEC 60079-0 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Flameproof properties of the enclosure where mounted: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | maintained (including when plug and socket are separated); |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | for joint details refer to sheets covering Clause 5 (alternatively, see details below) |  |  |  |  |  |  |  |
| Enclosure of contact parts; volume existing at the moment of separation of contacts, |  |  |  |  |  |  |  |  |  |  | cm3 |  |  |  |
|  |  | for joint details refer to sheets covering Clause 5 (alternatively, see details below) |  |  |  |  |  |  |  |  |  |  |  |  |
| Note: Joint width and gap are determined by the volume existing at the moment of separation of contacts in ignition-capable circuits. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Flameproof properties in the event of internal explosion: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | maintained when connected together; details: |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | maintained at the moment of separation of contacts in ignition-capable circuits; details: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Note: Excluding contacts for earthing /bonding or in circuits complying with IEC 60079-11 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Exemption: (i.e. where the requirements of 13.3.2 and 13.3.3 do not apply) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | connecting assembly fixed together by special fasteners conforming to 11.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | ###### label(s) applied, with the warning. “DO NOT SEPARATE WHEN ENERGIZED” (method): |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Sheet 42 of 67 (IEC 60079-1: 2001) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| ###### IEC 60079-1 |  |  |  | SECTION TWO  CONSTRUCTION 25/25 |  |  |  |  |  |  | ENCL. |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ###### CLAUSE |  |  |  | REQUIREMENTS |  |  |  |  |  |  |  |  |
| 13.4 |  |  |  | Bushings |  |  |  |  |  |  |  |  |
| Item: |  |  |  |  |  |  |  |  |  |  |  |  |
| Bushings |  |  |  |  |  |  |  |  |  |  |  |  |
| Number of conductors, |  |  |  |  |  |  |  |  |  |  |  |  |
|  | certified ( |  |  |  |  |  |  |  |  |  |  | ) |
|  | specific to apparatus flameproof enclosure |  |  |  |  |  |  |  |  |  |  |  |
| Mounting in enclosure walls: |  |  |  |  |  |  |  |  |  |  |  |  |
|  | refer to (e.g. drawings): |  |  |  |  |  |  |  |  |  |  |  |
|  | for joint details refer to sheets covering Clauses 5 and 6 (alternatively, see details below) |  |  |  |  |  |  |  |  |  |  |  |
| Insulation moulded on metallic parts: (Requirements for non-threaded, threaded and gasketed joints do not apply) |  |  |  |  |  |  |  |  |  |  |  |  |
|  | insulation material contributes to the mechanical strength of the enclosure |  |  |  |  |  |  |  |  |  |  |  |
|  | for details refer to sheet covering Clause 6 (alternatively, see details below) |  |  |  |  |  |  |  |  |  |  |  |
| Parts assembled with adhesive: |  |  |  |  |  |  |  |  |  |  |  |  |
|  | for joint details refer to sheet covering Clause 6 (alternatively, see details below) |  |  |  |  |  |  |  |  |  |  |  |
|  | for joint details refer to sheets covering Clause 5 (alternatively, see details below) |  |  |  |  |  |  |  |  |  |  |  |
| Parts of bushing outside flameproof enclosure; details re protection type(s): (i.e. protection per IEC 60079-0) |  |  |  |  |  |  |  |  |  |  |  |  |
| ###### Bushing(s) not specific to a flameproof enclosure; resistance to pressure: |  |  |  |  |  |  |  |  |  |  |  |  |
|  | refer to sheet for clause 15.1.3.1 |  |  |  |  |  |  |  |  |  |  |  |
|  | 20 bar (electrical apparatus for Group I) |  |  |  |  |  |  |  |  |  |  |  |
|  | 30 bar (electrical apparatus for Group II) |  |  |  |  |  |  |  |  |  |  |  |
|  | routine pressure test applied (or |  |  |  |  |  |  | assembly procedure described in manufacturer's documentation; ref. |  |  |  |  |
| Sheet 43 of 67 (IEC 60079-1: 2001) |  |  |  |  |  |  |  |  |  |  |  |  |

| ###### IEC 60079-1 |  |  | SECTION THREE  VERIFICATIONS AND TESTS 1/16 |  |  |  |  |  |  |  |  |  |  |  | ENCL. |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ###### CLAUSE |  |  | REQUIREMENTS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 |  |  | Verification and tests |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 15 |  |  | Type tests |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 15.1 |  |  | Tests of ability of the enclosure to withstand pressure |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 15.1.2 |  |  | Determination of explosion pressure |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Item: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Verification and tests |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | refer to forms for IEC 60079-0 for prerequisite verifications and tests |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | temperature test voltage in accordance with Table 5; i.e. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type tests (with reference to given order) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | conducted in order |  |  |  |  |  |  | other; |  |  |  |  |  |  |  |  |  |  |
| Tests of ability of the enclosure to withstand pressure General; enclosure tested with:- |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | all internal apparatus in place |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | internal apparatus simulated by |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | all or part of the internal apparatus removed; details: |  |  |  |  |  |  |  |  |  |  |  |
|  | Notes: 1. Tests with internal apparatus removed shall consider the possibility of removal according to features of design and shall represent the most severe conditions as determined by the testing laboratory. 2. The reported results of the following tests shall consider enclosure damage, permanent deformation liable to weaken any enclosure parts or permanent enlargement of enclosure joints. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Determination of explosion pressure (refer to 2/16, Reference pressure table) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Pressure measurement by (pressure sensing device/circuit used to measure maximum smoothed pressure): |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Note: Smoothed pressure can be achieved by inserting a 5 kHz  10% filter in the pressure signal circuit. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Ignition source: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | high voltage sparking plug ( |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ) |  |
|  | low-energy ignition source ( |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ) |  |
|  | contained ignition-capable switching device ( |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ) |  |
| Detachable gaskets in place: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | YES |  |  |  |  |  | NO; reason: |  |  |  |  |  |  |  |  |  |  |  |
| Rotating electrical machines: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | running tests at |  |  |  |  |  |  |  | rpm, based on maximum rated speed of |  |  |  |  |  |  | rpm |  |  |
|  | running tests waived (reason): |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | tests at rest waived (reason): |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| continued 2/16 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Sheet 44 of 67 (IEC 60079-1: 2001) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| ###### IEC 60079-1 |  |  |  | SECTION THREE  VERIFICATIONS AND TESTS 2/16 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ENCL. |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ###### CLAUSE |  |  |  | REQUIREMENTS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 15.1.2 |  |  |  | Determination of explosion pressure (continued) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Item: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Determination of explosion pressure (continued) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Rotating electrical machine; running speed: |  |  |  |  |  |  |  |  |  |  |  |  | rpm Required: 90% rated < running < 100% rated |  |  |  |  |  |  |  |  |  |  |  |  |
| Specific gas/gases excluded: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Required: Apparatus to be marked "X" |  |  |  |  |  |  |  |
| Reference pressure |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Test No | Ignition source location Required: Three tests per mixture with each gas (five tests per mixture where pressure piling may occur). |  |  |  |  |  |  | Test mixture |  |  | Rotating machine |  |  |  | Pressure |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | (bar) | Measured at: (Note: Pressure-piling is presumed when pressure values deviate by ≥ 1.5 factor or pressure rise time < 5ms) |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  | (a – g) |  | C |  |  |  |  |  |  |  |  |  | ignition end | opposite end |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | ON |  | OFF |  |  |  |  |  |  | (X) | (X) |  |  |  |
|  |  | (1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | (2) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | (3) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | (4) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Notes (explosive test mixture, concentration in air based on volumetric ratio at atmospheric pressure): (a) Group I; 9.8  0.5 % methane (b) Group IIA; 4.6  0.3 % propane (c) Group IIB; 8.0  0.5 % ethylene (d) Group IIC; 31  1.0 % hydrogen (e) Group IIC; 14.0  1 % acetylene (f) Group IIB; 24  1 % hydrogen-methane (85/15) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Certification only for specifiied gas (g): |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Note: Double marking can be applied in the case of a specified gas if also tested for the next lower group (e.g. IIB + H2) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Damage/deformation: |  |  |  |  |  |  | NO |  |  |  |  |  |  |  | YES; Test No |  |  |  |  |  |  |  |  |  |  |
| Joint enlargement: |  |  |  |  |  |  | NO |  |  |  |  |  |  |  | YES; Test No |  |  |  |  |  |  |  |  |  |  |
| Sheet 45 of 67 (IEC 60079-1: 2001) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| ###### IEC 60079-1 |  |  |  | SECTION THREE  VERIFICATIONS AND TESTS 3/16 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ENCL. |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ###### CLAUSE |  |  |  | REQUIREMENTS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 15.1.3 |  |  |  | Overpressure test |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Overpressure test |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test method: |  |  |  |  |  | static |  |  |  |  | dynamic |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Static test: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Test No | Compartment |  |  |  |  | Damage or deformation |  |  |  |  | Joint enlargement |  |  |  | Subjected to routine testing |  |  |  | Volume (cm3) |  | Test pressure |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | (bar) |  | Duration (s) | Based on reference pressure: (bar) |  |  |
|  |  |  |  |  |  |  | NO |  | YES |  |  | NO |  | YES |  | YES |  | NO |  |  |  |  |  |  |  |  |  |
|  | Required 1. For enclosures of volume  10 cm3, which are not subjected to routine tests, applied test pressure = 4  max. reference pressure; otherwise 3.5 bar  applied test pressure = 1.5  max. reference pressure. Refer to 2/16 for reference test pressures. 3. Duration of test pressure application, 10 (+2 - 0) s. 4. Applied test pressure of 10 bar (Groups I, IIA, IIB) or 15 bar (Group IIC) where reference pressure measurement and dynamic method are not practicable. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Dynamic test: (ref. 1/16 for ignition source) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Test No | Reference pressure test (location No) | Compartment |  |  |  |  |  |  | Damage or deformation |  |  |  |  | Joint enlargement |  |  |  | Explosive mixture |  |  |  |  | Test pressure |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Test mixture per reference pressure test |  |  | Initial pressure (bar) |  | (bar) | Based on reference pressure: (bar) |  |  |
|  |  |  |  |  |  |  |  |  |  | NO |  |  | YES |  | NO |  | YES |  | (a – f) |  | C |  |  |  |  |  |  |
| Required 1. Maximum test pressure = 1.5  reference pressure. 2. Rate of rise of pressure shall closely approximate that obtained during determination of reference pressure. 3. Where reference pressure measurement is not practicable, the test shall be made by explosive mixture at 1.5  atmospheric pressure. - For Group IIC the test shall be made three times with each explosive test mixture. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Sheet 46 of 67 (IEC 60079-1: 2001) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| ###### IEC 60079-1 |  |  |  |  | SECTION THREE  VERIFICATIONS AND TESTS 4/16 |  |  |  |  |  |  |  |  |  |  |  |  | ENCL. |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ###### CLAUSE |  |  |  |  | REQUIREMENTS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 15.2 |  |  |  |  | Test for non-transmission of an internal ignition |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Item: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test for non-transmission of an internal ignition |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | enclosure, ( |  |  |  | gasket(s) removed) placed in test chamber (volume |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ) |  |  |  |
|  | Test No |  | Ignition source (1/16) |  |  |  | Explosive test mixture (Notes 3, 4, 5) in test chamber and inside enclosure | Test conditions relative to Groups |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Flame transmission (NO/YES) |  |  |
|  |  |  |  |  |  |  |  | I, IIA, IIB |  |  | Enlarged flanged gaps of spigot joints where the width of joint L consists only of a cylindrical part (figure 3) |  |  | IIC |  |  | I, IIA, IIB, IIC |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | IIA, IIB only |  | Precompression factor (Note 6) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | Increased joint gap where enclosure is subject to damage (Note 4) |  |  |  |  |  | Gaps of joints other than threaded |  |  | Reduction in length of threaded joint (Table 6) |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | First method | Second method |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Location (or location No for reference pressure test) |  |  |  |  | 1.42 iC (mm) Group IIA | 1.85 iC (mm) Group IIB |  | I, IIA | IIB | IIC | 1.5 iC (mm) min. 0,1 mm for flanged joints | 0,9 iC ≤ iE ≤ iC (mm) at 1,5 x atmosperic pressure | Precompression factor (Note 8) | Based on joint type (i – iii of Note 7) |  |  | none | by ⅓ | by ½ |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | 1 mm | 0,5 mm | 0,3 mm |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | (a – l) |  |  |  | (X) | (X) | (X) |  |  |  |  |  |  | (X) | (X) | (X) |  |  |  |  |
| Notes: 1. iC = the maximum constructional gap, as specified in the manufacturer's drawings; iE = the test gap 2. Unless noted otherwise, test gaps of joints are subject to the condition 0.9 iC ≤ iE ≤ iC 3. Explosive mixture, concentration in air based on volumetric ratio at atmospheric pressure: (a) Group I; 12.5  0.5 % methane-hydrogen (58/42  1 %) (b) Group IIA; 55  0.5 % hydrogen (c) Group IIB; 37  0.5 % hydrogen (d) Group IIC; 27  1.5 % hydrogen (e) Group IIC; 7.5  1 % acetylene 4. Alternative test mixtures for Groups IIA and IIB where damage by test mixture (b) or (c) is possible; ref. Cl. 15.2.1.2: (f) Group IIA; 4.2  0.1 % propane (g) Group IIB; 6.5  0.5 % ethylene 5. Alternative explosive mixtures, where condition of 0.9 iC ≤ iE ≤ iC is not fulfilled: (h) Group I (iE ÷ iC ≥ 0,75); 55  0.5 % H2 (i) Group I (iE ÷ iC ≥ 0.6) and Group IIA (iE ÷ iC ≥ 0.75); 50  0.5 % H2 (j) Group IIA (iE ÷ iC ≥ 0.6); 45  0.5 % H2 (k) Group IIB (iE ÷ iC ≥ 0.75); 28  1 % H2 (l) Group IIB (iE ÷ iC ≥ 0.6); 28  1 % H2 at 1.4 bar 6. Precompression factor , Pk (=iC ÷ iE x 0.9) , for explosive mixtures (a), (b) or (c), where condition of 0.9 iC ≤ iE ≤ iC is not fulfilled. 7. Type of threaded joint considered for reduction in length: i) – cylindrical, complying with ISO 965 fit medium or better; ii) - cylindrical, with larger tolerances than in i); iii) - taper 8. Precompression factor , Pk (=iC ÷ iE x 1.35) , for explosive mixtures (d) or (e), where condition of 0.9 iC ≤ iE ≤ iC is not fulfilled. 9. For Group IIC, in the case of single constructions (i.e. limited production), the apparatus shall be tested five times with unaltered gaps, using the test mixture per Note 3 (d) or (e) at atmospheric pressure. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Sheet 47 of 67 (IEC 60079-1: 2001) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| ###### IEC 60079-1 |  |  |  |  |  | SECTION THREE  VERIFICATIONS AND TESTS 5/16 |  |  |  |  |  |  |  |  | ENCL. |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ###### CLAUSE |  |  |  |  |  | REQUIREMENTS |  |  |  |  |  |  |  |  |  |  |  |
| 15.2.2.2 |  |  |  |  |  | Second Method (Refer to Cl. 15.2) |  |  |  |  |  |  |  |  |  |  |  |
| 15.2.2.3 |  |  |  |  |  | Single construction (Refer to Cl. 15.2) |  |  |  |  |  |  |  |  |  |  |  |
| 15.3 |  |  |  |  |  | Test for Sealing of Stopping Boxes with Setting Compound |  |  |  |  |  |  |  |  |  |  |  |
| 15.4 |  |  |  |  |  | Tests of flameproof enclosure with breathing an draining devices |  |  |  |  |  |  |  |  |  |  |  |
| 15.4.1 |  |  |  |  |  | Tests of ability of the enclosure to withstand pressure |  |  |  |  |  |  |  |  |  |  |  |
| 15.4.1.1 |  |  |  |  |  | Replacement of breathing and draining devices |  |  |  |  |  |  |  |  |  |  |  |
| 15.4.1.2 |  |  |  |  |  | Over Pressure Test |  |  |  |  |  |  |  |  |  |  |  |
| Item: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test for Sealing of Stopping Boxes with Setting Compound |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Filling of stopping box carried out in accordance with instruction of the manufacturer |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | No reduction of pressure observed for 2 minutes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Blotting paper is free of any trace of leakage |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| # Tests of flameproof enclosure with breathing and draining devices |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | the following tests were conducted in the order given on a sample(s) subjected to the preceding impact test |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ###### Devices with non-measurable paths: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | ###### pore size of test sample, |  |  |  |  |  |  | ###### mm ( |  |  | ###### ³ 85 % of the specified maximum pore size) |  |  |  |  |  |  |
| ###### Test of ability of the enclosure to withstand pressure |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Refer to: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | SECTION THREE ¾ VERIFICATIONS AND TESTS, 1/16 and 2/16, Determination of explosion pressure |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | breathing/draining device(s) replaced by solid plug(s) |  |  |  |  |  |  |  |  |  |  |  |  |
| Over Pressure Test |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | SECTION THREE ¾ VERIFICATIONS AND TESTS, 3/16, Overpressure test |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | thin flexible membrane fitted to inner surfaces of device(s) |  |  |  |  |  |  |  |  |  |  |  |  |
| Observed permanent damage or deformation of breathing/draining device(s): |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | NO |  |  |  | YES (details) |  |  |  |  |  |  |  |  |  |  |  |  |
| type of protection affected: |  |  |  |  |  |  |  |  |  |  |  | NO |  | YES |  |  |  |
| Sheet 48 of 67 (IEC 60079-1: 2001) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| ###### IEC 60079-1 |  |  | SECTION THREE  VERIFICATIONS AND TESTS 6/16 |  |  |  |  | ENCL. |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ###### CLAUSE |  |  | REQUIREMENTS |  |  |  |  |  |  |  |  |  |  |  |  |
| 15.4.2 |  |  | Thermal Test |  |  |  |  |  |  |  |  |  |  |  |  |
| Item: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Thermal tests |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Ignition source: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | high voltage sparking plug ( |  |  |  |  |  |  |  |  |  |  |  |  | ) |  |
|  | low energy ignition source ( |  |  |  |  |  |  |  |  |  |  |  |  | ) |  |
|  | contained ignition-capable switching device ( |  |  |  |  |  |  |  |  |  |  |  |  | ) |  |
| Test set-up: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | sample device fitted to enclosure and placed in test chamber (volume |  |  |  |  |  |  |  |  | ) |  |  |  |  |  |
|  | same test mixture throughout (volume ratio, test chamber/enclosure to which the device is fitted: |  |  |  |  |  |  |  |  |  | / |  |  |  |  |
| Sheet 49 of 67 (IEC 60079-1: 2001) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| ###### IEC 60079-1 |  |  |  |  |  |  | SECTION THREE  VERIFICATIONS AND TESTS 7/16 |  |  |  |  |  |  |  |  |  |  |  | ENCL. |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ###### CLAUSE |  |  |  |  |  |  | REQUIREMENTS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 15.4.2.1 |  |  |  |  |  |  | Test Procedure |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 15.4.2.2 |  |  |  |  |  |  | Acceptance Criterion |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Item: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test Procedure |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Devices for Groups I, IIA, IIB, IIC: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Test No |  | Ignition source location |  |  |  |  |  |  | Test mixture (a or b) |  | test gas flow through the device and the enclosure |  |  | ventilation or sampling system operated per mfr.'s documentation |  |  |  | External surface temperature of device (°C) |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | (X) |  | (X) |  |  | (X) |  |  |  |  |  |  |  |  |
|  |  | Notes 1. For devices for hydrogen only, tests with mixtures including acetylene are not required. 2. The surface temperature of the device should be at test ambient at the start of each test. 3. Explosive mixture, concentration in air based on volumetric ratio at atmospheric pressure: (a) All Groups: 4.2 ± 0.1 % C3H8, (b) For use in acetylene: 7.5 ± 1 % C2H2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Test mixture maintained (external to device) after each test for at least 10 min |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Acceptance Criterion |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Continuous burning: |  |  |  |  |  |  |  |  |  | NO |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | YES; Test No |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Flame transmission: |  |  |  |  |  |  |  |  |  | NO |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | YES; Test No |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Maximum external surface temperature of the device: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  | °C at ambient, |  |  |  |  |  | °C |  |  |  |  |  |  |  |  |  |  |
| (surface temperatures measured by |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ) |
| (maximum external surface temperature rise of the device, |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | K) ´ 1.2 = |  |  |  | K |  |  |  |
| Note: The measured external surface temperature rise of the device shall be multiplied by a safety factor of 1.2 for the determination of the temperature class of the electrical apparatus |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Sheet 50 of 67 (IEC 60079-1: 2001) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| ###### IEC 60079-1 |  |  |  |  | SECTION THREE  VERIFICATIONS AND TESTS 8/16 |  |  |  |  |  |  |  |  |  |  | ENCL. |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ###### CLAUSE |  |  |  |  | REQUIREMENTS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 15.4.3 |  |  |  |  | Test procedure/Non-transmission test for breathing and draining devices/Acceptance criterion |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Item: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test procedure/ Non-transmission test for breathing and draining devices |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | enclosure, ( |  |  |  |  | gasket(s) removed) placed in test chamber (volume |  |  |  |  |  |  |  |  |  |  |  | ) |  |  |  |  |
| Devices for Groups I, IIA, IIB: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Test No |  | Ignition source (1/16) |  |  |  |  | Explosive mixture (Notes 3, 4, 5) in test chamber and inside enclosure | Test conditions relative to Groups |  |  |  |  |  |  |  |  |  |  |  | Flame transmission (NO/YES) |  |  |
|  |  |  |  |  |  |  |  |  | Test gap |  |  |  | Enlarged flanged gaps of spigot joints where the width of joint L consists only of a cylindrical part (figure 3) |  | Reduction in length of threaded joint (Table 6) |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  | Normal condition (Note 2) | IIA, IIB only |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | (Increased joint gap where enclosure is subject to damage Note 4) |  | Precompression factor (Note 6) |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Location (close to the inner surface of the device and then (if necessary) in one or more places producing the highest peak explosion pressure and rate of rise or location No for reference pressure test) |  |  |  |  |  |  | 1.42 iC Group IIA | 1.85 Group IIB |  | I, IIA 1 mm | IIB 0.5 mm | Based on joint type (i – iii of Note 7) |  | none | by ⅓ |  | by ½ |  |  |  |
|  |  |  |  |  |  |  |  | (a – k) | (X) | (X) | (X) |  | (X) | (X) |  |  | (X) | (X) |  | (X) |  |  |  |
| Notes: 1. iC = the maximum constructional gap, as specified in the manufacturer's drawings; iE = the test gap 2. Test gaps of joints are subject to the condition 0.9 iC ≤ iE ≤ iC 3. Explosive mixture, concentration in air based on volumetric ratio at atmospheric pressure: (a) Group I; 12.5  0.5 % methane-hydrogen (58/42  1 %) (b) Group IIA; 55  0.5 % hydrogen (c) Group IIB; 37  0.5 % hydrogen 4. Alternative test mixtures for Groups IIA and IIB where damage by test mixture (b) or (c) is possible; ref. Cl. 15.2.1.2: (d) Group IIA; 4.2  0.1 % propane (e) Group IIB; 6.5  0.5 % ethylene 5. Alternative explosive mixtures, where condition of 0,9 iC ≤ iE ≤ iC is not fulfilled: (f) Group I (iE ÷ iC ≥ 0.75); 55  0.5 % H2 (g) Group I (iE ÷ iC ≥ 0.6) 50  0.5 % H2 (h) Group IIA (iE ÷ iC ≥ 0.75); 50  0.5 % H2 (i) Group IIA (iE ÷ iC ≥ 0.6); 45  0.5 % H2 (j) Group IIB (iE ÷ iC ≥ 0.75); 28  1 % H2 (k) Group IIB (iE ÷ iC ≥ 0.6); 28  1 % H2 at 1.4 bar 6. Precompression factor , Pk (=iC ÷ iE x 0,9) , for explosive mixtures (a) or (b), where condition of 0,9 iC ≤ iE ≤ iC is not fulfilled. 7. Type of threaded joint considered for reduction in length: i) – cylindrical, complying with ISO 965 fit medium or better; ii) - cylindrical, with larger tolerances than in i); iii) - taper continued on 9/16 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Sheet 51 of 67 (IEC 60079-1: 2001) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| ###### IEC 60079-1 |  |  |  |  | SECTION THREE  VERIFICATIONS AND TESTS 9/16 |  |  |  |  |  |  |  | ENCL. |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ###### CLAUSE |  |  |  |  | REQUIREMENTS |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 15.4.3 |  |  |  |  | Test procedure/Non-transmission test for breathing and draining devices/Acceptance criterion (continued) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Item: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test procedure/Non-transmission test for breathing and draining devices (continued) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Devices for Group IIC: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Test No |  | Ignition source (1/16) |  |  | Measurable paths |  |  |  | Non-measurable paths |  |  |  | Reduction of length of thread engagement ("⅓"; ISO fit) or ("½"; worse than ISO fit) or ("⅓"; taper) | Flame transmission (NO/YES) |  |  |  |
|  |  |  |  |  |  | Explosive mixture (a or b) | Gaps of joints other than threaded |  |  | Method A | Method B (Note 8) |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | First method | Second method |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | 1.5 iC (mm) min. 0,1 mm for flanged joints | 0.9 iC ≤ iE ≤ iC (mm) at 1.5 x atmosperic pressure | Precompression factor (Note 9) | Explosive mixture (c or d) | Explosive mixture (e or f) |  |  |  |  |  |  |  |
|  |  |  | Location (close to the inner surface of the device and then (if necessary) in one or more places producing the highest peak explosion pressure and rate of rise or location No for reference pressure test) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Notes 1. iC = the maximum constructional gap, as specified in the manufacturer's drawings; iE = the test gap 2. Unless noted otherwise, test gaps of joints are subject to the condition 0,9 iC ≤ iE ≤ iC 3. Explosive mixture at atmospheric pressure: (a) 27.5  1.5 % H2 in air (b) 7.5  1 % C2H2 in air 4. Explosive mixture, Method A, 1.5 bar precompression: (c) 27  1.0 % H2 in air (1.5 bar precompression of test mixture) (d) 7.5  1 % C2H2 in air (1.5 bar precompression of test mixture) 5. Explosive mixture, Method B at atmospheric pressure:: (e) (40  1 %) H2, (20  1 %) O2, bal. N2 (f) (10  1 %) C2H2, (24  1 %) O2, bal. N2 6. For Group IIC, in the case of single constructions (i.e. limited production), the apparatus shall be tested five times with unaltered gaps, using the test mixture per Note 3 (d) or (e) at atmospheric pressure. 7. For devices for hydrogen only, tests with mixtures including acetylene are not required. 8. In the case of Method B the apparatus shall be marked with sign X indicating exclusion of ethyl nitrate and where the device is for use with enclosure volume > 100 cm3, indicating exclusion of carbon disulfide. 9. Precompression factor , Pk (=iC ÷ iE x 1.35) , for explosive mixtures (a) or (b), where condition of 0.9 iC ≤ iE ≤ iC is not fulfilled. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Sheet 52 of 67 IEC 60079-1: 2001 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| ###### IEC 60079-1 |  |  | SECTION THREE  VERIFICATIONS AND TESTS 10/16 |  |  |  |  | ENCL. |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ###### CLAUSE |  |  | REQUIREMENTS |  |  |  |  |  |  |  |
| 16.1 |  |  | Routine tests |  |  |  |  |  |  |  |
| Item: |  |  |  |  |  |  |  |  |  |  |
| Routine tests |  |  |  |  |  |  |  |  |  |  |
| Method/conditions of routine tests subject to agreement between the testing laboratory and the manufacturer: |  |  |  |  |  |  |  |  |  |  |
|  | Notes: 1. Empty enclosures may be tested. 2. Each of the parts forming the enclosure may be tested separately with similar stressing compared to the complete enclosure. 3. Consideration shall be given to the influence by the enclosed apparatus on rise of pressure during an internal explosion if the routine test is dynamic. 4. Observations shall be made as to structural damage or permanent deformation of the enclosure that could affect its flameproof properties as a result of the tests. 5. In the case of welded enclosures, observations shall also be made as to the presence of holes or cracks reducing the flameproof properties of the enclosure or any part of it. |  |  |  |  |  |  |  |  |  |
|  | static test at relative pressure (reference pressure measurement and dynamic method not practicable):- |  |  |  |  |  |  |  |  |  |
| Volume < 10 cm3 |  |  |  |  |  |  | 10 bar (Group I, IIA, IIB, IIC) |  |  |  |
| Volume > 10 cm3 |  |  |  |  |  |  | 10 bar (Group I) |  |  |  |
| Volume > 10 cm3 |  |  |  |  |  |  | 15 bar (Group IIA or IIB) |  |  |  |
| Volume > 10 cm3 |  |  |  |  |  |  | 20 bar (Group IIC) |  |  |  |
|  | based on dynamic type test; method:- |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | explosion test using mixture for explosion pressure determination per 15.1.2, inside and outside enclosure, at 1.5  atmospheric pressure or |  |  |  |  |
|  |  |  |  |  |  | dynamic pressure test per 15.1.3.2 at atmospheric pressure followed by tests for non-transmission of an internal ignition per 15.2.1.2 or 15.2.2.1, or |  |  |  |  |
|  |  |  |  |  |  | dynamic overpressure test per 15.1.3.2 followed by static test at a pressure of at least 2 bar |  |  |  |  |
| Sheet 53 of 67 (IEC 60079-1: 2001) |  |  |  |  |  |  |  |  |  |  |

| ###### IEC 60079-1 |  |  |  | SECTION THREE  VERIFICATIONS AND TESTS 11/16 |  |  |  |  |  |  |  |  | ENCL. |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ###### CLAUSE |  |  |  | REQUIREMENTS |  |  |  |  |  |  |  |  |  |  |
| 16.2 |  |  |  | Routine tests – where not required |  |  |  |  |  |  |  |  |  |  |
| 16.3 |  |  |  | Routine tests - acceptance criterion |  |  |  |  |  |  |  |  |  |  |
| Item: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Routine tests – where not required |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Not required because:- |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | enclosure does not involve welded construction |  |  |  |  |  |  |  |  |
| and |  |  |  |  |  | volume  10 cm3 ( |  |  |  |  |  | cm3) |  |  |
| or |  |  |  |  |  | volume > 10 cm3 and |  |  |  |  | static pressure test at 4  reference pressure |  |  |  |
| Routine tests – acceptance criterion |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Routine tests are considered satisfactory if |  |  |  |  |  | the enclosure withstands the pressure without suffering permanent deformation of the joints, or, damage to the enclosure |  |  |  |  |  |  |
|  | & when dynamic followed by static |  |  |  |  |  |  | no leakage through walls of enclosure |  |  |  |  |  |  |
|  | or if dynamic |  |  |  |  |  |  | there is no transmission of an internal ignition |  |  |  |  |  |  |
| Sheet 54 of 67 (IEC 60079-1: 2001) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| ###### IEC 60079-1 |  |  |  | SECTION THREE  VERIFICATIONS AND TESTS 12/16 |  |  |  | ENCL. |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ###### CLAUSE |  |  |  | REQUIREMENTS |  |  |  |  |  |  |
| 17 |  |  |  | Switchgear for Group I |  |  |  |  |  |  |
| 17.1 |  |  |  | Means of Isolation |  |  |  |  |  |  |
| 17.2 |  |  |  | Doors or Covers |  |  |  |  |  |  |
| Item: |  |  |  |  |  |  |  |  |  |  |
| Switchgear for Group I |  |  |  |  |  |  |  |  |  |  |
| Apparatus for Group I, that is to be open from time to time shall comply with the requirements in Cl. 17. |  |  |  |  |  |  |  |  |  |  |
| Means of Isolation |  |  |  |  |  |  |  |  |  |  |
| Fitted inside Ex d enclosure |  |  |  |  |  |  |  |  |  |  |
| Parts fitted inside the flameproof enclosure which remain energized after the means of isolation has been opened comply with the following. |  |  |  |  |  |  |  |  |  |  |
|  |  | Protection means listed in IEC60079-0: |  |  | IEC 60079 - |  |  |  |  |  |
|  |  | have clearances and creepage distances between phases and to earth in accordance with the requirements of IEC 60079-7, and |  |  |  |  |  |  |  |  |
|  |  | are protected by an enclosure that provides a degree of protection of at least IP20 according to IEC 600529 |  |  |  |  |  |  |  |  |
| Details: |  |  |  |  |  |  |  |  |  |  |
| Note: This does not apply to parts of intrinsically safe circuits complying with IEC 60079-11 which remain energized. |  |  |  |  |  |  |  |  |  |  |
|  |  | Warning label “DO NOT OPEN WHEN ENERGIZED” |  |  |  |  |  |  |  |  |
| Fitted inside another enclosure |  |  |  |  |  |  |  |  |  |  |
|  |  | Fitted inside another enclosure complying with one of the standard types of protection listed in IEC 60079-0: |  |  |  |  |  |  |  |  |
| Plug and socket |  |  |  |  |  |  |  |  |  |  |
|  |  | plug and socket, or a cable coupler complying with the requirements in 13.3: |  |  |  |  |  |  |  |  |
| Doors or covers |  |  |  |  |  |  |  |  |  |  |
| Quick acting doors or covers |  |  |  |  |  |  |  |  |  |  |
|  |  | Mechanical interlock with an isolator: enclosure retains Ex d when isolated is closed and isolator can only be closed when doors and covers ensure enclosure is Ex d; details: |  |  |  |  |  |  |  |  |
| Doors or covers fixed by screws |  |  |  |  |  |  |  |  |  |  |
|  |  | Doors bear a label “DO NOT OPEN WHEN ENERGIZED” |  |  |  |  |  |  |  |  |
| Threaded doors or covers |  |  |  |  |  |  |  |  |  |  |
|  |  | Doors or covers have a label “DO NOT OPEN WHEN ENERGIZED” |  |  |  |  |  |  |  |  |
| Sheet 55 of 67 (IEC 60079-1: 2001) |  |  |  |  |  |  |  |  |  |  |

| ###### IEC 60079-1 |  |  |  | SECTION THREE  VERIFICATIONS AND TESTS 13/16 |  |  |  |  | ENCL. |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ###### CLAUSE |  |  |  | REQUIREMENTS |  |  |  |  |  |  |  |  |  |
| 18 |  |  |  | ###### Lampholders and lampcaps |  |  |  |  |  |  |  |  |  |
| 18.1 |  |  |  | Device Preventing lamps working loose |  |  |  |  |  |  |  |  |  |
| 18.2 |  |  |  | Holders and caps for lamps with cylindrical caps |  |  |  |  |  |  |  |  |  |
| 18.3 |  |  |  | Holders for lamps with threaded caps |  |  |  |  |  |  |  |  |  |
| Item: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Lampholders and lampcaps forming flameproof enclosure (for use in increased safety luminaires) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Parts/details: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Device Preventing lamps working loose |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Complies with Annex A of IEC 60079-7, or |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Device is threaded and provided with a quick disconnect in a Ex d enclosure and breaks all poles before contact separation; refer to: |  |  |  |  |  |  |  |  |  |  |  |  |
| Holders and caps for lamps with cylindrical caps |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Tubular fluorescent lamps |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Complies with dimensional requirements of data sheets Fa6 of IEC 60061 |  |  |  |  |  |  |  |  |  |  |  |  |
| Other holders |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Refer to Clause 5 for details |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Joint between holder and cap > 10mm |  |  |  |  | mm at moment of contact separation |  |  |  |  |  |  |  |
| Holders for lamps with threaded caps |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Resistance to Corrosion |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Threaded part of the holder is resistant to corrosion in service; material/corrosion protection: |  |  |  |  |  |  |  |  |  |  |  |  |
| Contact Separation |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Number of threads engaged at moment of contact separation: |  |  |  |  |  |  |  | Required: > 2 threads |  |  |  |  |  |
| ## E26/E27 and E39/E40 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Electrical contact established by spring-loaded contact elements: |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Threaded electrical contact for E26/E27 for Group IIA |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Threaded electrical contact for E39/E40 for Group IIA |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Threaded electrical contact for E26/E27 for Groups IIB, IIC, make/break in flameproof enclosure: |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Threaded electrical contact for E39/E40 for Groups IIB, IIC, make/break in flameproof enclosure: |  |  |  |  |  |  |  |  |  |  |  |  |
| Note:The above requirements are not necessary for threaded lampholders E10 and E14 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Sheet 56 of 67 (IEC 60079-1: 2001) |  |  |  |  |  |  |  |  |  |  |  |  |  |

| ###### IEC 60079-1 |  |  |  |  |  | SECTION THREE  VERIFICATIONS AND TESTS 14/16 |  |  |  |  |  |  |  |  |  | ENCL. |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ###### CLAUSE |  |  |  |  |  | REQUIREMENTS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 19.1 |  |  |  |  |  | Permitted non-metallic enclosures |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 19.2 |  |  |  |  |  | Special construction requirements |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 19.2.1 |  |  |  |  |  | Resistance to tracking and creepage distance on internal surfaces of the enclosure walls |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 19.3 |  |  |  |  |  | ###### Supplementary requirements for type tests |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 19.3.1 |  |  |  |  |  | ###### Test for flameproofness |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 19.3.1.1 |  |  |  |  |  | ###### Test procedure |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 19.3.1.2 |  |  |  |  |  | ###### Test of ability of the enclosure to withstand pressure |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Item: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Permitted non-metallic enclosures |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Non-metallic enclosure; details: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Non-metallic enclosure part(s); details: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | volume  3000 cm3 |  |  |  |  |  |  | volume  3000 cm3 and |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  | surface area of (each) non-metallic part  500 cm2, |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  | surface area of light-transmitting part (luminaire)  8000 cm2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ###### Special constructional requirements |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ###### Resistance to tracking and creepage distance on internal surfaces of the enclosure walls |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Grade of insulating material |  |  |  |  |  |  | Between (conductive parts): |  |  | Working voltage U | Distance (mm) |  |  |  | grooves |  | CTI |  |  |
|  |  |  | Component |  |  |  |  |  |  |  |  |  |  | Measured | Required |  |  |  | ribs |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | (X) | (X) |  |  |  |
| Required (i) CTI and creepage distances on internal surfaces shall comply with IEC 60079-7 (ref. IEC 60079-7, Cl. 4.4 and Tables 2, 3). (ii) Surface ribs  3 mm high,  1 mm thick, or grooves  3mm deep,  3mm wide if taken into account for reduced creepage. (iii) Group I; CTI  400 M (ref. IEC 60112) for insulating materials subject to arc-capable electrical stresses due to rated current  16 A. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Supplementary requirements for type tests |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Tests for flameproofness/ Test procedure |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | The following tests were conducted in the order given on enclosures subjected to tests per 23.4.7 of IEC 60079-0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Test of erosion by flame for enclosures or parts of enclosures of plastic materials, refer 16/16 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test of ability of the enclosure to withstand pressure |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | refer to TESTS OF ABILITY OF THE ENCLOSURE TO WITHSTAND PRESSURE (Pages |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ) |  |
|  |  | ###### refer to: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Sheet 57 of 67 (IEC 60079-1: 2001) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| ###### IEC 60079-1 |  |  |  |  |  | SECTION THREE  VERIFICATIONS AND TESTS 15/16 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ENCL. |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ###### CLAUSE |  |  |  |  |  | REQUIREMENTS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 19.3.1.3 |  |  |  |  |  | ###### Test of erosion by flame |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ###### 19.3.1.4 |  |  |  |  |  | ###### Test for non-transmission of an internal ignition |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ###### 19.3.2 |  |  |  |  |  | ###### Flammability |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Item: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test of erosion by flame (to be considered where enclosure volume is >100cm3 and flameproof joints have at least one face of plastics material or for enclosures or parts of enclosures of plastic material) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | ###### not applicable (i.e. enclosure volume  100 cm3 and flameproof joints do not involve plastic material or no plastic parts) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | static gap (flanged joint/plane part of spigot joint) set to |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | mm Required: 0.1mm  value  0.15mm |  |  |  |  |  |  |  |  |  |  |  |  |
| or |  |  |  |  |  |  |  |  | gap set to maximum permitted value ( 0.15 mm); |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | mm |  |  |  |  |  |  |
|  |  | explosive test mixture; |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | in air (test ambient, |  |  |  |  |  |  |  |  |  | C) |  |  |  |
|  |  |  |  |  | ignitions by (ignition source) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (ignition source location, |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ) |  |
| Required: Group I, IIA, IIB - 50 ignitions; Group IIC – 25 ignitions for each explosive mixture per 15.1.2.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ###### Test for non-transmission of an internal ignition |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | refer to TEST FOR NON-TRANSMISSION OF AN INTERNAL IGNITION (Page |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ) |  |  |  |  |  |
| Flammability |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ## Test method/First alternative test method |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Component |  |  |  |  |  |  |  |  |  | Test Specimen |  |  |  |  |  |  |  | Test method |  |  |  | Worst case results after flame application to each specimen |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ISO 1210 |  |  | First Alternative method |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | Number of pieces |  |  | Number of sets | Cut from enclosure | Moulded | Cut from plates |  | ISO 1210 |  | First alternative | Second alternative | Burning ≤ 15 s | Specimens burnt completely |  | Flaiming < 10 s | Flaiming < 50s total for 10 applications |  | Specimen did not burn up to holding clamp | Cotton layer did not ignite | Glowing < 30 s |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | (x) | (x) | (x) |  | (x) |  | (x) | (x) | (x) | (x) |  | (x) | (x) |  | (x) | (x) | (x) |  |  |
| Notes (i) Test pieces cut from enclosure or moulded as individual pieces or cut from plates prepared for this test produced under the same condition as the enclosure. The conditions shall be recorded in the manufacturer's documentation. (ii) One of the alternative test methods shall be applied if the test per ISO 1210 results in distortion of the test piece out of the flame. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Sheet 58 of 67 (IEC 60079-1: 2001) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| ###### IEC 60079-1 |  |  |  | SECTION THREE  VERIFICATIONS AND TESTS 16/16 |  |  |  |  |  |  |  |  |  |  | ENCL. |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ###### CLAUSE |  |  |  | REQUIREMENTS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 19.3.2.2.2 |  |  |  | ###### Second alternative test method |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 19.3.2.2.3 |  |  |  | Flame erosion test for enclosures or parts of enclosures of plastic material |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 19.4 |  |  |  | Test report |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Item: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Second alternative test method |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Component |  |  |  | Test Specimens |  |  |  |  |  | Test Method |  | Results/Classification (ref. IEC 60695-11-10, Table 1) |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Number of pieces | Number of sets | Cut from enclosure | Moulded | Cut from plates | Specimen size | Conditioned per Cl. 4, IEC 60707 and Cl. 9.1, IEC 60695-11-10 | B – Vertical burning test per Cl. 9.2, IEC 60695-11-10 | Individual test specimen afterflame time (s) | Total set afterflame time (s) |  | Individual test specimen afterflame time plus afterglow time after 2nd application (s) | Afterflame and/or afterglow progressed up to holding clamp (NO/YES) | Cotton indicator pad ignited by flaming particles or drops (NO/YES) | Category (V-0, V-1, V-2) OR *(n/c) *Material cannot be categorized by test method |  |  |
|  |  |  |  |  |  |  | (x) | (x) | (x) |  | (x) | (x) |  |  |  |  |  |  |  |  |  |
| Notes (i) Test pieces moulded as individual pieces or plates from which the test pieces are cut shall be produced under conditions equivalent to those used to produce the actual enclosure and the conditions shall be recorded in the manufacturer's documentation. (ii) The second alternative test method shall be in accordance with IEC 60707 (Method FV: Flame – Vertical specimen) per the requirements and criteria for classification of IEC 60695-11-10. (iii) Per IEC 60695-10-11, for material that cannot be categorized *(n/c) , the horizontal burning test method can be used. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Flame erosion test for enclosures or parts of enclosures of plastic material carried out before tests of ability of enclosure to withstand pressure (clause 19.3.1.2) and Test for non-transmission of an internal ignition (clause l9.3.1.4) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Flame erosion test carried out after the tests of ability of the enclosure to withstand pressure |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ## Test report |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| For complete reference of the electrical apparatus, non-metallic material used , results obtained in each test specified, and description of tests not made according to specified requirements (inluding reasons for deviations), refer to: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Sheet 59 of 67 (IEC 60079-1: 2001) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| ###### IEC 60079-1 |  |  |  | ANNEX A  ADDITIONAL REQUIREMENTS FOR CRIMPED RIBBON ELEMENTS OF BREATHING AND DRAINING DEVICES |  |  |  |  |  | ENCL. |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ###### CLAUSE |  |  |  | REQUIREMENTS |  |  |  |  |  |  |  |
| ###### A.1 |  |  |  | ###### Crimped Ribbon |  |  |  |  |  |  |  |
| A.2 |  |  |  | ###### Path dimensions |  |  |  |  |  |  |  |
| A.3 |  |  |  | ###### Annex B requirements |  |  |  |  |  |  |  |
| A.4 |  |  |  | ###### Type Tests |  |  |  |  |  |  |  |
| Item: |  |  |  |  |  |  |  |  |  |  |  |
| # Crimped ribbon |  |  |  |  |  |  |  |  |  |  |  |
| ###### Material: |  |  |  |  |  |  |  |  |  |  |  |
|  |  | cupro-nickel |  |  |  | stainless steel |  |  |  |  |  |
|  |  | Note: Aluminium, titanium, magnesium and their alloys shall not be used. |  |  |  |  |  |  |  |  |  |
| Path Dimensions |  |  |  |  |  |  |  |  |  |  |  |
|  | Measurable paths specified in drawings: |  |  |  |  |  |  |  |  |  |  |
| Does A.2 apply? |  |  |  |  |  |  |  |  |  |  |  |
|  |  | NO (refer to Annex B, Elements with non-measurable paths) |  |  |  |  |  |  |  |  |  |
|  |  | YES; details/reference specification defining upper and lower tolerance limits: |  |  |  |  |  |  |  |  |  |
|  |  | tolerance limits are monitored in production; method: |  |  |  |  |  |  |  |  |  |
| Type Tests (refer to 15.4.3, Test for non-transmission of an internal ignition): |  |  |  |  |  |  |  |  |  |  |  |
|  |  | conducted on samples manufactured with the largest permitted gap dimensions; |  |  |  |  |  |  |  |  |  |
|  |  | sample details: |  |  |  |  |  |  |  |  |  |
| Sheet 60 of 67 (IEC 60079-1: 2001) |  |  |  |  |  |  |  |  |  |  |  |

| ###### IEC 60079-1 |  |  | ANNEX B  ADDITIONAL REQUIREMENTS FOR ELEMENTS, WITH NON-MEASURABLE PATHS, OF BREATHING AND DRAINING DEVICES |  |  |  |  |  |  |  |  |  |  |  |  |  | ENCL. |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ###### CLAUSE |  |  | REQUIREMENTS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| B.1 |  |  | ###### Sintered metal elements |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Item: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| # Sintered metal elements |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Notes: 1. Elements are classified according to their density as well as their pore size. 2. For functional reasons, it may also be necessary to state the fluid permeability and the porosity. 3. The appropriate specifications shall be in accordance with a recognized standard method (i.e. as indicated below) for the particular material and manufacturing method. 4. Reference documentation shall clearly identify the metal element in accordance with specifications given below. 5. Test sample pore size shall be not less than 85 % of the specified maximum pore size and the Mechanical strength test and Tests of flameproof enclosure with breathing and draining devices shall be applied to the same sample. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ###### Additional requirements for elements with non-measurable paths, of breathing and draining devices: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ###### Construction |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | stainless steel |  |  |  |  | 90/10 copper-tin bronze |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | minimum thickness, |  |  |  |  |  |  |  |  |  |  | mm |  |  |  |  |  |  |  |  |
| - Bubble test pore size |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | maximum pore size (ISO 4003), |  |  |  |  |  |  |  |  |  |  |  |  | m |  |  |  |  |  |  |
| - Density of sintered metal element |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | density (ISO 2738), |  |  |  |  |  |  |  |  | g/cm3 |  |  |  |  |  |  |  |  |  |  |
| Determination of open porosity |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | functional aspects (ISO 4022 and ISO 2738):- |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  | open porosity, |  |  |  |  |  |  |  |  | % |  |  |
|  |  |  |  |  |  |  |  |  | fluid permeability, |  |  |  |  |  |  |  |  | m2 |  |  |
| ###### Identification |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ###### Specifications identified in documentation (refer to |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ): |  |
| Sheet 61 of 67 (IEC 60079-1: 2001) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| ###### IEC 60079-1 |  |  |  | ANNEX B  ADDITIONAL REQUIREMENTS FOR ELEMENTS, WITH NON-MEASURABLE PATHS, OF BREATHING AND DRAINING DEVICES |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ENCL. |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ###### CLAUSE |  |  |  | REQUIREMENTS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| B.2 |  |  |  | ###### Pressed Metal Wire Elements |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| B.3 |  |  |  | ###### Metal foam elements |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Item: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Pressed metal wire elements |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ###### Construction |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ###### Density and mass |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | stainless steel wire |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | density |  |  |  |  |  | g/cm3 (based on the information given below) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (wire diameter |  |  |  |  |  |  | cm; mass |  |  |  |  |  |  |  |  |  |  |  |  |  | g; length of wire braid, |  |  |  |  |  |  | cm |  |  |
| thickness of element |  |  |  |  |  |  |  |  |  |  |  |  |  | cm; mesh size |  |  |  |  |  |  |  |  |  | m |  |  |  |  |  |  |
| ratio between mass of element and identical volume of same solid metal, |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Required: 0.4  ratio  0.6 |  |  |  |  |  |
| Bubble test pore size |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | maximum pore size (ISO 4003), |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | m |  |  |  |  |  |  |  |  |  |  |
| Density of element determined in accordance with ISO 2738 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | density (ISO 2738), |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | g/cm3 |  |  |  |  |  |  |  |  |  |  |
| Determination of open porosity and fluid permeability |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | dimensions (incl. tolerances), |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | original wire diameter, |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | mm |  |  |  |  |  |  |  |  |  |  |  |  |
|  | functional aspects (ISO 4022 and ISO 2738):- |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  | open porosity, |  |  |  |  |  |  |  |  |  |  |  |  |  | % |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  | fluid permeability, |  |  |  |  |  |  |  |  |  |  |  |  |  | m2 |  |  |  |
| Note: The element shall be rigid (i.e. wire braid, compressed into a dye to form a homogeneous matrix). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Identification of metal wire elements |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Specifications identified in documentation (refer to |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ) |
| Metal foam elements - Construction |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Thermal decomposition i.e. by gaseous diffusion, and compressing the material: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Contain min 15% chromium by mass |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | nickel-chrome alloy ( |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | % chromium by mass) Required: at least 15% chromium |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Sheet 62 of 67 (IEC 60079-1: 2001) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| ###### IEC 60079-1 |  |  | ANNEX B  ADDITIONAL REQUIREMENTS FOR ELEMENTS, WITH NON-MEASURABLE PATHS, OF BREATHING AND DRAINING DEVICES |  |  |  |  |  |  |  | ENCL. |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ###### CLAUSE |  |  | REQUIREMENTS |  |  |  |  |  |  |  |  |  |  |  |  |  |
| B.3 |  |  | Metal foam elements (cont’d) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Item: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Bubble test pore size |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | equivalent pore size as determined by (ISO 4003), |  |  |  |  |  |  |  | m |  |  |  |  |  |  |  |
| minimum thickness, |  |  |  |  |  |  |  |  | mm |  |  |  |  |  |  |  |
| Density |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Density (ISO 2738), |  |  |  |  | g/cm3 |  |  |  |  |  |  |  |  |  |  |
| Open porosity and/or fluid permeability |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | functional aspects (ISO 4022 and ISO 2738):- |  |  |  |  |  | open porosity, |  |  |  |  | % |  |  |  |  |
|  |  |  |  |  |  |  | fluid permeability, |  |  |  |  | m2 |  |  |  |  |
| Note The element shall be produced by coating a reticulated polyurethane foam with nickel, removing the polyurethane by thermal decomposition and converting the nickel into a nickel-chrome alloy (e.g. by gaseous diffusion) and compressing the material as necessary. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Identification of metal foam elements |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Specifications identified in documentation (refer to |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ): |
| Sheet 63 of 67 (IEC 60079-1: 2001) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| ###### IEC 60079-1 |  |  | ANNEX C  ADDITIONAL REQUIREMENTS FOR FLAMEPROOF CABLE ENTRIES |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ENCL. |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ###### CLAUSE |  |  | REQUIREMENTS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| C.1 |  |  | Compliance with IEC 60079-0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| C.2 |  |  | Constructional requirements |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| C2.1 |  |  | Sealing methods |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| C2.1.1 |  |  | Cable entries with elastomeric sealing rings |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| C2.1.2 |  |  | Cable entries sealed with setting compound |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| C2.2 |  |  | Threaded cable entries |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Item: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Compliance with IEC 60079-0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Refer to IEC 60079-0 for additional details |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Constructional requirements |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ## Minimum uncompressed axial height |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Cable entry can accept any sealing ring with the same outside diameter but with different internal dimensions, |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | minimum uncompressed axial height of ring: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Circular cables diameter |  |  |  |  |  |  |  |  | < 20 mm, |  |  |  |  | 20 mm minimum uncompressed axial height for ring |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Non circular cable of perimeter |  |  |  |  |  |  |  |  | < 60 mm, |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Circular cables diameter |  |  |  |  |  |  |  |  | >20 mm |  |  |  |  | 25 mm minimum uncompressed axial height for ring |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Non circular cable perimeter |  |  |  |  |  |  |  |  | >60 mm |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Cable entries with elastomeric sealing rings |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Elastomeric sealing ring |  |  |  |  |  |  |  |  |  |  | Axial height |  |  |  |  |  |  |  | > 5 mm uncompressed |  |  |  |  |  |  |  |  |
| The cable is to be marked with an X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Flameproof enclosures for group I and IIC, with Volume > 2000 cm3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Axial height refer to C2.1.1.1 |  |  |  |  | Actual Height |  |  |  |  |  |  |  |  |  |  |  |  |  | mm |  |  |  |  |  |  |  |  |  |
| Cable entries sealed with setting compound |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Minimum length of the compound 20 mm, Actual |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | mm |  |  |  |  |  |  |  |  |  |  |  |
| Manufacturer shall specify: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Maximum diameter over cores of the cable |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | cm2 |  |  |  |  |  |  |  |  |
|  | Maximum number of cores that can pass through compound |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Installation instructions drawing no. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Rev. level |  |  |  |  |  |  |  |  |  |  |  |  |
| Threaded cable entries |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Refer to Cl. 5.3 for flameproof joints |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Cylindrical threads minimum length 8 mm, actual |  |  |  |  |  |  |  |  |  |  |  |  |  | mm, at least 6 full threads; actual |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Sheet 64 of 67 (IEC 60079-1: 2001) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| ###### IEC 60079-1 |  |  |  | ANNEX C  ADDITIONAL REQUIREMENTS FOR FLAMEPROOF CABLE ENTRIES |  |  |  |  |  |  |  | ENCL. |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ###### CLAUSE |  |  |  | REQUIREMENTS |  |  |  |  |  |  |  |  |  |
| C3 |  |  |  | Type Tests |  |  |  |  |  |  |  |  |  |
| C3.1 |  |  |  | Sealing Test |  |  |  |  |  |  |  |  |  |
| C3.2 |  |  |  | Test of mechanical strength |  |  |  |  |  |  |  |  |  |
| Item: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Cable entries with sealing ring |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Sealing Test |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Ring Type |  |  |  |  | Leakage |  | Test pressure |  |  | Torque applied (Nm) |  |  |
|  |  |  |  |  |  |  |  | (bar) |  | Duration (min) |  |  |  |
|  |  |  |  |  |  | NO | YES |  |  |  |  |  |  |
| Note: 20 bar for Group I and 30 bar for Group II. Duration of time 2 min. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Cable entries sealed with setting compound |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Prior to the hydraulic test, the cable entires were tested as described in Cl. 23.4.7.3 and 23.4.7.4 of IEC 60079-0 |  |  |  |  |  |  |  |  |  |  |  |
|  | Ring Type |  |  |  | Ring Size | Leakage |  | Test pressure |  |  |  |  |  |
|  |  |  |  |  |  |  |  | (bar) |  | Duration (min) |  |  |  |
|  |  |  |  |  |  | NO | YES |  |  |  |  |  |  |
| Note: 20 bar for Group I and 30 bar for Group II. Duration of time 2 min. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Cable entries with a screwed compression element |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Cable Type |  |  |  | Cable diameter/ perimeter (mm) |  | Deformation |  | Torque test |  |  |  |  |
|  |  |  |  |  |  |  |  |  | Nm |  |  |  |  |
|  |  |  |  |  |  |  | NO | YES |  |  |  |  |  |
| Sheet 65 of 67 (IEC 60079-1: 2001) |  |  |  |  |  |  |  |  |  |  |  |  |  |

| ###### IEC 60079-1 |  |  |  | ANNEX C  ADDITIONAL REQUIREMENTS FOR FLAMEPROOF CABLE ENTRIES |  |  |  |  | ENCL. |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ###### CLAUSE |  |  |  | REQUIREMENTS |  |  |  |  |  |  |
| C3.2 |  |  |  | Test of mechanical strength (cont’d) |  |  |  |  |  |  |
| Item: |  |  |  |  |  |  |  |  |  |  |
| Cable entries with a compression element fixed by screws |  |  |  |  |  |  |  |  |  |  |
|  | Compression element screw size |  |  |  | Deformation |  | Torque Test |  |  |  |
|  |  |  |  |  |  |  | Nm | Minimum values: M6: 10 Nm M12: 60 Nm M8: 20 Nm M14: 100 Nm M10: 40 Nm M16: 150 Nm |  |  |
|  |  |  |  |  | NO | YES |  |  |  |  |
| ###### Cable entries sealed with setting compound |  |  |  |  |  |  |  |  |  |  |
|  | Cable entry size |  |  |  | Deformation |  | Torque Test |  |  |  |
|  |  |  |  |  | NO | YES | Nm |  |  |  |
| Acceptance criteria |  |  |  |  |  |  |  |  |  |  |
|  |  | No permanent damage found for C3.2.1 to C3.2.3 |  |  |  |  |  |  |  |  |
| Sheet 66 of 67 (IEC 60079-1: 2001) |  |  |  |  |  |  |  |  |  |  |

| ## IEC 60079-1 |  | COMMENTS |  |  |
| --- | --- | --- | --- | --- |
| #### Clause |  |  | Page |  |
| Sheet 67 of 67 (IEC 60079-1: 2001) |  |  |  |  |